3GPP TSG-SA2 Meeting #135	S2-1910105

# Split, Croatia, 14th Oct 2019 - 18th Oct 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0004 | rev | 2 | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarifications to Abnormal behaviour analytics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ORANGE, Huawei, HiSilicon, Convida Wireless, Spirent Comm, NEC, China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA |  |  |  |  |  | Date: |  |  | 2019-10-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - No quantitative information on number of abnormal UEs (required for UE groups) - Missing parameters to limit the size of results in the case of large numbers or UEs (but done elsewhere in TS.23288) - No technique to provide lists of detected UEs - No confidence output in predictions (but done elsewhere in TS.23288) - Need of additional measurement for each risk (e.g. for unexpected UE location, NWDAF needs to output unexpected UE location for PCF to block the wrong location) - Other risks missing: unexpected radio link failures, ping ponging across cells |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add optional maximum number of results and Area of Interest to limit the scope of analysis - Add list of UE IDs in the outputs - Separate outputs for statistics & predictions, as done in otherchapters - Add confidence in outputs for predictions - Add examples of additional measurement for each risk - Add two new risks: unexpected radio link failures, frequent ping ponging across cells |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Implementation difficult in case of lists of abnormal UEs |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7.5.1, 6.7.5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | rev1 is a merge of rev0 with CR0011, CR0020 and CR0021. |  |  |  |  |  |  |  |  |

* * * Start of changes * * * *

### 6.7.5	Abnormal behaviour related network data analytics

#### 6.7.5.1	General

This clause defines how to identify a group of UEs or a specific UE with abnormal behaviour, e.g. being misused or hijacked, with the help of NWDAF.

NOTE 1:	The misused or hijacked UEs are UEs in which there are malicious applications running or UEs which have been stolen.

The consumer of this analytics could be a 5GC NF. The 5GC NF subscribes analytics on abnormal behaviour from a NWDAF based on the UE subscription, network configuration or application layer request.

The NWDAF performs data analytics on abnormal behaviour if there is a related subscription.

The consumer of this analytics shall indicate in the request:

-	Analytics ID set to "Abnormal behaviour";

-	The Target of Analytics Reporting can be one UE, any UE or an Internal Group Identifier;

-	Optionally an Application ID, DNN or S-NSSAI;

-	An Analytics target period indicates the time period over which the statistics or predictions are requested;

-	Analytics Filter Information: optional expected UE behaviour, expected analytics type or list of Exception IDs with associated thresholds, where the expected analytics type can be mobility related, communication related or both, Area of interest,  maximum number of  results;

NOTE 2:	The expected analytics type generally indicates whether mobility or communication related abnormal behaviour analytics or both are expected by the consumer, and the list of exception IDs indicates what specific analytics are expected by the consumer, the expected analytics type and the list of Exception IDs are not presented simultaneously.

-	In a subscription, the Notification Correlation Id and the Notification Target Address are included.

When the NWDAF detects those UEs that deviate from the expected UE behaviour, e.g. unexpected UE location, abnormal traffic pattern, wrong destination address etc., the NWDAF shall notify the result of the analytics to the consumer as specified in clause 6.7.5.3.

* * * Next change * * * *

#### 6.7.5.3	Output Analytics

The NWDAF services as defined in the clauses 7.2 and 7.3 are invoked to notify consumer NFs. A new Analytics ID named "Abnormal behaviour" is defined.

Corresponding to the Analytics ID, the analytics result provided by the NWDAF is defined in Table 6.7.5.3-1 and Table 6.7.5.3-2. When the level of an exception trespasses above or below the threshold, the NWDAF shall notify the consumer of the exception with an appropriate exception ID if the exception has an identity within the list of exception IDs indicated by the consumer or matches the expected analytics type indicated by the consumer.

Abnormal behaviour statistics information is defined in Table 6.7.5.3-1.

Table 6.7.5.3-1: Abnormal behaviour statistics

| Information | Description |
| --- | --- |
| Exceptions (1..max) | List of observed exceptions |
| > Exception ID | The risk detected by NWDAF |
| > Exception Level | Measured level, compared to the threshold |
| > Exception trend | Measured trend (up/down/unknown/stable) |
| > UE characteristics |  | UE group identifier, TAC |  |
| > SUPI list (1..N) |  | SUPI(s) of the UE(s) concerned with the Exception |  |
| > Ratio |  | Estimated percentage of UEs concerned by the Exception within the Target of Analytics Reporting. |  |
| > Amount |  | Estimated number of UEs concerned by the Exception (applicable when the Target of Analytics Reporting = "any UE"). |  |
| > Additional measurement | Specific information for each risk |

Abnormal behaviour predictions information is defined in Table 6.7.5.3-2.

Table 6.7.5.3-2: Abnormal behaviour predictions

| Information | Description |
| --- | --- |
| Exceptions (1..max) | List of predicted exceptions |
| > Exception ID | The risk detected by NWDAF |
| > Exception Level | Measured level, compared to the threshold |
| > Exception trend | Measured trend (up/down/unknown/stable) |
| > UE characteristics | UE group identifier, TAC |  |
| > SUPI list (1..N) | SUPI(s) of the UE(s) concerned with the Exception |  |
| > Ratio | Estimated percentage of UEs concerned by the Exception within the Target of Analytics Reporting. |  |
| > Amount | Estimated number of UEs concerned by the Exception (applicable when the Target of Analytics Reporting = "any UE"). |  |
| > Additional measurement | Specific information for each risk |
| > Confidence | Confidence of this prediction |

The UE characteristics may provide a set of features common to all UEs concerned with the exception.

The length of the SUPI list shall be lower than the parameter "maximum number of results".

If PCF subscribes notifications on "Abnormal behaviour", the NWDAF shall send the PCF notifications about the risk, which may trigger the PCF to update the AM/SM policies.

The NWDAF also sends the notification directly to the AMF or SMF, if the AMF or SMF subscribes the notification, so that the AMF or SMF may, based on operator local policies defined on a per S-NSSAI or per (DNN,S-NSSAI), take actions for risk solving. The following Table 6.7.5.3-2 gives examples of additional measurement, AM/SM policies and corresponding actions for solving each risk.

Table 6.7.5.3-3: Examples of policies and actions for risk solving

| Exception ID and description |  | Additional measurement |  | AM/SM policy |  | Actions of NFs |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Unexpected UE location |  | Unexpected UE location (TA or cells which the UE stays) |  | Add the area of current UE location into mobility restriction |  | PCF may extend the Service Area Restrictions. AMF may extend the mobility restriction |  |
| Unexpected long-live/large rate flows |  | Unexpected flow template (IP address 5-tuple) |  | Decrease the MBR for the related QoS flow |  | SMF updates the QoS rule. PCF, if dynamic PCC applies for corresponding DNN, S-NSSAI, updates PCC Rules that triggers SMF updates the QoS rule. |  |
| Unexpected wakeup |  | Time of unexpected wake-up |  | Apply MM back-off timer to the UE |  | AMF applies MM back-off timer to the UE |  |
| Suspicion of DDoS attack |  | Victim’s address (target IP address list) |  | Release the PDU session and Apply SM back-off timer |  | PCF may request SMF to release the PDU session. SMF may release the PDU session and applies SM back-off timer |  |
| Wrong destination address |  | Wrong destination address (target IP address list) |  | Update the packet filter of the related QoS flow to block the wrong SDF |  | PCF updates the packet filter in the PCC Rules that triggers the SMF to update the related QoS flow and configures the UPF |  |
| Ping-pong stationary UE |  | Numbers, frequency, time and location, assumptions about the possible circumstances |  | NWDAF notifies the AMF or AF (Service Provider) |  | AMF may adjust the UE registration area. |  |
| Too frequent Service Access/Abnormal traffic volume |  | Volume, frequency, time, assumptions about the possible circumstances |  | NWDAF notifies AF (Service Provider) |  |  |  |
| Unexpected radio link failures | Numbers, frequency, time and location, assumptions about the possible circumstances | Not applicable. | If the unexpected radio link failures are per UE location bases, the AMF may allow the use of CE (Coverage Enhancement) in the affected location. Also, the Operator may improve the coverage conditions in the affected location. If the unexpected radio link failures are per UE bases, then the AMF and/or the AF may allow the use of CE for the affected UE. |
| Ping-ponging across neighbouring cells | Numbers, frequency, time and location information, assumption about the possible circumstances of the ping-ponging | Not applicable. | If the amount of ping-ponging across neighbouring cells is above the thresholds set by the service provider, the service provider may adjust and improve the antenna tilts of the neighbouring cells or the overlapping coverage conditions in the affected location. If the ping-ponging are per UE, then the AMF and/or the AF may allow the use of Coverage Enhancement for the affected UE. |

* * * End of changes * * * *