3GPP TSG-WG SA2 Meeting #135 	S2-1910134

# Split, Croatia, October 14 – 18, 2019	(revision of S2-1909221)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0048 | rev | 1 | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Removal of the EN on the location information in Clause 5.3.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS |  |  |  |  |  | Date: |  |  | 2019-09-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In case of 4G trusted N3GPP access, the line id can be regarded as the location information, however in case of 5G trusted N3GPP access, there is no definition about the line id, so we shall use the TNAP id as the location information. More reasons can refer to the proposal S2-1905649 from the 5WWC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Related EN can be removed, and a new reference to the TNAP id can be added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Existing descriptions about 5G trusted N3GPP access are not accurate. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.3.1	Location Information for Non-3GPP Access

If the UE registered to non-3GPP access, following information can be regarded as UE location information:

Table 5.3.1-1: Supported UE location information of non-3GPP access

|  | Untrusted non-3GPP Access | Trusted non-3GPP Access | Wireline Access |
| --- | --- | --- | --- |
| UE Side 5G-RG side for Wireline Access | UE local IP address, In case of WLAN access, BSSID of the attached AP or BSSID of detected AP, Civic address and/or geospatial location information (NOTE 1, NOTE 5, NOTE 6). | Same as Untrusted non-3GPP Access | Null |
| N3IWF Side for Untrusted non-3GPP Access; TNGF Side for trusted non-3GPP Access; W-AGF Side for wireline Access | UE local IP address and optionally UDP or TCP source port (NOTE 2) | UE local IP address and optionally UDP or TCP source port (NOTE 2), TNAP Id (NOTE 2) | HFC node ID for 5G-CRG in TS 23.316 [21] clause 10.1; Line ID and operator identifier of the operator administrating the Line ID value for 5G-BRG in TS 23.316 [21] clause 10.1. |
| AMF Side | UE local IP address and optionally UDP source port (NOTE 3). Last known 3GPP access User Location Info (NOTE 4). | UE local IP address and optionally UDP source port (NOTE 3). Last known 3GPP access User Location Info (NOTE 4). TNAP Id (NOTE 2) | HFC node ID for 5G-CRG in TS 23.316 [21] clause 10.1; Line ID and operator identifier of the operator administrating the Line ID value for 5G-BRG in TS 23.316 [21] clause 10.1 |
| NOTE 1: In case of WLAN access, the UE may retrieve its location from a WLAN AP, prior or after association with the AP, requesting the Civic Location ANQP element, the Geospatial Location ANQP element or both as specified in IEEE Std 802.11-2012, using ANQP procedures described in HS2.0 Rel-12 specification. NOTE 2: More details can refer to TS 23.501 [18] clause 5.6.2. NOTE 3: This location information can be provided by location change event, more details can refer to TS 23.502 [19] clause 5.2.2.3.1. NOTE 4: This location information is also named as Last known Cell-Id, more details can refer to TS 23.501 [18] clause 5.6.2. NOTE 5: Geospatial location information can be obtained if UE (e.g. laptop) has installed GNSS receiver, i.e. GPS. NOTE 6: Some Applications (e.g. Google Map) may map the WiFi AP's BSSID with the geospatial locations obtain through GPS when the UE switch on the GPS and WiFi simultaneously. When another UE detect the same AP, the Application will send the geospatial locations to the UE. Thus the UE obtain the geospatial locations even without switch on the GPS. If the Application map the geospatial locations to civic address, the UE can also obtain the civic address. |  |  |  |

If the UE registered to 3GPP access and non-3GPP access simultaneously, following information can be regarded as UE location information:

-	All location information when the UE only registered to non-3GPP access,

-	All location information when the UE only registered to 3GPP access, more details can refer to TS 36.305 [7].

# * * * * End of change * * * *