3GPP TSG-WG SA2 Meeting #135 	S2-1910204

# Split, Croatia, October 14 – 18, 2019	(revision of S2-1909274)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0069 | rev | 1 | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add "Data target period" as a filter of data collection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA |  |  |  |  |  | Date: |  |  | 2019-10-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | As defined in clause 6.2.1, TS 23.288, the NWDAF may decide to reduce the amount of data collection in case of high signaling load, by either prioritizing requests, reducing the duration of data collection, or the sampling ratios. However, it does not define how to support data collection per target period in the current eNA specification. As defined in clause 6.2.2.5, during SMF data subscription, the NWDAF shall provide Event ID, Target of Event Reporting, Event Filter Information, A Notification Target Address and a Notification Correlation ID, Event Reporting Information and Expiry time to the SMF. In these parameters, only the Event Reporting Information (such as Maximum duration of reporting or Group Reporting Guard Time as defined in Table 4.15.1-1, TS 23.502) and the Expiry time in these parameters are time related. However, all the time information in the two parameters are related to Event Reporting time but not related to data collection time. For example, these time information could not help NWDAF collect the history information, e.g. the UE locations during the [2019.9.1 00:00~2019.9.30 23:59]. So it is proposed to add the Data target period to support data collection per target period and reduce the amount of data collection. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the "Data target period" as a filter of data collection to reduce the amount of data collection in case of high signaling load. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support for data collection per target period. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Clause 6.2.2.5, 6.2.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

6.2.2.5	Usage of Exposure framework by the NWDAF for Data Collection

The NWDAF shall subscribe (and unsubscribe) to the Event exposure service from NF(s) reusing the framework defined in TS 23.502 [3] clause 4.15. This framework supports the possibility for the NWDAF to indicate / request:

-	Events-ID: one or multiple Event ID(s) defined in TS 23.502 [3] clause 4.15.1

-	Target of Event Reporting defined in TS 23.502 [3] clause 4.15.1: the objects targeted by the Events. Within a subscription, all Event ID(s) are associated with the same target of event reporting. In the case of NWDAF, the objects can be UE(s), UE group(s), any UE.

-	Event Filter Information defined in TS 23.502 [3] clause 4.15.1. This provides Event Parameter Types and Event Parameter Value(s) to be matched against, which is used to reduce the amount of data collection:

-	Data target period i.e. time interval [start..end].

-	A Notification Target Address and a Notification Correlation ID as defined in TS 23.502 [3] clause 4.15.1, allowing the NWDAF to correlate notifications received from the NF with this subscription.

-	Event Reporting Information described in TS 23.502 [3] Table 4.15.1-1.

-	Expiry time as defined in TS 23.502 [3] clause 4.15.1.

The notifications from NFs/AFs contain on top of the Event being reported (and of dedicated information being reported for this event):

-	the Notification Correlation Information provided by the NWDAF in its request,

-	(when applicable to the event) the Target Id e.g. UE ID (SUPI and if available GPSI), and

-	a time stamp.

# * * * * End of changes * * * *