3GPP TSG-SA WG2 Meeting #135	S2-1910286

14 - 18 October, 2019, Split Croatia	(revision of S2-1909809)

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 1902 | rev | 1 | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Handling of OAM traffic for IAB-node |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated, AT&T |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | IABARC |  |  |  |  |  | Date: |  |  | 2019-09-23 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In SA2#134, the potential use of a new 5QI/QCI for the IAB-node's OAM traffic was discussed, and it was documented in TS 23.501 clause 5.x.1: Editor's Note: It is FFS if new 5QI/QCI needs to be defined for the OAM traffic. The main motivation for defining a new 5QI/QCI is to ensure the OAM traffic being treated with highest priority, e.g. higher than MC-PTT signaling (5QI=69) and MC-PTT voice (5QI=65). To achieve that, two potential approaches can be used without standardizing a new 5QI: - 5GS supports signalled QoS characteristics (as clarified in in clause 5.7.3.1), and the SMF can signal a high priority (i.e. value smaller than 5) as part of the QoS Profile to the NG-RAN node when its detects that the PDU session is for OAM traffic, e.g. due to the use of a special DNN or PCC rules; - alternatively, a pre-configured 5QI with high priority can be used for the QoS flows of the OAM PDU session. It is expected that the IAB-node does not needs to support roaming in this release, pre-configured 5QI can be utilized. Therefore, it is proposed that no new 5QI/QCI to be introduced, and either signallled QoS characteristics or pre-configured 5QI to be used for handling the OAM traffic for IAB-node. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove editor's note, and add a NOTE to explain the handling of QoS for OAM traffic for IAB-node. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | OAM traffic for IAB-node is not properly supported in 5GS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.35.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

*** BEGIN CHANGES ***

### 5.35.1	IAB architecture and functional entities

Integrated access and backhaul (IAB) enables wireless in-band and out-of-band relaying of NR Uu access traffic via NR Uu backhaul links.

At high level, IAB has the following characteristics:

-	IAB uses the CU/DU architecture defined in TS 38.401 [42], and the IAB operation via F1 (between IAB-donor and IAB-node) is invisible to the 5GC;

-	IAB performs relaying at layer-2, and therefore does not require a local UPF;

-	IAB supports multi-hop backhauling;

-	IAB supports dynamic topology update, i.e. the IAB-node can change the parent node, e.g. another IAB-node, or the IAB-donor, during operation, for example in response to backhaul link failure or blockage.

Figure 5.35.1-1 shows the IAB reference architecture with two backhaul hops, when connected to 5GC.

Figure 5.35.1-1: IAB architecture for 5GS

Editor's note:	This figure may be updated based on progress on IAB in RAN WGs.

Each relay, referred to as IAB-node, consists of a gNB-DU function and a UE function (referred to as MT). The gNB-DU in the IAB-node is responsible for providing NR Uu access to UEs and child IAB-nodes. The corresponding gNB-CU function resides on the IAB-donor gNB, which controls IAB-node gNB-DU via the F1 interface. IAB-node appears as a normal gNB to UEs and other IAB-nodes, and allows them to connect to the 5GC.

Editor's note:	RAN WGs chose to refer to the UE function in the IAB-node as MT. This may cause confusion and SA2 will determine a more suitable name.

The IAB-node MT function behaves as a UE, and reuses UE procedures to connect to:

-	the gNB-DU on a parent IAB-node or IAB-donor for access and backhauling;

-	the gNB-CU on the IAB-donor via RRC for control of the access and backhaul link;

-	5GC, e.g. AMF, via NAS;

-	OAM system via a PDU session or PDN connection (based on implementation).

NOTE: 	The 5GC, e.g. SMF, may detect that a PDU session for the IAB-UE is for the OAM system access, e.g. by checking the DNN and/or configuration. It is up to the operator configuration to choose whether to use 1 or multiple QoS flows for OAM traffic and the appropriate QoS parameters, e.g. using 5QI=6 for software downloading, and 5QI=80 with signalled higher priority or a pre-configured 5QI for alarm or control traffic.

The IAB-node MT can connect to 5GC over NR (SA mode) or connect to EPC (EN-DC mode). The UE connecting to the IAB-node can operate in the same or different modes than the IAB-node as defined in TS 38.401 [42].

*** END CHANGES ***