3GPP TSG SA WG2 Meeting #135	S2-1910550

# Split, Croatia, October 14 - 18, 2019														Rev of S2-1910108

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 1739 | rev | 2 | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | No impact on IMS voice session by a change of the IMS voice over PS session indicator during fallback |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1, TEI16 |  |  |  |  |  | Date: |  |  | 2019-10-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| ` |  |  |  |  |  |  |  |  |  |  |
| Reason for change: |  | The UE has received from the AMF that IMS Voice over PS is supported before EPS and RAT Fallback is initiated. The UE may receive during EPS Fallback from the MME and during RAT Fallback from the AMF the indication that IMS Voice over PS is not supported, however, the call establishment should continue. For 4G, TS 23.221 states ## 7.2d Domain selection decisions during ongoing sessions Ongoing IMS voice sessions are not impacted by a change of the IMS voice over PS session indicator (i.e. the UE performs a TAU or Combined TA/LA update in ECM-CONNECTED mode and receives IMS voice over PS session indicator indicating that IMS voice over PS sessions are not supported). If the UE has an ongoing IMS voice over PS session, then regardless of the value of the IMS voice over PS session indicator, the UE shall originate new calls in the same access domain as the ongoing IMS voice over PS session until the release of the ongoing IMS voice over PS session. TS 24.501 describes persistent PDU sessions as Persistent PDU session: either a non-emergency PDU session contains a GBR QoS flow with QoS equivalent to QoS of teleservice 11 and where there is a radio bearer associated with that PDU session, or an emergency PDU session where there is a radio bearer associated with that PDU session. However, the case seems not described that the PDU session is “persistent” during EPS/RAT Fallback even if the GBR QoS flow hasn’t been established and the IMS voice over PS session indicator changes from supported to unsupported. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The IMS voice session is not impacted by a change of the IMS voice over PS session indicator from supported to unsupported during RAT or EPS Fallback |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Possibility that the voice call establishment is aborted by the UE if the UE receives during EPS/RAT Fallback the indication that IMS Voice over PS session is not supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.16.3.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*************** Start of changes ***************

#### 5.16.3.10	IMS Voice Service via EPS Fallback or RAT fallback in 5GS

In order to support various deployment scenarios for obtaining IMS voice service, the UE and NR connected to 5GC may support the mechanism to direct or redirect the UE from NR connected to 5GC either towards E-UTRA connected to 5GC (RAT fallback) or towards EPS (E-UTRAN connected to EPC System fallback).

Following principles apply for IMS Voice Service:

-	The serving AMF indicates toward the UE during the Registration procedure that IMS voice over PS session is supported.

-	If a request for establishing the QoS flow for IMS voice reaches the NG-RAN, the NG-RAN responds indicating rejection of the establishment request and the NG-RAN may trigger one of the following procedures depending on UE capabilities, N26 availability, network configuration and radio conditions:

-	Redirection to EPS;

-	Handover procedure to EPS;

-	Redirection to E-UTRA connected to 5GC; or

-	Handover to E-UTRA connected to 5GC.

-	If needed, Network Provided Location Information is provided as described in clauses 4.13.6.1 and 4.13.6.2 of TS 23.502 [3].

-	The ongoing IMS voice session is not impacted by a change of the IMS voice over PS session indicator from supported to unsupported (i.e. the UE receives during RAT Fallback or EPS Fallback the IMS voice over PS session indicator indicating that IMS voice over PS sessions are not supported). The UE shall not terminate the ongoing IMS voice session in this case.

NOTE:	Any change in IMS voice over PS session indicator applies to new IMS sessions initiated only after the ongoing IMS voice session is terminated.

*************** End of changes ***************