SA WG2 Meeting #136 	S2-1911203

# 18 - 22 Nov 2019, Reno, USA

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 1891 | rev |  | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Supporting bulk dictionary entries creation and deletion for UCMF service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | RACS |  |  |  |  |  | Date: |  |  | 2019-9-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The current UMCF service provisioned to AF/NEF contain create and delete operation. During the two operations, it seems only one dictionary is created and deleted for each operation. It clarifies that bulk operation can be provided. Additionally, when the manufactuer wants to delete all the dictionaries in the UCMF, the manufacturer don’t need to list all the capability IDs to be deleted, so Flush operation is proposed to be supported. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Supporting bulk dictionary entries creation and deletion service. Support dictonary entries flush service. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Don’t support dictionary entries bulk creation and deletion service. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.18.1, 5.2.18.2.1, 5.2.18.2.2, 5.2.18.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

5.2.18	UCMF Services

5.2.18.1	General

The following table illustrates the UCMF Services.

Table 5.2.18.1-1: NF services provided by UCMF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nucmf_Provisioning | Create | Request/Response | AF, NEF |
|  | Delete | Request/Response | AF, NEF |
|  | Flush | Request/Response | AF, NEF |
| Nucmf_UECapabilityManagement | Resolve | Request/Response | AMF |
|  | Assign | Request/Response | AMF |
|  | Subscribe | Subscribe/Notify | AMF |
|  | Unsubscribe | Subscribe/Notify | AMF |
|  | Notify | Subscribe/Notify | AMF |

5.2.18.2	Nucmf_Provisioning service

5.2.18.2.1	Nucmf_Provisioning_Create service operation

Service operation name: Nucmf_Provisioning_Create.

Description: The consumer creates a UCMF dictionary entry(s) for a Manufacturer-assigned UE Radio Capability ID(s).

Inputs (required): UE Radio Capability ID(s), set(s) of UE radio access capabilities, Type Allocation Code

Inputs (optional): None.

Outputs (required): None.

Outputs (optional): None.

5.2.18.2.2	Nucmf_Provisioning_Delete service operation

Service operation name: Nucmf_Provisioning_Delete.

Description: The consumer deletes a UCMF dictionary entry(s) for a Manufacturer-assigned UE Radio Capability ID(s).

Inputs (required): UE Radio Capability ID (s) of the UCMF dictionary entry(s) to be deleted, Type Allocation Code

Inputs (optional): None.

Outputs (required): None.

Outputs (optional): None.

**** Next Change ****

##### 5.2.18.2.x	Nucmf_Provisioning_Flush service operation

Service operation name: Nucmf_Provisioning_Flush.

Description: The consumer flushes all of UCMF dictionary entries for all of Manufacturer-assigned UE Radio Capability IDs.

Inputs (required): The UCMF dictionary entries for all of the UE Radio Capability ID to be deleted.

Inputs (optional): None.

Outputs (required): None.

Outputs (optional): None.

**** Next Change ****

##### 5.2.18.3.5	Nucmf_UECapabilityManagement_Notify service operation

Service Operation name: Nucmf_UECapabilityManagement_Notify

Description: Producer NF provides notifications about creation or deletion of UCMF dictionary entries to subscribed consumer NF.

Inputs, Required: Notification Type ("creation", "deletion", "flush").

Inputs, Optional: If Notification Type is set to "creation": One or more UCMF dictionary entries, each UCMF dictionary entry consisting of a UE Radio Capability ID and the corresponding UE radio access capability. If Notification Type is set to "deletion": One or more UE Radio Capability IDs. If Notification Type is set to "flush": All UE Radio Capability IDs.

Outputs, Required: None.

Outputs, Optional: None.

**** End of Changes ****