SA WG2 Meeting #136	S2-1911433

Reno, USA, November 18 – 22, 2019

Source:	China Mobile

Title:	Key issue on limitation of data rate per Network Slice in UL and DL.

Document for:	Discussion and Agreement

Agenda Item:	8.8

Work Item / Release:	FS_eNS_Ph2, Rel-17

Abstract of the contribution:

Discussion

One of the attributes in the GST documented in NG.116 is the following:

Maximum downlink throughput

This attribute defines the maximum data rate supported by the network slice in downlink. These parameters can be used to offer different network slice contract qualities level, e.g. Gold, silver and bronze which have different maximum throughput values.

| Parameters |  |
| --- | --- |
| Value | Integer |
| Measurement unit | kbps |
| Example | 100 Mbps 20 Gbps |
| Tags | Scalability attributes KP |

| Attribute Presence |  |
| --- | --- |
| Mandatory |  |
| Conditional | X |
| Optional |  |

Maximum uplink throughput

This attribute defines the maximum data rate supported by the network slice in uplink. These parameters can be used to offer different network slice contract qualities level, e.g. Gold, silver and bronze which have different maximum throughput values.

| Parameters |  |
| --- | --- |
| Value | Integer |
| Measurement unit | kbps |
| Example | 100 Mbps 20 Gbps |
| Tags | Scalability attributes KP |

| Attribute Presence |  |
| --- | --- |
| Mandatory |  |
| Conditional | X |
| Optional |  |

There is no way for a PLMN to support per slice data rate limitation as so far. it is possible to perform per UE data rate limitation (UE-AMBR, see TS 23.501[x]), but not per network slice.

This key issue addresses this problem space.

Proposal

It is proposed the following key issue is added to TS 23.700-40

Proposed Text

5.X	Key Issue #X: limitation of data rate per network slice in UL and DL.

5.X.1	General description

One of the attributes in the GST documented in GSMA 5GJA NG.116[x] is the following:

Maximum downlink throughput

This attribute defines the maximum data rate supported by the network slice in downlink. These parameters can be used to offer different network slice contract qualities level, e.g. Gold, silver and bronze which have different maximum throughput values.

Table 5.X.1-1: Maximum downlink throughput Table

| Parameters |  |
| --- | --- |
| Value | Integer |
| Measurement unit | kbps |
| Example | 100 Mbps 20 Gbps |
| Tags | Scalability attributes KP |
| Attribute Presence |  |
| Mandatory |  |
| Conditional | X |
| Optional |  |

Maximum uplink throughput

This attribute defines the maximum data rate supported by the network slice in uplink. These parameters can be used to offer different network slice contract qualities level, e.g. Gold, silver and bronze which have different maximum throughput values.

Table 5.X.1-2: Maximum uplink throughput

| Parameters |  |
| --- | --- |
| Value | Integer |
| Measurement unit | kbps |
| Example | 100 Mbps 20 Gbps |
| Tags | Scalability attributes KP |
| Attribute Presence |  |
| Mandatory |  |
| Conditional | X |
| Optional |  |

There is no way for a PLMN to support per slice data rate limitation as so far. It is possible to perform per UE data rate limitation (UE-AMBR, see TS 23.501[x]), but not per network slice.

This key issue shall study:

- Whether and how to limit the data rate of all the UEs for a Network Slice, ensuring that the aggregate of the UEs that use the slice are rate limited to the rate defined for the Network Slice in DL and UL, including:

  - How to signal the rate limits

  - Impact on subscription data

- Any RAN impacts shall be identified and alignment with RAN WGs shall be pursued if any impacts are identified.

- Any SA5 impact shall be identified.