3GPP TSG-SA WG2 Meeting #136	S2-1912067

# Reno, Nevada, USA, November 18-22 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 1936 | rev | 1 | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NSSAA Services |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, Telecom Italia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNS |  |  |  |  |  | Date: |  |  | 2019-10-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The specifcation of NSSAA services is missing |  |  |  |  |  |  |  |  |
| Summary of change: |  | Services for NSSAA reauthentication trigger and for authorization revokation are defined |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 5.2.10	AUSF Services

#### 5.2.10.1	General

The following table illustrates the AUSF Services.

Table 5.2.10.1-1: List of AUSF Services

| Service Name | Service Operations |  | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- | --- |
| Nausf_UEAuthentication | Authenticate |  | Request/Response | AMF |
| Nausf_SoRProtection | Protect |  | Request/Response | UDM |
| Nausf_NSSA | Authenticate | Request/Response | AMF |
|  | Notify | Subscribe/Notify | AMF |

#### 5.2.10.2	Nausf_UEAuthentication service

##### 5.2.10.2.1	General

Service Description: the AUSF provides UE authentication service to the requester NF. For AKA based authentication, this operation can be also used to recover from synchronization failure situations.

##### 5.2.10.2.2	Nausf_UEAuthentication_Authenticate service operation

See TS 33.501 [15].

##### 5.2.10.2.3	Void

#### 5.2.10.3	Nausf_SoRProtection service

##### 5.2.10.3.1	General

Service Description: The AUSF provides the Steering of Roaming information protection service to the requester NF.

##### 5.2.10.3.2	Nausf_SoRProtection Protect service operation

See TS 33.501 [15].

#### 5.2.10.4	Nausf_NSSAA service

##### 5.2.10.4.1	General

Service Description: the AUSF provides NetworkSlice-Specific Authentication and Authorization (NSSAA) service to the requester NF by relaying EAP messages towards a AAA-S or AAA-P and performing related protocol conversion as needed. It also provides notification to the current AMF where the UE is of the need to re-authenticaticate and re-authorize the UE or to revoke the UE authorization. The AMF to receive the notification is implicitly subscribed and it is found in the UDM by providing the UE GPSI.

##### 5.2.10.4.2	Nausf__NSSAA_Authenticate service operation

See TS 33.501 [15].

##### 5.2.10.4.3	Nausf_NSSAA_Notify

See TS 33.501 [15].