3GPP TSG-SA2 Meeting #136	S2-1912174

# Reno, Nevada, United States, 18th Nov 2019 - 22nd Nov 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0093 | rev | 1 | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Alignments on Analytics Filter Information and clarifications on Reporting Thresholds |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ORANGE, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA |  |  |  |  |  | Date: |  |  | 2019-11-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - Some Analytics Filter Information is specified for some analytics but missing in the general clause on Analytics Exposure. - The definition of Reporting Thresholds is incomplete and there is a need for a hysteresis mechanism to avoid non-significant bursts. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Analytics Filter Information is completed according to the information specified for all analytics. - Reporting Thresholds are defined with aotion of hysteresis (start and stop triggers). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete and misaligned specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of changes * * * *

### 6.1.3	Contents of Analytics Exposure

The consumers of the Nnwdaf_AnalyticsSubscription or Nnwdaf_AnalyticsInfo service operations described in clause 7 may provide the following input parameters listed below.

-	A list of Analytics ID(s): identifies the requested analytics.

-	Analytics Filter Information: indicates the conditions to be fulfilled for reporting Analytics Information. This set of optional parameter types and values enables to select which type of analytics information is requested. The following Analytics Filter Information may be used:

-	subset of analytics requested among those specified for the Analytics ID;

-	S-NSSAI, DNN, Application ID;

-	Area of Interest (list of TAI or Cell IDs);

-	NF instance IDs, NF Set IDs, NF types;

-	(Only for Nnwdaf_AnalyticsSubscription) reporting control parameters:

-	Reporting Thresholds, which indicate conditions on the level of each requested analytics that whenreached shall be notified by the NWDAF;

-	Periodic (time-based) reporting, i.e. a number of messages per time unit;

-	maximum number of results expected;

-	Target of Analytics Reporting: indicates the object(s) for which Analytics information is requested, entities such as specific UEs, a group of UE(s) or any UE (i.e. all UEs).

-	A Notification Target Address (+ Notification Correlation ID) as defined in TS 23.502 [3] clause 4.15.1, allowing to correlate notifications received from NWDAF with this subscription.

-	Analytics Reporting Information with the following parameters:

-	Analytics Reporting Parameters defined in Table 4.15.1-1, TS 23.502 [3].

-	Analytics target period: time interval [start..end], either in the past or in the future, expressed as positive or negative offsets to the present. An Analytics target period in the past is a request for statistics. An Analytics target period in the future is a request for predictions.

-	Preferred level of accuracy of the analytics (e.g. Low/High).

-	Time when analytics information is needed (if applicable). If the time is reached the consumer does not need to wait for the analytics information any longer, yet the NWDAF may send an error response to the consumer, applicable to Nnwdaf_AnalyticsInfo_Request service operation.

NOTE:	The feasibility of the parameter "Time when analytics are needed" will be checked by stage 3.

The NWDAF provides the consumer the Nnwdaf_AnalyticsSubscription or Nnwdaf_AnalyticsInfo service operations described in clause 7, the output information listed below:

-	The Notification Correlation Information.

-	For each Analytics ID the analytics information in the requested Analytics target period.

-	In addition, the following additional information:

-	Timestamp of analytics generation, which allows consumers to decide until when the received information shall be used. For instance, an NF can deem a received notification from NWDAF for a given feedback as invalid based on this timestamp;

-	Validity period, which defines the time period for which the analytics information is valid.

-	Probability assertion: level of certainty, degree of confidence in statistics/prediction.

When one or more Reporting Thresholds are provided in the Analytics Filter Information, the NWDAF shall provide notifications as follows:

-	if the start condition is lower than the stop condition, then the notifications start when the analytics level goes below the start condition and stop when the analytics level goes above the stop condition;

-	if the start condition is higher than the stop condition, then the notifications start when the analytics level goes above the start condition and stop when the analytics level goes below the the stop condition;

-	if Reporting Thresholds are provided for multiple different analytics, a logical "OR" is applied between the different start conditions for starting the notifications, and a logical "AND" is applied between the different stop conditions for stopping the notifications.

* * * End of changes * * * *