3GPP TSG SA WG2 Meeting #136	S2-1912190

# Reno, US, 18 – 22 November, 2019														Rev of S2-1911020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 1715 | rev | 4 | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | SMS Correction to handle SMS-Termination when a UE is not able to receive SMS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1, TEI16 |  |  |  |  |  | Date: |  |  | 2019-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The procedure for MT-SMS delivery re-atttempt is not considering the case when the re-attempt is needed due to the UE is not registered in 5GC for SMS service in the first place (i.e. there is no SMSF registered in UDM when the SMS-GMSC requests routing information to UDM). This can happen when the UE is not registered in the 5GC at all (i.e. not registered in AMF) or when the UE is registered in 5GC via an AMF but for some reason the AMF did not select an SMSF for the UE (e.g. the did not indicate support for SMS or the AMF area being restricted for the UE or due to error situation when contacting the SMSF). |  |  |  |  |  |  |  |  |
| Summary of change: |  | The procedure for MT-SMS delivery re-atttempt is updated with the support of a new UDM internal flag to notify the SMS-GMSC about UE being registered for SMS service so that pending MT-SMS for the UE in the SMS-GMSC can be reattempted. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification leading to not possible to re-attempt MT-SMS delivery in some cases. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.13.3.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | The change in the revision clarifies the scope of the CR and where the UE is not in 2G/3G/4G coverage, and which is covered in TS 23.040. Hence HSS is removed from the text. The change also includes some restructuring of the text for better readability. |  |  |  |  |  |  |  |  |

*************** First changes**************

#### 4.13.3.9	Unsuccessful Mobile terminating SMS delivery re-attempt

The procedure of Unsuccessful Mobile terminating SMS delivery re-attempt is defined as follows:

-	If the UE is registered over both 3GPP access and non-3GPP access in the same AMF (i.e. the UE is registered in the same PLMN for both access types):

-	if the MT-SMS delivery over one Access Type has failed, the AMF, based on operator local policy, may re-attempt the MT-SMS delivery over the other Access Type before indicating failure to SMSF;

-	if the MT-SMS delivery on both Access Types has failed, the AMF shall inform the SMSF immediately.

-	If the AMF informs the SMSF that it cannot deliver the MT-SMS to the UE, the SMSF sends a failure report to the first SMS-GMSC (which can be co-located with IP-SM-GW or SMS Router) as defined in TS 23.040 [7]. If the SMS-GMSC has more than one entity for SMS transport towards the UE, then upon receiving MT-SMS failure report, the SMS-GMSC, based on operator local policy, may re-attempt the MT-SMS delivery via the other entity.

-	After the first SMS-GMSC informs the UDM that the UE is not able to receive MT-SMS, the UDM shall set its internal URRP-AMF flag.

-	If the UE is registered in an AMF and the UDM has not subscribed to UE Reachability Notification in the AMF yet, the UDM immediately initiates a subscription procedure as specified in clause 4.2.5.2.

-	When the AMF detects UE activities, it notifies UDM with UE Activity Notification as described in clause 4.2.5.3. The UDM clears its URRP-AMF flag and alerts related SCs to retry MT-SMS delivery.

- 	When the SMS-GMSC requests routing information from UDM for a UE not registered in 5GC, or for a registered UE which has not been yet registered for SMS service, the UDM reponds to the SMS-GMSC that the UE is absent, stores the SC address in the MWD list (if not yet stored) and indicates that to the SC as defined in TS 23.040 [7]. The UDM also sets an internal SMSF registration notification flag to notify the SC upon subsequent SMSF registration for the UE.

When the UDM receives an Nudm_UECM_Registration Request from an SMSF for a UE for which the SMSF registration notification flag is set, the UDM clears the flag and alerts the related SCs to retry the MT-SMS delivery.

NOTE :	This scenario assumes that the UE is not in 2G/3G/4G coverage.

*************** End of changes ***************