3GPP TSG-WG SA2 Meeting #136 	S2- 1912274

# Reno, USA, November 18 – 22, 2019	                  (Revision of S2-1912268)

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0087 | rev | 3 | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on NWDAF-assisted expected UE behavioural analytics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA |  |  |  |  |  |  | Date: |  | 2019-9-30 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In the procedural definition of 6.7.4.4.1 NWDAF-assisted expected UE behavioral analytics, the consumer request the NWDAF to get only predictions analytics, actually requested analytics could be statistics or predictions, based on the clause 6.7.4.1 General. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Requested analytics of NWDAF-assisted expected UE behavioral analytics are extended to include statistics or predictions. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requested analytics of NWDAF-assisted expected UE behavioral analytics are incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Clause 6.7.4.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of Change * * * *

#### 6.7.4.1	General

The clause 6.7.4 defines how a service consumer learns from the NWDAF the expected UE behaviour parameters as defined in clause 4.15.6.3, TS 23.502 [3] for a group of UEs or a specific UE.

The service consumer may be an NF (e.g. AMF, UDM, AF), or the OAM.

The consumer of these analytics shall indicate in the request:

-	Analytics ID set to "UE Mobility" or "UE Communication".

-	The Target of Analytics Reporting can be any UE or an Internal Group Identifier.

-	An Analytics target period, which indicates the time period over which the statistics or predictions are requested.

-	In a subscription, the Notification Correlation Id and the Notification Target Address are included.

The NWDAF shall notify the result of the analytics to the consumer as indicated in clause 6.7.4.3.

#### 6.7.4.2	Input Data

In order to produce "UE Mobility" analytics, the NWDAF collects UE mobility information as defined in clause 6.7.2.2.

In order to produce "UE Communication" analytics, the NWDAF collects UE communication information as defined in clause 6.7.3.2.

#### 6.7.4.3	Output Analytics

The analytics results for "UE Mobility" are specified in Table 6.7.2.3-1.

The analytics results for "UE Communication" are specified in Table 6.7.3.3-1.

#### 6.7.4.4	Procedures

##### 6.7.4.4.1	NWDAF-assisted expected UE behavioural analytics

Figure 6.7.4.4.1-1: NWDAF assisted expected UE behavioural analytics procedure

1.	5GC NF (e.g., AMF, SMF, AF and UDM) to NWDAF: Nnwdaf_AnalyticsInfo_Request (Analytics ID, Target of Analytics Reporting=SUPI) or Nnwdaf_AnalyticsSubscription_Subscribe (Analytics ID, Target of Analytics Reporting =SUPI).

The Analytics ID is set to "UE Mobility" or to "UE Communication"," and the consumer request analytics.

2a-b.	[Conditional] The NWDAF requests to AMF Namf_EventExposure_Subscribe (Event ID, SUPI), to request location report for the UE, if the Analytics ID is set to "UE Mobility", unless the information is available.

2c-d.	[Conditional] The NWDAF requests to SMF Nsmf_EventExposure_Subscribe (Event ID, SUPI), to request communication report for the UE, if the Analytics ID is set to "UE Communication", unless the information is available.

3.	The NWDAF derives requested analytics.

4.	NWDAF to 5GC NF: Nnwdaf_AnalyticsInfo_Response or Nnwdaf_AnalyticsSubscription_Notify.

The NWDAF provides requested Expected UE behaviour to the NF, using either Nnwdaf_AnalyticsInfo_Response or Nnwdaf_AnalyticsSubscription_Notify, depending on the service used in step 1.

5-6.	If the NF subscribed to at step 1, when the NWDAF generates new analytics, it provides the new generated analytics to the NF.

* * * * End of Changes * * * *