3GPP TSG-SA WG2 Meeting #137E	S2-2002050

# 24 - 27 February, 2020, Electronic meeting                              (revision of S2-2000555)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2071 | rev | 1 | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Additional traffic pattern calculations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Vertical_LAN |  |  |  |  |  | Date: |  |  | 2020-02-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | It addresses the FASMO item #22 It is possible to easily calculate traffic pattern parameters for DL aggregated flow of TSN Streams (all streams that belong to a traffic class) using 802.1Qbv scheduling information. Indeed, since TSN Streams of the same traffic class are already aggregated using the 802.1Qbv, the information can be useful to extract traffic pattern. This is only used for DL (DS-TT egress port) since there is a single PDU session associated to this port. The calculation method is the same as used for PSFP parameters. However, these traffic pattern parameters based on 802.1Qbv are with reference to the egress port, as opposed to the parameters obtained using PSFP which are referenced to the ingress port. Therefore a new option to determine the Burst Arrival time is added in clauses 5.27.2 and 5.27.3. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Additional information on calculation of traffic pattern parameters of aggregated TSN Stream using 802.1Qbv (reference to egress port) only for DL. Accordingly, changes in 5.27.2 and 5.27.3 to reflect that traffic pattern parameters may be referred to the time at ingress port (if based on PSFP information) or at egress port (if based on 802.1Qbv). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing traffic pattern parameter calculation for aggregated TSN Streams (in DL). |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.27.2, 5.27.3, I.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Need to implement S2-2001510 23.501 CR2085R2 first. The new text on Annex I.1 goes after the text from CR2085. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***BEGIN CHANGES***

### 5.27.2	TSC Assistance Information (TSCAI)

TSC assistance information describes TSC traffic characteristics for use in the 5G System. The knowledge of TSN traffic pattern is useful for the gNB to allow it to more efficiently schedule periodic, deterministic traffic flows either via Configured Grants, Semi-Persistent Scheduling or with dynamic grants. TSC assistance information, as defined in Table 5.27.2-1, is provided from SMF to 5G-AN, e.g. upon QoS Flow establishment. The TSCAI parameters are set according to corresponding parameters obtained from the AF. The AF gets per TSN stream traffic characteristics according to IEEE 802.1Q [98] clause 8.6.5.1 for 5GS logical bridge to identify the PDU session for the UL traffic.

The determination of the TSCAI by the 5GC is based on information received from the TSN AF via PCF. The TSN AF is responsible for obtaining PSFP (IEEE 802.1Q [98]) parameters and use them to calculate traffic pattern parameters (such as burst arrival time with reference to the ingress port if using PSFP or egress port if using 802.1Qbv, periodicity, and flow direction) and responsible of forwarding these parameters to the SMF (via PCF). TSN AF may aggregate TSN streams if the TSN streams belong to the same traffic class, terminate in the same egress port and have the same periodicity.

Annex I describe how the traffic pattern information is determined.

NOTE 1:	Further details of aggregation of TSN streams are left for implementation.

The TSN AF may aggregate multiple TSN Streams if they have the same periodicity and traffic class. In this case, TSN AF creates one TSN QoS container for the aggregated TSN streams. 5GC derives TSCAI on a per QoS Flow basis and send it to 5G-AN. The Burst Arrival Time and Periodicity component of the TSCAI that the SMF signals to the 5G-AN are specified with respect to the 5G clock. The 5GC is responsible for mapping the Burst Arrival Time and Periodicity from a TSN clock (that the TSN stream is based on) to the 5G clock based on the time offset and cumulative rateRatio between TSN time and 5GS time as measured and reported by the UPF. 5GC sets the DL TSCAI Burst Arrival Time as the sum of TSN QoS Burst Arrival Time and CN PDB. 5GC sets the UL TSCAI Burst Arrival Time as the sum of TSN QoS Burst Arrival Time and UE-DS-TT Residence Time. 5GC sets the Periodicity as the TSN QoS Periodicity.

NOTE 2:	In order to get Burst Arrival Time, Periodicity on a per TSN stream basis, support for IEEE 802.1Q [98] (as stated in clause 4.4.8.2) Per-Stream Filtering and Policing (PSFP) with stream gate operation is a prerequisite.

In the case of drift between TSN time and 5G time, the UPF updates the offset to SMF. In the case of change of cumulative rateRatio between TSN time and 5G time, the UPF updates the cumulative rateRatio to SMF. The SMF may then trigger a PDU Session Modification as defined in TS 23.502 [3] clause 4.3.3 in order to update the TSCAI parameter to the NG-RAN without requiring AN or N1 specific signalling exchange with the UE.

NOTE 3:	In order to prevent frequent updates from the UPF, the UPF sends the offset or the cumulative rateRatio only when the difference between the current measurement and the previously reported measurement is larger than a threshold.

Table 5.27.2-1: TSC Assistance Information

| Assistance Information | Description |
| --- | --- |
| Flow Direction | The direction of the TSC flow (uplink or downlink). |
| Periodicity | It refers to the time period between start of two bursts. |
| Burst Arrival time | The arrival time of the data burst at either the ingress of the RAN (downlink flow direction) or egress interface of the UE (uplink flow direction). |

***NEXT CHANGE***

### 5.27.3	Support for TSC QoS Flows

TSC QoS Flows use a Delay Critical GBR resource type and TSC Assistance Information. TSC QoS Flows may use standardized 5QIs, pre-configured 5QIs or dynamically assigned 5QI values (which requires signalling of QoS characteristics as part of the QoS profile) as specified in clause 5.7.2. For each instance of Periodicity, within each Period (defined by periodicity value), TSC QoS Flows are required to transmit only one burst of maximum size MDBV within the AN-PDB. Known QoS Flow traffic characteristics provided in the TSCAI may be used to optimize scheduling in the 5GS.

The following is applicable for the 5QI defined for TSC QoS Flows:

1.	The TSC Burst Size may be used to set the MDBV as follows:

The maximum TSC Burst Size is considered as the largest amount of data within a time period that is equal to the value of 5G-AN PDB of the 5QI that was set for this traffic class. The maximum value of TSC Burst Size should be mapped to a 5QI with MDBV that is equal or higher. This 5QI also shall have a PDB value that satisfies the bridge delay capabilities reported for the corresponding traffic class. For TSC QoS flows, the Maximum Burst Size of the aggregated TSC streams to be allocated to this QoS flow can be similarly mapped to a 5QI with MDBV value that is equal or higher, and the PDB of this 5QI shall also satisfy the bridge delay capabilities reported.

2.	The PDB is explicitly divided into 5G-AN PDB and CN PDB. The 5G-AN PDB is the packet delay budget applicable to the radio interface, including AN processing. The CN PDB is the delay between a UPF terminating N6 and a 5G-AN. Separate delay budgets are necessary for calculation of expected packet transmit times on 5G System interfaces.

3.	The TSCAI Burst Arrival Time calculation uses UE-DS-TT residence time and CN PDB as per clause 5.27.5. If the traffic pattern parameter Burst Arrival time is referred to the ingress port: for DL TSCAI Burst Arrival Time determination, CN PDB is needed, for UL TSCAI Burst Arrival Time determination, UE-DS-TT residence time is needed. If the traffic pattern parameter Burst Arrival time is referred to the egress port: for DL TSCAI Burst Arrival Time determination, UE-DS-TT residence time is needed, for UL TSCAI Burst Arrival Time determination, CN PDB is needed.

***NEXT CHANGE***

Annex I (normative):
TSN usage guidelines

# I.1	Determination of traffic pattern information usingPSFP

As described in clause 5.27.2, the calculation of the TSCAI relies upon mapping of information for the TSN stream(s) based upon certain information.

The traffic pattern parameter determination based on PSFP (IEEE P802.1Q [98]) is as follows:

-	Periodicity of a TSN stream is set equal to PSFPAdminCycleTime. For aggregated TSN streams with same periodicity, the periodicity is set equal to PSFPAdminCycleTime received from CNC for one of the TSN streams that are aggregated.

NOTE:	Given that only TSN streams that have the same periodicity can be aggregated, the PSFPAdminCycleTime for those TSN streams is assumed to be the same.

-	Burst Arrival time of a TSN stream at the ingress port is determined based on the following conditions:

-	If the PSFPgateStatesValue is Closed for the first timeIntervalValue, then the Arrival time is set to PSFPAdminBaseTime plus the first timeIntervalValue. If the PSFPgateStatesValue is Open for the first timeIntervalValue, then the Burst Arrival time is set to PSFPAdminBaseTime. For aggregated TSN streams, the arrival time is calculated similarly, but using the time interval to the first PSFPgateStatesValue that is Open from the aggregated TSN streams.

-	Flow direction of a TSN stream is determined based on the following conditions:

-	If the PSFP information is targeted for a DS-TT port, the Flow direction is UL. If the PSFP information is targeted for a NW-TT port, the Flow direction is DL. For aggregated TSN streams, the flow direction of any TSN stream in the group is used for the aggregated flow.

The traffic pattern parameter determination for DL aggregated TSN streams is based on 802.1Qbv [96] as follows:

-	Periodicity of an aggregation of all TSN streams belonging to a traffic class in DL is set equal to AdminCycleTime, if there is a single open gate in the cycle.

-	Burst Arrival time of an aggregation of all TSN streams belonging to a traffic class in DL at the egress port is determined based on the following conditions:

-	The Burst Arrival Time should be set to AdminBaseTime plus the sum of the timeIntervalValues for which the gateStatesValue is Closed in the AdminControlList until the first gate open time (i.e. until gateStatesValue set to Open is found). If the gateStatesValue is Open for the first timeIntervalValue, then the Burst Arrival time is set to AdminBaseTime.

-	Flow direction of an aggregation of all TSN streams belonging to a traffic class in DL at the egress port is determined based on the following:

-	Since determination of traffic pattern based on 802.1Qbv information is targeted for a DS-TT port (DL), the Flow direction is DL.

-	Burst size of an aggregation of all TSN streams belonging to a traffic class in DL at the egress port (which is useful to map to MDBV) is determined as follows:

-	The Burst size is equal to the timeIntervalValue (converted from ns to s) with gateStatesValue = Open multiplied by the bitrate of the corresponding port, if there is a single burst in the cycle.

-	Maximum Flow Bitrate size of an aggregation of all TSN streams belonging to a traffic class in DL at the egress port (which is useful to map to GFBR) is determined as follows:

-	The Maximum Flow Bitrate is equal to the summation of all timeIntervalValue (converted from ns to s) with gateStatesValue = Open, multiplied by the bitrate of the corresponding port, and divided by AdminCycleTime.

***END OF CHANGE***