SA WG2 Meeting #137E (e-meeting)	S2-2002087

# Feb 24 - 27, 2020, Elbonia

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2175 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UE capability match request during the registration procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1 |  |  |  |  |  | Date: |  |  | 2020-02-02 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel 16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | As discussed in the S2-200xxx, it is proposed to trigger “Initial Context Setup” procedure before sending the UE Radio Capability Match Request message if only NAS signalling connection is established during the registration procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a sentence to clarify that, the AMF triggers “Initial Context Setup” procedure before sending the UE Radio Capability Match Request message if only NAS signalling connection is established during the registration procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The AMF can not get UE Radio Capability Match Respone from the NG RAN. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.4.2a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First change * * *

5.4.4.2a	UE Radio Capability Match Request

If the AMF requires more information on the UE radio capabilities support to be able to set the IMS voice over PS Session Supported Indication (see clause 5.16.3), then the AMF may send a UE Radio Capability Match Request message to the NG-RAN. This procedure is typically used during the Registration Procedure or when AMF has not received the Voice Support Match Indicator (as part of the 5GMM Context).

During the Registration Procedure, if only NAS signalling connection is established, the AMF should initiate Initial Context Setup procedure as specified in TS 38.413 to establish AS security context before sending the UE Radio Capability Match Request message.

* * * End of changes * * *