3GPP TSG-WG SA2 Meeting #137E e-meeting 	S2-2002117

# Elbonia, February 24 – 27, 2020	(revision of S2-200xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2181 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Overview of idle mode mobility with data forwarding in 5GS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_CIoT |  |  |  |  |  | Date: |  |  | 2020-02-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | When the UE performs Registration if there is buffered downlink data pending then the AMF should trigger the SMF to forward the data so it can be delivered. The AMF can determine this based previous indications of buffered data and buffering time, or from indications of buffered data from the old AMF, as appropriate. The indication to forward the buffered data is missing, and therefore the data may not be delivered or additionally unexpectedly and unnecessarily delayed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | When data is buffered the AMF is informed about the time the data will be buffered for. When Registration occurs the AMF determines whether there is buffered data and triggers the SMF to invoke data forwarding. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Buffered downlink data will not be forwarded during Registration, leading to additional delays in delivering buffered downlink data or downlink data that could have been delivered being discarded. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.31.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 5.31.8	High latency communication

Functions for High latency communication may be used to handle mobile terminated (MT) communication with UEs being unreachable while using power saving functions as specified in clause 5.31.7. "High latency" refers to the initial response time before normal exchange of packets is established. That is, the time it takes before a UE has woken up from its power saving state and responded to an initial downlink packet or signal.

High latency communication is supported by extended buffering of downlink data in the UPF or SMF or NEF when a UE is using power saving function in CM-IDLE and not reachable. For UPF anchored PDU Sessions, the SMF configures during AN release the UPF with user data Forwarding Action Rule and user data Buffering Action Rule according to TS 29.244 [65]. The rules include instructions whether UPF buffering applies or the user data shall be forwarded to the SMF for buffering in the SMF. For NEF anchored PDU session, only extended buffering in the NEF is supported in this release of specification. During the Network Triggered Service Request procedure or Mobile Terminated Data Transport procedures in Control Plane CIoT 5GS Optimisation, the AMF provides an Estimated Maximum Wait Time to the SMF if the SMF indicates the support of extended buffering. The SMF determines the Extended Buffering Time based on the received Estimated Maximum Wait Time or local configuration, and indicates the determined Extended Buffering Time to the AMF. The handling is e.g. specified in the Network Triggered Service Request procedure, clauses 4.2.3.3, 4.2.6, 4.24.2 and 4.25.5 of TS 23.502 [3]. During Registration procedure, if the AMF determines that DL data is being buffered in the SMF/UPF and the DL Data Expiration Time has not expired, or has received the Buffered DL Data Waiting indication from the old AMF, the AMF indicates the SMF to invoke the data forwarding.

Notification procedures can be used to support High latency communication. The following procedures are available based on different monitoring events:

-	UE Reachability; or

-	Availability after DDN failure; or

-	Extended Buffering Status Change.

An AF may request a one-time "UE Reachability" notification when it wants to send data to a UE which is using a power saving function (see event subscription procedure in clause 4.15.3.2 of TS 23.502 [3]). The SCS/AS/AF then waits with sending the data until it gets a notification that the UE is reachable (see notification procedures in TS 23.502 [3]).

An AF may request repeated "Availability after DDN failure" notifications where each UE reachability notification is triggered by a preceding DDN failure, i.e. the AF sends a downlink packet to request a UE reachability notification when the UE becomes reachable. That downlink packet is discarded by the UPF or SMF or NEF (see notification procedures in TS 23.502 [3]).

An AF may request repeated "Extended Buffering Status Change" notifications when it wants indications that DL data has been buffered or when buffered DL data has been delivered to the UE.

An AF may provide parameters related to High Latency Communication for different methods to UDM, via NEF, as part of provisioning capability as specified in clause 5.20 "External exposure of Network capability". The UDM can further deliver the parameters to other NFs (e.g. AMF or SMF) as specified in clause 4.15.6 of TS 23.502 [3].

# * * * * End of changes * * * *