3GPP TSG-WG SA2 Meeting #137E e-meeting 	S2-2002157

# Elbonia, February 24 – 27, 2020	(revision of S2-200xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2187 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | clarification for PDU sessions transferring gPTP messages |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Vertical_LAN |  |  |  |  |  | Date: |  |  | 2020-02-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | UPF forwards the gPTP message to the UE via all PDU sessions terminating in this UPF that the UE has established to the TSN network. Clarify that the PDU session is not dedicated for gPTP message transfer, it can also support TSC QoS Flows which is used for TSN streams scheduling. |  |  |  |  |  |  |  |  |
| Summary of change: |  | See Reason for Change |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Confusions on PDU sessions for gPTP message transfer |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.27.1.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.27.1.2.2	Distribution of TSN clock and time-stamping

The mechanisms for distribution of TSN clock and time-stamping described in this clause are according to IEEE 802.1AS [104].

Within the 5GS, a PDU Session serving TSC shall support TSC QoS Flow(s) for TSN stream transmission, and a QoS Flow dedicated for all gPTP messages transmission. The QoS Flow dedicated for gPTP messages transmission is set up together with the QoS Flow with default QoS Rule and complies with the residence time upper bound requirement specified in IEEE 802.1AS [104].

NOTE:	The sum of the UE-DS-TT residence time and the PDB of the QoS Flow dedicated for gPTP messages transmission needs to be lower than the residence time upper bound requirement for a time-aware system specified in IEEE 802.1AS [104].

Upon reception of a downlink gPTP message the NW-TT makes an ingress timestamping (TSi) for each gPTP event (Sync) message and uses the cumulative rateRatio received inside the gPTP message payload (carried within Sync message for one-step operation or Follow_up message for two-step operation) to calculate the link delay from the upstream TSN node (gPTP entity) expressed in TSN GM time as specified in IEEE 802.1ASTM [104]. NW-TT then calculates the new cumulative rateRatio (i.e. the cumulative rateRatio of the 5GS) as specified in IEEE 802.1ASTM [104] and modifies the gPTP message payload (carried within Sync message for one-step operation or Follow_up message for two-step operation) as follows:

-	Adds the link delay from the upstream TSN node in TSN GM time to the correction field.

-	Replaces the cumulative rateRatio received from the upstream TSN node with the new cumulative rateRatio.

-	Adds TSi in the Suffix field of the gPTP packet as described in Annex H.

UPF then forwards the gPTP message to the UE via user plane (i.e. using the PDU session applicable for sending gPTP messages). Only one PDU session per UE per UPF is used for sending gPTP messages regardless of how many external TSN working domains have their clock information delivered through a given UPF serving that UE.

A UE receives the gPTP messages and forwards them to the DS-TT. The DS-TT then creates egress timestamping (TSe) for the gPTP event (Sync) messages for external TSN working domains. The difference between TSi and TSe is considered as the calculated residence time spent within the 5G system for this gPTP message expressed in 5GS time. The DS-TT then uses the rateRatio contained inside the gPTP message payload (carried within Sync message for one-step operation or Follow_up message for two-step operation) to convert the residence time spent within the 5GS in TSN GM time and modifies the payload of the gPTP message that it sends towards the downstream TSN node as follows:

-	Adds the calculated residence time expressed in TSN GM time to the correction field.

-	Removes TSi from the Suffix field.

# * * * * Second change * * * *

# * * * * Third change * * * *

# * * * * Fourth change * * * *

# * * * * Fifth change * * * *

# * * * * End of changes * * * *