SA WG2 Meeting #137E	S2-2002266

# Feb 24 - 27, 2020, Elbonia  															(revision of S2-20xxxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | DRAFT | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UE misbehaving information storage in EIR and UDR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17 |  |  |  |  |  | Date: |  |  | 2020-02-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Abnormal behaviour analytics allow NWDAF to identify a group of UEs or a specific UE with abnormal behaviour, e.g. being misused or hijacked. Misused or hijacked UEs are e.g. UEs in which there are malicious applications running or UEs which have been stolen (but not reported as stolen) or UE with a faulty SW version. Currently TS 23.288 specifies, for abnormal behaviour analytics, outputs related to SUPI. However, if a malicious application is running on the device, or the software of the device has been altered by some malware or UE has a faulty SW version, the mis-behaviour should be associated with the PEI. Once NWDAF has identified abnormal behaviour for a SUPI or for a PEI, the information is communicated to the analytics consumer. However, the information will be lost if e.g. the UE is switched off and on again: new AMF, SMF or PCF serving the UE would need to request again NWDAF for abnormal behavior analytics which may induce extra signaling and potentially extra delay to detect again the issue (the UE may pop up in another region served by a different NWDAF). It would be useufl to store the abnormal behaviour information in either UDR (when related to SUPI) or EIR (when related to PEI) so that the information can be used again by new NFs serving the UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add PEI list to output analytics. The procedure for abnormal behaviour analytics is updated to allow NWDAF to store the abnormal behaviour in UDR (when related to SUPI) or EIR (when related to PEI). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7.5.3, 6.7.5.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.502 CR ... TS 23.503 CR … |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Related draft CR on TS 23.502 is available in S2-2002269 and related draft CR on TS 23.503 is available in S2-2002270. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE (1)

#### 6.7.5.3	Output Analytics

The NWDAF services as defined in the clauses 7.2 and 7.3 are invoked to notify consumer NFs. A new Analytics ID named "Abnormal behaviour" is defined.

Corresponding to the Analytics ID, the analytics result provided by the NWDAF is defined in Table 6.7.5.3-1 and Table 6.7.5.3-2. When the level of an exception trespasses above or below the threshold, the NWDAF shall notify the consumer of the exception with an appropriate exception ID if the exception has an identity within the list of exception IDs indicated by the consumer or matches the expected analytics type indicated by the consumer. The NWDAF shall provide the Exception Level and determine which of the other information elements to provide, depending on the observed exception.

Abnormal behaviour statistics information is defined in Table 6.7.5.3-1.

Table 6.7.5.3-1: Abnormal behaviour statistics

| Information | Description |
| --- | --- |
| Exceptions (1..max) | List of observed exceptions |
| > Exception ID | The risk detected by NWDAF |
| > Exception Level | Measured level, compared to the threshold |
| > Exception trend | Measured trend (up/down/unknown/stable) |
| > UE characteristics | Internal Group Identifier, TAC |
| > SUPI list (1..N) | SUPI(s) of the UE(s) affected with the Exception |
| > PEI list (1..N) | PEI of the device(s) affected with the Exception |
| > Ratio | Estimated percentage of UEs affected by the Exception within the Target of Analytics Reporting |
| > Amount | Estimated number of UEs affected by the Exception (applicable when the Target of Analytics Reporting = "any UE") |
| > Additional measurement | Specific information for each risk |

Abnormal behaviour predictions information is defined in Table 6.7.5.3-2.

Table 6.7.5.3-2: Abnormal behaviour predictions

| Information | Description |
| --- | --- |
| Exceptions (1..max) | List of predicted exceptions |
| > Exception ID | The risk detected by NWDAF |
| > Exception Level | Measured level, compared to the threshold |
| > Exception trend | Measured trend (up/down/unknown/stable) |
| > UE characteristics | Internal Group Identifier, TAC |
| > SUPI list (1..N) | SUPI(s) of the UE(s) affected with the Exception |
| > PEI list (1..N) | PEI of the device(s) affected with the Exception |
| > Ratio | Estimated percentage of UEs affected by the Exception within the Target of Analytics Reporting |
| > Amount | Estimated number of UEs affected by the Exception (applicable when the Target of Analytics Reporting = "any UE") |
| > Additional measurement | Specific information for each risk |
| > Confidence | Confidence of this prediction |

The UE characteristics may provide a set of features common to all UEs affected with the exception.

The length of the SUPI list shall be lower than the parameter "maximum number of results" (see clause 6.1.3).

If PCF subscribes notifications on "Abnormal behaviour", the NWDAF shall send the PCF notifications about the risk, which may trigger the PCF to update the AM/SM policies.

The NWDAF also sends the notification directly to the AMF or SMF, if the AMF or SMF subscribes the notification, so that the AMF or SMF may, based on operator local policies defined on a per S-NSSAI or per (DNN,S-NSSAI), take actions for risk solving. The following Table 6.7.5.3-2 gives examples of additional measurement, AM/SM policies and corresponding actions for solving each risk.

Table 6.7.5.3-3: Examples of policies and actions for risk solving

| Exception ID and description | Additional measurement | AM/SM policy | Actions of NFs |
| --- | --- | --- | --- |
| Unexpected UE location | Unexpected UE location (TA or cells which the UE stays) | Add the area of current UE location into mobility restriction | PCF may extend the Service Area Restrictions. AMF may extend the mobility restriction |
| Unexpected long-live/large rate flows | Unexpected flow template (IP address 5 tuple) | Decrease the MBR for the related QoS flow | SMF updates the QoS rule. PCF, if dynamic PCC applies for corresponding DNN, S-NSSAI, updates PCC Rules that triggers SMF updates the QoS rule. |
| Unexpected wakeup | Time of unexpected wake-up | Apply MM back-off timer to the UE | AMF applies MM back-off timer to the UE |
| Suspicion of DDoS attack | Victim's address (target IP address list) | Release the PDU session and Apply SM back-off timer | PCF may request SMF to release the PDU session. SMF may release the PDU session and applies SM back-off timer |
| Wrong destination address | Wrong destination address (target IP address list) | Update the packet filter of the related QoS flow to block the wrong SDF | PCF updates the packet filter in the PCC Rules that triggers the SMF to update the related QoS flow and configures the UPF |
| Ping-pong UE | Numbers, frequency, time and location, assumptions about the possible circumstances | NWDAF notifies the AMF or AF (Service Provider) | AMF may adjust the UE (e.g. a stationary UE) registration area. |
| Too frequent Service Access/Abnormal traffic volume | Volume, frequency, time, assumptions about the possible circumstances | NWDAF notifies AF (Service Provider) |  |
| Unexpected radio link failures | Numbers, frequency, time and location, assumptions about the possible circumstances | Not applicable | If the unexpected radio link failures are per UE location bases, the AMF may allow the use of CE (Coverage Enhancement) in the affected location. Also, the Operator may improve the coverage conditions in the affected location. If the unexpected radio link failures are per UE bases, then the AMF and/or the AF may allow the use of CE for the affected UE. |
| Ping-ponging across neighbouring cells | Numbers, frequency, time and location information, assumption about the possible circumstances of the ping-ponging | Not applicable | If the amount of ping-ponging across neighbouring cells is above the thresholds set by the service provider, the service provider may adjust and improve the antenna tilts of the neighbouring cells or the overlapping coverage conditions in the affected location. If the ping-ponging are per UE, then the AMF and/or the AF may allow the use of Coverage Enhancement for the affected UE. |

NEXT CHANGE (2)

#### 6.7.5.4	Procedure

Figure 6.7.5.4-1: Procedure for NWDAF assisted misused or hijacked UEs identification

1a.	A consumer NF subscribes to NWDAF using Nnwdaf_AnalyticsSubscription_Subscribe (Analytics ID set to "Abnormal behaviour", Target of Analytics Reporting = Internal-Group-Identifier or SUPI).

A consumer NF may subscribe to abnormal behaviour notification from NWDAF for a group of UEs or a specific UE. The Analytics ID indicates the NWDAF to identify misused or hijacked UEs through abnormal behaviour analytic.

1b.	AF to NWDAF: Nnwdaf_AnalyticsSubscription_Subscribe (Analytics ID, Target of Analytics Reporting = External-group identifier or External UE ID).

For untrusted AFs, the AF sends the subscription via a NEF, where the AF invokes NEF service Nnef_AnalyticsExposure_Subscribe (Analytics ID, Target of Analytics Reporting = External-group-identifier or External UE ID).

An AF may also subscribe to abnormal behaviour notification from NWDAF for a group of UEs or a specific UE, where the subscription message may contain expected UE behaviour parameters identified on the application layer. If an External-Group-Identifier is provided by the AF, the NEF interrogates UDM to map the External-Group-Identifier to the Internal-Group-Identifier and obtain SUPI list corresponding to the Internal-Group-Identifier.

2.	NWDAF to AMF (Conditional): Namf_EventExposure_Subscribe (Event ID(s), Event Filter(s), Internal-Group-Identifier or SUPI).

The NWDAF sends subscription requests to the related AMF to collect UE behavioural information if it has not subscribed such data.

NOTE 1:	The NWDAF determines the AMF serving the UE as described in clause 6.2.2.1.

The AMF sends event reports to the NWDAF based on the report requirements contained in the subscription request received from the NWDAF.

If requested by NWDAF via Event Filter(s), the AMF checks whether the UE's behaviour matches its expected UE behavioural information. In this case, the AMF sends event reports to the NWDAF only when it detects that the UE's behaviour deviated from its expected UE behaviour.

3.	NWDAF to SMF (Conditional): Nsmf_EventExposure_Subscribe (Event ID(s), Event Filter(s), Internal-Group-Identifier or SUPI).

The NWDAF sends subscription requests to the related SMF if it has not subscribed such data.

NOTE 2:	The NWDAF determines the SMF serving the UE as described in clause 6.2.2.1.

The SMF sends event reports to the NWDAF based on the report requirements contained in the subscription request received from the NWDAF.

If requested by NWDAF via Event Filter(s), the SMF checks whether the UE's behaviour matches its expected UE behavioural information. In this case, the SMF sends event reports to the NWDAF only when it detects that the UE's behaviour deviated from its expected UE behaviour.

4.	The NWDAF performs data analytics for misused or hijacked UEs identification. Based on the analytics and operator's policies the NWDAF determines whether to send a notification to 5GC NFs or the AF.

5a.	NWDAF to consumer NF (AMF or PCF or SMF depending on the subscription) (Conditional): Nnwdaf_AnalyticsSubscription_Notify (Analytics ID, (Exception ID, Internal-Group-Identifier or SUPI, Exception level)).

If the NWDAF determines to send a notification to the subscribed 5GC NFs, the NWDAF invokes Nnwdaf_EventSubscription_Notify services. Based on the notification, the 5G NFs adopt configured actions to resolve/mitigate/avoid the risks as described in the Table 6.7.5.3-1.

5b.	NWDAF to AF (Conditional): Nnwdaf_AnalyticsSubscription_Notify (Analytics ID, (Exception ID, External UE ID)).

If the NWDAF determines to send a notification to the subscribed AF, the NWDAF needs to include external UE ID of the identified UE into the notification message.

NOTE 3:	Based on the notification, the AF can adopt corresponding actions, e.g. adjusting recommended TCP Window Size, adjusting recommended Service Start and End.

6.	If the abnormal behaviour is related to SUPI, the NWDAF stores the misbehaviour information within subscription data in UDR.

7.	If the abnormal behaviour is related to PEI, the NWDAF stores the misbehaviour information in EIR data associated with the PEI.

END OF CHANGES