SA WG2 Meeting #S2-137E	S2-2002453

24 - 27 February, 2020, Elbonia	(revision of S2-2001944)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0127 | rev | 1 | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarifications on data collection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA |  |  |  |  |  | Date: |  |  | 2020-02-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | 1. The correlation information to correlate data from SMF and AF is still unspecified, while referring to e.g. clause 6.4.2, the correlation information can be Timestamp, Application ID and IP filter information. 2. Time interval is used as (important) part of the correlation information, but more accurately, Timestamp should be used as the correlation information instead of the Timer interval. The input data from different NFs with the same Timestamp can be used by the NWDAF for analytics, e.g. as described in clause 6.4.2 and 6.7.2.2; while Time Interval has only definition related to Analytics target period in current specification and it is not for input data collection but for analytics reporting. 3. In the general description for data collection: a) it is unclear where behaviour data and pre-computed metrics are from; b) data from NFs and "NF data" (from NRF) should be differentiated; c) "With zero confidence, no analytics shall be returned" seems contradict with the description "If no sufficient data is collected to provide an estimation for the requested level of accuracy before the time deadline, the service shall return a zero confidence" in the following bullet, because the confidence is part of the output analytics. It is proposed to modify the former description to avoid confusion in Stage 3. 4. Some editorial modifications are needed in the relevant clauses. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Add correlation information for data correlation from SMF and AF. 2. Change Time Interval to Timestamp in correlation information. 3. In the general description for data collection: a) clarify that behaviour data and pre-computed metrics are from the NFs; b) replace the description of "NF data" with "(data …) available in the NFs" for exposure services offered by NFs; c) clarify that prediction may be returned with zero confidence. 4. Editorial modifications to the relevant clauses. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect and incomplete specification on correlation information. Ambiguous description on input data/output analytics which may lead to confusion in Stage 3. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1, 6.2.4 |  |  |  |  |  |  |  |  |
| “ |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

******************** 1st Change ********************

6.2	Procedures for Data Collection

6.2.1	General

The Data Collection feature permits NWDAF to retrieve data from various sources (e.g. NF such as AMF, SMF, PCF, and AF; OAM), as a basis of the computation of network analytics.

All available data encompass:

-	OAM global NF data,

-	Data available in NFs, e.g. behaviour data related to individual UEs or UE groups (e.g. UE reachability), and pre-computed metrics covering UE populations (e.g. number of UEs present in a geographical area), per spatial and temporal dimensions (e.g. per region for a period of time),

-	NF data available in the 5GC (e.g. NRF),

-	Data available in AF.

The NWDAF shall use at least one of the following services:

-	the Generic management services as defined in TS 28.532 [6], the Performance Management services as defined in TS 28.550 [7] or the Fault Supervision services as defined in TS 28.545 [9], offered by OAM in order to collect OAM global NF data.

-	the Exposure services offered by NFs in order to retrieve data and other non-OAM pre-computed metrics available in the NFs.

-	Other NF services in order to collect NF data (e.g. NRF)

The NWDAF shall obtain the proper information to perform data collection for a UE or group of UEs:

-	For an Analytics ID, NWDAF is configured with the corresponding NF Type(s) and/or event ID(s) and/or OAM measurement types.

-	NWDAF shall determine which NF instance(s) of the relevant NF type(s) are serving the UE or group of UEs, optionally taking into account the S-NSSAI(s) and area of interest as defined in clause 7.1.3, TS 23.501 [2].

-	NWDAF invokes Nnf_EventSubscribe services to collect data from the determined NF instance(s), and/or triggers the procedure in clause 6.2.3.2 to subscribe to OAM services to collect the OAM measurement.

The NWDAF performs data collection from an AF directly as defined in clause 6.2.2.2 or via NEF as defined in clause 6.2.2.3.

The NWDAF shall be able to discover the events supported by a NF.

Data collection procedures enables the NWDAF to efficiently obtain the appropriate data with the appropriate granularity.

When a request or subscription for statistics or predictions is received, the NWDAF may not possess the necessary data to perform the service, including:

-	Data on the monitoring period in the past, which is necessary for the provision of statistics and predictions matching the Analytics target period.

-	Data on longer monitoring periods in the past, which is necessary for model training.

Therefore, in order to optimize the service quality, the NWDAF may undertake the following actions:

-	The NWDAF may return a probability assertion as stated in clause 6.1.3 expressing the confidence in the prediction produced. Prediction may be returned with zero confidence as described below. This confidence is likely to grow in the case of subscriptions.

-	The value of the confidence depends on the level or urgency expressed by the parameter "preferred level of accuracy of the analytics" as listed in clause 6.1.3, the parameter "time when analytics information is needed" as listed in clause 6.1.3, and the availability of data. If no sufficient data is collected to provide an estimation for the requested level of accuracy before the time deadline, the service shall return a zero confidence. Otherwise, the NWDAF may wait until enough data is collected before providing a response or a first notification.

-	In order to be prepared for future requests on statistics from NFs/OAM, the NWDAF, upon operator configuration, may collect data on its own initiative, e.g. on samples of UEs (e.g. mobility), and retain the data collected in the data storage.

The volume and maximum duration of data storage is also subject of operator configuration.

The NWDAF may decide to reduce the amount of data collected to reduce signalling load, by either prioritizing requests received from analytics consumers, or reducing the extent (e.g. duration, scope) of data collection, or modifying the sampling ratios.

The NWDAF may skip data collection phase when the NWDAF already has enough information to provide requested analytics.

The data which NWDAF may collect is listed for each analytics in input data clause and is decided by the NWDAF.

NOTE:	NWDAF can skip data collection phase for some specific input data per the requested analytics e.g. when some of the data is already available at NWDAF for the requested analytics, or when NWDAF considers that some of the data is not needed at all to provide the requested analytics as per the analytics consumer request (e.g. based on preferred level of accuracy or based on the time when analytics are needed).

******************** 2nd Change ********************

6.2.4	Correlation between network data and service data

The Correlation information in each NF input data which helps NWDAF correlate data from different NFs is defined in Table 6.2.4-1, which is subject to all the network data analytics.

NOTE:	For simplicity, the correlation information is not listed in the input data per network data analytics.

Table 6.2.4-1: Correlation Information

| Correlation Information | Description |
| --- | --- |
| Timestamp, IP address 5-tuple | To correlate the data from AF and from UPF. |
| Timestamp, AN Tunnel Info (Clause 9.3.2.2, TS 38.413 [16]) | To correlate the UPF data and OAM data which are reported by the RAN (e.g. Reference Signal Received Power or Reference Signal Received Quality as defined in Table 6.4.2-3). |
| Timestamp, UE IP address | To correlate the data from UPF and SMF. |
| Timestamp, SUPI | To correlate data from SMF and AMF. |
| Timestamp, SUPI, DNN, S-NSSAI or UE IP address | To correlate data from SMF and PCF. |
| Timestamp, RAN UE NGAP ID (Clause 9.3.3.2, TS 38.413 [16]) and Global RAN Node ID | To correlate the AMF data and OAM data reported by the RAN (e.g. Reference Signal Received Power or Reference Signal Received Quality as defined in Table 6.4.2-3). |
| Timestamp, Application ID, IP filter information | To correlate data from SMF and AF. |

******************** End of Changes ********************