3GPP TSG-WG SA2 Meeting #137E e-meeting 	S2-2002588

# Elbonia, February 24 – 27, 2020	(revision of S2-2002145)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2046 | rev | 2 | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of N2 based handover considering CAG IDs supported by the target NG-RAN node |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Vertical_LAN |  |  |  |  |  | Date: |  |  | 2020-02-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In LS R3-194786, RAN3 asks SA2 that should we consider the case that the AMF may reject the NG based handover request based on the CAG IDs supported by the target NG-RAN node. In some scenarios, e.g. the Xn interface does not exist or failed, the source NG-RAN may not know the CAG ID supported in the target NG-RAN. In this case, the target cell needs to determine whether the handover can be performed based on CAG IDs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | During N2 based handover procedure, the target cell needs to determine whether the handover can be performed based on CAG IDs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Current handover procedure description is not complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.30.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 5.30.3.4	Network and cell (re-)selection, and access control

The following is assumed for network and cell selection, and access control:

-	The CAG cell shall broadcast information such that only UEs supporting CAG are accessing the cell (see TS 38.300 [27], TS 38.304 [50]);

NOTE 1:	The above also implies that cells are either CAG cells or normal PLMN cells.

-	In order to prevent access to NPNs for authorized UE(s) in case of network congestion/overload, existing mechanisms defined for Control Plane load control, congestion and overload control in clause 5.19 can be used, as well as the access control and barring functionality described in clause 5.2.5, or Unified Access Control using the access categories as defined in TS 24.501 [47] can be used.

-	For aspects of automatic and manual network selection in relation to CAG, see TS 23.122 [17];

-	For aspects related to cell (re-)selection, see TS 38.304 [50];

-	The Mobility Restrictions shall be able to restrict the UE's mobility according to the Allowed CAG list (if configured in the subscription) and include an indication whether the UE is only allowed to access CAG cells (if configured in the subscription);

-	During transition from CM-IDLE to CM-CONNECTED, if the UE is accessing the 5GS via a CAG cell, the UE shall provide the selected CAG Identifier to NG-RAN and the NG-RAN shall provide the CAG Identifier to the AMF:

-	The AMF shall verify whether UE access is allowed by Mobility Restrictions:

-	If the CAG Identifier received from the NG-RAN is part of the UE's Allowed CAG list, then the AMF accepts the NAS request;

-	If the CAG Identifier received from the NG-RAN is not part of the UE's Allowed CAG list, then the AMF rejects the NAS request with an appropriate cause code, whereas the UE removes that CAG Identifier, if it exists, from its Allowed CAG list, as defined in TS 24.501 [47]. The AMF shall then release the NAS signalling connection for the UE by triggering the AN release procedure; and

-	If the UE is accessing the network via a non-CAG cell and the UE's subscription contains an indication that the UE is only allowed to access CAG cells, then the AMF rejects the NAS request with an appropriate cause code, whereas the UE updates its local configuration, as defined in TS 24.501 [47]. The AMF shall then release the NAS signalling connection for the UE by triggering the AN release procedure.

-	During transition from RRC Inactive to RRC Connected state:

-	When the UE initiates the RRC Resume procedure for RRC Inactive to RRC Connected state transition in a CAG cell, NG-RAN shall reject the RRC Resume request from the UE if none of the CAG Identifiers supported by the CAG cell are part of the UE's Allowed CAG list according to the Mobility Restrictions received from the AMF.

-	When the UE initiates the RRC Resume procedure for RRC Inactive to RRC Connected state transition in a non-CAG cell, NG-RAN shall reject the UE's Resume request if the UE is only allowed to access CAG cells according to the Mobility Restrictions received from the AMF.

-	During connected mode mobility procedures:

-	Based on the Mobility Restrictions received from the AMF:

-	Source NG-RAN shall not handover the UE to a target NG-RAN node if the target is a CAG cell and none of the CAG Identifiers supported by the CAG cell are part of the UE's Allowed CAG list;

-	Source NG-RAN shall not handover the UE to a non-CAG cell if the UE is only allowed to access CAG cells;

-	If the target cell is a CAG cell, target NG-RAN shall reject the N2 based handover procedure if none of the CAG Identifiers supported by the CAG cell are part of the UE’s Allowed CAG list in the Mobility Restriction List.

-	If the target cell is a non-CAG cell, target NG-RAN shall reject the N2 based handover procedure if the UE is only allowed to access CAG cells based on the Mobility Restriction List.

-	Update of Mobility Restrictions:

-	When the AMF receives the Nudm_SDM_Notification from the UDM and the AMF determines that the Allowed CAG list or the indication whether the UE is only allowed to access CAG cells have changed;

-	If the Allowed CAG List becomes empty and the CAG-only indication is set, the AMF shall not update the CAG information in the UE and NG-RAN accordingly. The AMF updates the Non-Allowed Area including all the service areas of the UE. If the UE sends NAS request message to the AMF (e.g. Registration Request, Service Request), the AMF shall reject the NAS request with an appropriate cause code which does not require the UE to remove the CAG Identifier. The AMF allows the UE to access the network only for configuration update. In other cases,

-	The AMF shall update the CAG information in the Mobility Restrictions in the UE and NG-RAN accordingly; and

-	If the UE is currently accessing a CAG cell and the CAG Identifier(s) supported by the CAG cell have been removed from the Allowed CAG list or if the UE is currently accessing a non-CAG cell and the indication that the UE is only allowed to access CAG cells has been set in the subscription, then the AMF shall release the NAS signalling connection for the UE by triggering the AN release procedure.

NOTE 2:	When the UE is accessing the network for emergency service the conditions for AMF in clause 5.16.4.3 apply.

-	After UCU, the AMF may release the NAS signalling connection by triggering the AN release procedure to allow the UE to reselect a cell based on the updated Allowed CAG list and CAG-only indication, e.g., if the CAG Identifier of current cell is not part of the updated Allowed CAG list.

# * * * * End of changes * * * *