SA WG2 Meeting #S2-138e	S2-2002761

# Electronic meeting,  2020-04-20 -- 2020-04-24

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 2086 | rev | 1 | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | SCP selection of another SCP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eSBA |  |  |  |  |  | Date: |  |  | 2020-04-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | It is allowed to have multiple SCPs between a consumer and a producer, but it is not described how an SCP determines which is the next SCP to be reached. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Define a centralized routing information in the NRF that is accessible by the NF/SCP to identify the next SCP in the routing path. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No specified solution for how a consumer can reach a producer when multiple SCPs are deployed in the routing path. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.501 CR 2139 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** START CHANGES ***

### 5.2.7	NRF Services

#### 5.2.7.1	General

The following table shows the NRF Services and Service Operations:

Table 5.2.7.1-1: NF services provided by the NRF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnrf_NFManagement | NFRegister | Request/Response | AMF, SMF, UDM, AUSF, NEF, PCF, SMSF, NSSF, UPF, BSF, CHF, NWDAF, P-CSCF, HSS, UDR |
|  | NFUpdate | Request/Response | AMF, SMF, UDM, AUSF, NEF, PCF, SMSF, NSSF, UPF, BSF, CHF, NWDAF, P-CSCF, HSS, UDR |
|  | NFDeregister | Request/Response | AMF, SMF, UDM, AUSF, NEF, PCF, SMSF, NSSF, UPF, BSF, CHF, NWDAF, P-CSCF, HSS, UDR |
|  | NFStatusSubscribe | Subscribe/Notify | AMF, SMF, PCF, NEF, NSSF, SMSF, AUSF, CHF, NRF, NWDAF, I-CSCF, S-CSCF, IMS-AS, SCP, UDM |
|  | NFStatusNotify |  | AMF, SMF, PCF, NEF, NSSF, SMSF, AUSF, CHF, NWDAF, I-CSCF, S-CSCF, IMS-AS, SCP, UDM |
|  | NFStatusUnSubscribe |  | AMF, SMF, PCF, NEF, NSSF, SMSF, AUSF, CHF, NRF, NWDAF, I-CSCF, S-CSCF, IMS-AS, SCP, UDM |
| Nnrf_NFDiscovery | Request | Request/Response | AMF, SMF, PCF, NEF, NSSF, SMSF, AUSF, CHF, NRF, NWDAF, I-CSCF, S-CSCF, IMS-AS, SCP, UDM, AF (NOTE 2) |
| Nnrf_AccessToken | Get | Request/Response | AMF, SMF, PCF, NEF, NSSF, SMSF, AUSF, UDM, NWDAF, I-CSCF, S-CSCF, IMS-AS, HSS |
| Nnrf_NextHop | Get | Request/Response | SCP |

NOTE 1:	HSS_IMS services are defined in TS 23.228 [55].

NOTE 2:	The AF is a trusted AF by an operator.

*** NEXT CHANGE ***

#### 5.2.7.X	Nnrf_NextHop service

##### 5.2.7.X.1	General

The Nnrf_NextHop service provides a requester with the next hop address of an SCP (or multiple addresses for redundant SCPs). This allows for multiple SCPs to be deployed, where the routing map is held by the NRF.

##### 5.2.7.X.2	Nnrf_NextHop_Get service operation

Service Operation name: Nnrf_NextHop_Get.

Description: Provides the next hop SCP (i.e. one or multiple SCP address(es)) that should be used by the consumer for routing messages.

Inputs, Required: SCP identity of consumer and a target.

Where target is:

-	NF service in case of indirect communication with delegated discovery and requester is an NF.

-	NF service instance address if requester is SCP or NF using indirect communication without delegated discovery.

-	Notification Target Address in case of a notification request.

Inputs, Optional: none,

Outputs, Required: Null or one or multiple SCP address(es), validity time.

NOTE:	In case of null the Consumer routes to target NF.

Outputs, Optional: SCP priority for selection/re-selection (for load-balancing and failover situations).

*** NEXT CHANGE ***

Annex x (informative):
SCP to SCP routing

# x.1	General

Annex x describes how the Nnrf_NextHop service can be used to route message in a network.

## x.2	Multi-SCP path without delegated discovery

Figure x.2-1: Example flow with 3 SCPs

The prerequisite for this flow is that the NF consumer (denoted NFc) has done service instance selection of NF producer (denoted NFp).

1.	NFc has selected the target address of wanted service. It has the SCP1 as a configured SCP. NFc sends the service request to SCP1.

2.	The SCP1 checks if it has the received target address in its routing cache. In this case it did not have it, so it uses the Nnrf_NextHop_Get operation to get the next hop address.

3.	The NRF uses the configured routing map to select a suitable SCP. In this case it is SCP2, and NRF responds with the address of SCP2.

4.	SCP1 forwards the service request to SCP2 and includes the target address of the selected service instance.

5.	The SCP2 checks if it has the received target address in its routing cache. In this case it did not have it, so it uses the Nnrf_NextHop_Get operation to get the next hop address.

6.	The NRF uses the configured routing map to select a suitable SCP. In this case it is SCP3, and NRF responds with the address of SCP3.

7.	SCP2 forwards the service request to SCP3 and includes the target address of the selected service instance.

8.	The SCP3 checks if it has the received target address in its routing cache. In this case it did not have it, so it uses the Nnrf_NextHop_Get operation to get the next hop address.

9.	The NRF uses the configured routing map to select a suitable SCP. In this case it recognizes that the SCP3 should use the target address and by this it responds with NULL.

10.	SCP3 received NULL, this is an indication that SCP3 shall use the target address for next hop. SCP3 sends the request to the target address.

*** END CHANGES ***