SA WG2 Meeting #138-E	S2-2002950

Elbonia, April 20 - 24, 2020

Source:	China Mobile

Title:	Discussion on enhancement to Support Exposing UE mobility analytics

Document for:	Decision

Agenda Item:	9.1

Work Item / Release:	TEI17/Rel-17

Abstract: The discussion paper aims at explaining the requirements on exposing UE mobility analytics, and proposes the corresponding enhancements accordingly.

1. Discussion

The fundamental feature of UE mobility analytics produced by the NWDAF is available in Rel-16 specs. Also in TS 23.502 clause 4.15.6.2 NEF service operations information flow, the external AF can subscribe to NWDAF via NEF in order to learn the UE mobility analytics.

Furthermore, the UE mobility information is regarded as useful information for e.g. commercial usage for deriving commercial footprint between shopping centers, and therefore it would be much beneficial to allow the ASPs to consume UE mobility analytics.

The awareness of users who even stayed in some areas may become useful e.g., the ASPs inside Area B (Source Area of Interest) may need to enquiry the users who once stayed in Area A (Target Area of Interest) in target period as depicted in the below figure.

The proposed spec update is as follows:

1) 23.288: add a new procedure to request UE mobility analytics for ASPs.

2. Conclusion and proposal(s)

It’s proposed to enhance the existing specification to support exposing UE mobility analytics for the ASPs, and the related CR just for information is attached in the same .ZIP packet.

The expected time units is 0.5 TU.