3GPP TSG SA WG2 Meeting #138E	S2-2002711

# April 20 – 24, 2020, Elbonia	(Revision of S2-20xxxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2231 | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on SMS over NAS architecture in 5GS only Deployments |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1, UDICOM |  |  |  |  |  | Date: |  |  | 2020-04-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | 3GPP TS 23.501 section 4.4.2 shows the architecture to support SMS over NAS in 5GS, where the UDM interacts directly with the SMS-GMSC/IWMSC/SMS Router via the standardized MAP or Diameter interfaces defined in 3GPP TS 23.040. In interworking scenarios with EPS (and 2G/3G), a combined HSS+UDM is used as defined in 3GPP TS 23.501 section 5.17, and the HSS may facilitate the interactions with the SMS-GMSC/IWMSC/SMS Router via legacy interfaces. In case the HSS and the UDM are deployed as separate network functions, the interworking between HSS and UDM follows the procedures defined in 3GPP TS 23.632. However, for SMS delivery over 5G NAS in deployment scenarios where 2G/3G/4G accesses are not supported, it is expected that the MAP/Diameter interfaces with the SMS-GMSC/IWMSC/SMS Router are exposed by the UDM instead, since there will be no need to deploy HSS/HLR functionality supporting the legacy accesses and their corresponding reference points. It is expected that the 5G System can support autonomously the required services without the need to deploy legacy network functions that are not defined as part of the 5GS architecture. Hence, it is assumed that the UDM can support the functionality and reference points required for SMS in 5GS in deployment scenarios where 2G/3G/4G access are not supported, i.e. without the need to deploy the HSS/HLR only for the purpose of SMS handling. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that the UDM supports the legacy interfaces with the SMS-GMSC/IWMSC/SMS Router for SMS in 5GS without the need to deploy the HSS. Clarify that in interworking scenarios with EPS (and 2G/3G), the HSS is deployed in a combined HSS+UDM or as a separate network function and may expose the legacy interfaces required for SMS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not clear the architecture to be used to support SMS in 5GS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*************** First changes ***************

#### 4.4.2.1	Architecture to support SMS over NAS

Figure 4.4.2.1-1 shows the non-roaming architecture to support SMS over NAS using the Service-based interfaces within the Control Plane.

Figure 4.4.2.1-1: Non-roaming System Architecture for SMS over NAS

Figure 4.4.2.1-2 shows the non-roaming architecture to support SMS over NAS using the reference point representation.

Figure 4.4.2.1-2: Non-roaming System Architecture for SMS over NAS in reference point representation

NOTE 1:	SMS Function (SMSF) can be connected to the SMS-GMSC/IWMSC/SMS Router via one of the standardized interfaces as shown in TS 23.040 [5].

NOTE 2:	UDM supports the required interfaces for SMS in 5GS and can be connected to the SMS-GMSC/IWMSC/SMS Router via one of the standardized interfaces as shown in TS 23.040 [5].

For scenarios requiring interworking with EPC (and 2G/3G), a combined HSS+UDM is used as defined in section 5.17. In this case, the HSS can facilitate the interactions with the SMS-GMSC/IWMSC/SMS Router via the interfaces as shown in TS 23.040 [5]. In the case that the HSS and the UDM are deployed as separate network functions, the HSS interacts with the UDM as defined in 3GPP TS 23.632 [102].

NOTE 3:	Each UE is associated with only one SMS Function in the registered PLMN.

NOTE 4:	SMSF re-allocation while the UE is in RM-REGISTERED state in the serving PLMN is not supported in this Release of the specification. When serving AMF is re-allocated for a given UE, the source AMF includes SMSF identifier as part of UE context transfer to target AMF. If the target AMF, e.g. in the case of inter-PLMN mobility, detects that no SMSF has been selected in the serving PLMN, then the AMF performs SMSF selection as specified in clause 6.3.10.

NOTE 5:	To support MT SMS domain selection by IP-SM-GW/SMS Router, IP-SM-GW/SMS Router may connect to SGs MSC, MME and SMSF via one of the standardized interfaces as shown in TS 23.040 [5].

Figure 4.4.2.1-3 shows the roaming architecture to support SMS over NAS using the Service-based interfaces within the Control Plane.

Figure 4.4.2.1-3: Roaming architecture for SMS over NAS

Figure 4.4.2.1-4 shows the roaming architecture to support SMS over NAS using the reference point representation.

Figure 4.4.2.1-4: Roaming architecture for SMS over NAS in reference point representation

*************** End of changes ***************