3GPP TSG-SA2 Meeting #138-E	S2-2002975

Elbonia, April 20 – 24, 2020

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0166 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates of data collection for slice service experience |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA |  |  |  |  |  |  | Date: |  | 2020-04-10 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | There is a description in TS 23.288 that: If 'any UE' is the target of analytics reporting, NWDAF may subscribe to UE mobility event notifications of AMF using event ID "Area of Interest" if it is needed to retrieve the list of SUPIs. However, according to existing event IDs descriptions specified in SA2 and CT specifications, AMF can only monitor and report UE numbers of AOI and UE moving in and out of AOI, it can not report the currently UE ID list in AOI. In order to retrieve the list of SUPIs in AOI, it is necessary to extend the existing event ID or add a new event ID for AMF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - To retrieve the list of SUPI in AOI, the NWDAF subscribes to AMF using a modified event ID“Number of UEs and/or UE ID(s) in Area of Interest”. - Clarify the analytics of Observed Service Experience analytics also include variance, which had been added in the output table. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The list of SUPI can not be retrived using existing event ID “Area of Interest”. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.1, 6.4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  | TS/TR 23.501 CR 2140 |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR 23.502 CR 2089 |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of Changes * * * *

6.4.1	General

This clause specifies how NWDAF can provide Observed Service Experience (i.e. average of observed Service MoS, and/or variance of observed Service MoS indicating service MOS distribution) analytics, in the form of statistics or predictions, to a service consumer.

The Observed Service Experience analytics may provide one or both of the following:

-	Service Experience for a Network Slice: Service Experience for UEs (for a UE or a group of or any UE) for a given Application or a set of Applications or any Application (i.e. all Applications) in a Network Slice;

-	Service Experience for an Application: Service Experience (i.e. for a UE or a group of UEs or any UE) in an Application.

Therefore, Observed Service experience may be provided individually per UE or group of UEs, or globally, averaged per Application or averaged across a set of Applications on a Network Slice.

The service consumer may be an NF (e.g. PCF), or the OAM.

The consumer of these analytics shall indicate in the request or subscription:

-	Analytics Id set to "Service Experience";

-	The Target of Analytics Reporting: one or more SUPI(s) or Internal Group Identifier(s), or "any UE";

-	Analytics Filter Information as defined in Table 6.4.1-1 and, optionally, maximum number of objects and maximum number of SUPIs;

Table 6.4.1-1: Analytics Filter Information related to the observed service experience

| Information | Description |
| --- | --- |
| Application ID (1..n) (NOTE 1) | The identification of the application(s) for which the analytics information is subscribed or requested. |
| S-NSSAI (NOTE 2) | Identifies the Network Slice for which analytics information is subscribed or requested. |
| Area of Interest | Identifies the Area (i.e. set of TAIs), as defined in TS 23.501 [2] where the analytics information is subscribed or requested. |
| Media/application bandwidth | Identifies the Media/application bandwidth requirement of the application. |
| DNN | DNN to access the application. |
| DNAI | Identifier of a user plane access to one or more DN(s) where applications are deployed as defined in TS 23.501 [2] |
| NOTE 1: If no Application ID is provided, the Analytics Filter information applies to any application (i.e. all applications) in the Network Slice. NOTE 2: The S-NSSAI is mandatory if the NWDAF Service Consumer subscribes or requests the Service Experience in a Network Slice. Optionally, when multiple Network Slice instances of the same Network Slice (i.e. S-NSSAI) are deployed, associated NSI ID is used in addition to S-NSSAI. |  |

-	An Analytics target period that indicates the time window for which the statistics or predictions are requested;

-	In a subscription, the Notification Correlation Id and the Notification Target Address.

The NWDAF shall notify the result of the analytics to the consumer as specified in clause 6.4.3.

NWDAF collects the network data from AF (directly or via NEF) and from other 5GC NF(s) in order to calculate and provides statistics and predictions on the observed service experience to a consumer NF or to OAM.

* * * * Next Change * * * *

6.4.5	Procedures to request Service Experience for a Network Slice

Figure 6.4.5-1: Procedure for NWDAF providing Service Experience for a UE or a group of UEs in a Network Slice

This procedure is similar to the procedure in clause 6.4.4, with the following differences. The consumer needs to request the Analytics ID "Service Experience" for all UEs or a group of UEs or a UE on a Network Slice, identified by an S-NSSAI. If multiple Network Slice instances of the same Network Slice are deployed, associated NSI ID(s) may be used in addition to S-NSSAI. If 'any UE' is the target of analytics reporting, NWDAF may subscribe to UE mobility event notifications of AMF as described in clause 5.3.4.4 of TS 23.501 [2] using event ID " List of UEs in Area of Interest" and event filters as described in Table 5.2.2.3.1-1 of TS 23.502 [3] if it is needed to retrieve the list of SUPIs (and GPSIs if available) in the area of interest. The event exposure service request may also include the immediate reporting flag as event reporting information as described in Table 4.15.1-1 of TS 23.502 [3]. In addition, service experience data may need to be collected from multiple Applications. If each Application is hosted in a separate AF, NWDAF subscribes the service data in the Table 6.4.2-1 from the different AFs by invoking Nnef_EventExposure_Subscribe or Naf_EventExposure_Subscribe services for each Application (Event ID = Service Experience information, Event Filter information = Application ID) as defined TS 23.502 [3]. Figure 6.4.5-1 shows an example procedure with two AFs.

If one AF provides the service experience data of multiple Applications, the set of Application IDs is provided by NWDAF to the AF with the Naf_EventExposure_Subscribe service operation, as defined TS 23.502 [3].

The Observed Service Experience for a Network Slice when consumed by OAM could be used as described in Annex H of TS 28.550 [7].

* * * * End of Changes * * * *