SA WG2 Meeting #138E (e-meeting)	S2-2002682

# April 20 - 24, 2020, Elbonia

Title:	[Draft] Reply LS on manual CAG ID selection and granularity of UAC parameters for PNI-NPNs

Response to:	LS on manual CAG ID selection and granularity of UAC parameters for PNI-NPNs

Release:	Rel-16

Work Item:	NG_RAN_PRN-Core, Vertical_LAN

Source:	SA WG2

To:	RAN WG2

CC:	CT WG1, RAN WG3

# Contact Person:

# Name:	Xiaowan Ke

Email Address:	xiaowan.ke@vivo.com

# Attachments:

# 1. Overall Description:

SA2 thanks RAN2 for LS on Manual CAG ID selection and granularity of UAC parameters for PNI-NPNs (S2-2002656/R2-2002417). Accordingly, SA2 would like to provide the feedback to the questions as follows:

- Question 1.1; TO: SA2; CC: CT1

If a UE performs manual CAG selection and a successful registration, then whether the UE shall stay on cells supporting the manually selected CAG ID in RRC_CONNECTED state especially in the case when after registration the Allowed CAG List in the UE does not contain the manually selected CAG ID?

Answer1: If a UE performs manual CAG selection and a successful registration, the UE’s lastest CAG information including allowed CAG ID list is provided to the UE and the NG RAN. The UE is under the control of network during RRC_CONNECTED state. Furthermore, SA2 would like to provide the following clarification to RAN’s question and assumption:

-  1) In case the manually selected CAG ID is in the UE’s latest allowed CAG ID list, there is no issue for the UE to stay at the selected cell and the NG RAN can keep the UE in the selected cell since the selected cell support at least one of the allowed CAG ID.

-  2) In case the manually selected CAG ID is not in the UE’s latest allowed CAG ID list at the network side but the selected cell supports both the manually selected CAG ID and at least one of the allowed CAG ID for the UE, the UE registration can be successful. This is described in 23.501 clause 5.30.3.4 quote:

-     If at least one of the CAG Identifier(s) received from the NG-RAN is part of the UE's Allowed CAG list, then the AMF accepts the NAS request;

SA2 understands only this case can match RAN’s assumption “especially in the case when after registration the Allowed CAG List in the UE does not contain the manually selected CAG ID”. In this case, the selected cell is allowed for the UE, there is no issue for UE to stay at the selected cell and the NG RAN can keep the UE at the selected cell.

-  3) In case the selected cell doesn’t support any allowed CAG ID for the UE but registration success, no matter the manually selected CAG ID is in the UE’s latest allowed CAG ID list not, this is an error case and should not exist. SA2 would like to clarify that the AMF rejects the UE registration in case the selected cell doesn’t support any allowed CAG ID in the UE's subscription. This is described in 23.501 clause 5.30.3.4 quote:

-  If none of the CAG Identifier(s) received from the NG-RAN are part of the UE's Allowed CAG list, then the AMF rejects the NAS request with an appropriate cause code, etc.

- Question 1.2; TO: SA2; CC: CT1

Shall a UE prioritize for cell reselection the cells supporting the manually selected CAG ID over other suitable cells that do not support the manually selected CAG ID after a successful registration?

Answer2: From system perspective, when the manually selected CAG ID is not in the allowed CAG list after successful registration, SA2 does not see a need to prioritize for cell reselection the cells supporting the manually selected CAG ID over other suitable cells and to use the manually selected CAG ID for cell reselection.

When the manually CAG ID is in the allowed CAG list after successful registration, SA2 understands it need to be used for cell reselection during manual selection. Further details of manual selection and interaction with automatic selection are up to CT1 to define.

# 2. Actions:

# To RA2:

# ACTION: 	SA2 respectfully ask RAN2 to take the above into account.

# 3. Date of Next TSG-RAN WG2 Meetings:

SA2#139E (TBD)	25-29 May 2020			(TBD)

SA2#139Adhoc (TBD)	13-17 July 2020			(TBD)