3GPP TSG-SA2 Meeting #138-E	S2-2002746

Elbonia, April 20 – 24, 2020

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0150 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Miscellaneous FASMO corrections to service experience analytics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA |  |  |  |  |  |  | Date: |  | 2020-04-10 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The main corrections of this CR deal with the current incorrect indication that Area of Interest and Application ID can be AF event filters for NWDAF input data gathering. This is incorrect because i) Area of Interest does not have any meaning outside the PLMN, hence AF can not use it as event filter, and ii) Application ID is already an optional input parameter separate from event filters in AF event subscription service exposure as specified in TS 23.502, hence it is redundant to use it twice. In addition, a number of editorial corrections are required for clarity of the spec. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correction of the above technical issues by i) substituting AF event filter Area of Interest for “geographic zone identifiers” as described in clause 5.6.7.1 of TS 23.501, while keeping Area of Interest as event filter for NEF while access to AF is performed via NEF being NEF the translation entity; and ii) removing Application ID as AF event filter. The rest of changes are editorial clarifications. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect specifications |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.1, 6.4.2, 6.4.4, 6.4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS 23.502 CR 2303 |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of Changes * * * *

### 6.4.1	General

This clause specifies how NWDAF can provide Observed Service Experience (i.e. average observed Service MoS) analytics, in the form of statistics or predictions, to a service consumer.

The Observed Service Experience analytics may provide one or both of the following:

-	Service Experience for a Network Slice: Service Experience for UEs (for a UE or a group of or any UE) for a given Application or a set of Applications or any Application (i.e. all Applications) in a Network Slice;

-	Service Experience for an Application: Service Experience (i.e. for a UE or a group of UEs or any UE) in an Application.

Therefore, Observed Service experience may be provided individually per UE or group of UEs, or globally, averaged per Application or averaged across a set of Applications on a Network Slice.

The service consumer may be an NF (e.g. PCF), or the OAM.

The consumer of these analytics shall indicate in the request or subscription:

-	Analytics Id set to "Service Experience";

-	The Target of Analytics Reporting: one or more SUPI(s) or Internal Group Identifier(s), or "any UE";

-	Analytics Filter Information as defined in Table 6.4.1-1 and, optionally, maximum number of objects and maximum number of SUPIs;

Table 6.4.1-1: Analytics Filter Information related to the observed service experience

| Information | Description |
| --- | --- |
| Application ID (1..max) (NOTE 1) | The identification of the application(s) for which the analytics information is subscribed or requested. |
| S-NSSAI, NSI ID(s) (NOTE 2) | Identifies the Network Slice and Network Slice instance(s) for which analytics information is subscribed or requested. |
| Area of Interest (NOTE 3) | Identifies the Area (i.e. set of TAIs), as defined in TS 23.501 [2] where the analytics information is subscribed or requested. |
| Media/application bandwidth | Identifies the Media/application bandwidth requirement of the application. |
| DNN | DNN to access the application. |
| DNAI | Identifier of a user plane access to one or more DN(s) where applications are deployed as defined in TS 23.501 [2] |
| NOTE 1: If no Application ID is provided, the Analytics Filter information applies to any application (i.e. all applications) in the Network Slice. NOTE 2: The S-NSSAI is mandatory if the NWDAF Service Consumer subscribes or requests the Service Experience in a Network Slice. Optionally, when multiple Network Slice instances of the same Network Slice (i.e. S-NSSAI) are deployed, associated NSI ID(s) are used in addition to S-NSSAI. NOTE 3: Mandatory if Target Of Analytics Reporting is set to "any UE", optional otherwise. |  |

-	An Analytics target period that indicates the time window for which the statistics or predictions are requested;

-	In a subscription, the Notification Correlation Id and the Notification Target Address.

The NWDAF shall notify the result of the analytics to the consumer as specified in clause 6.4.3.

NWDAF collects the network data from AF (directly or via NEF) and from other 5GC NF(s) in order to calculate and provide statistics and predictions on the observed service experience to a consumer NF or to OAM.

### 6.4.2	Input Data

The service data collected from the AF, the network data from other 5GC NFs and the network data from OAM for observed service experience are defined in Table 6.4.2-1, Table 6.4.2-2 and Table 6.4.2-3, respectively.

Table 6.4.2-1: Service Data from AF related to the observed service experience

| Information | Source | Description |
| --- | --- | --- |
| Application ID | AF | To identify the service and support analytics per type of service (the desired level of service) |
| IP filter information | AF | Identify a service flow of the UE for the application |
| Locations of Application | AF/NEF | Locations of application represented by a list of DNAI(s). The NEF may map the AF-Service-Identifier information to a list of DNAI(s) when the DNAI(s) being used by the application are statically defined. |
| Service Experience | AF | Refers to the QoE per service flow as established in the SLA and during on boarding. It can be either e.g. MOS or video MOS as specified in ITU-T P.1203.3 [11] or a customized MOS |
| Timestamp | AF | A time stamp associated to the Service Experience provided by the AF, mandatory if the Service Experience is provided by the ASP. |

NWDAF subscribes to the service data from AF in the Table 6.4.2-1 either directly by invoking Naf_EventExposure_Subscribe service (Event ID = Service Experience information, Application ID) with geographic zone identifier(s) as optional Event Filter information as defined in TS 23.502 [3], or via NEF by invoking Nnef_EventExposure_Subscribe service (Event ID = Service Experience information, Application ID) with Area of Interest as optional Event Filter information.

Table 6.4.2-2: QoS flow level Network Data from 5GC NF related to the QoS profile assigned for a particular service (identified by an Application Id or IP filter information)

| Information | Source | Description |
| --- | --- | --- |
| Timestamp | 5GC NF | A time stamp associated with the collected information. |
| Location | AMF | The UE location information. |
| (list of) SUPI(s) | AMF | If UE IDs are not provided as target of analytics reporting for slice service experience, AMF returns the UE IDs matching the AMF event filters. |
| DNN | SMF | DNN for the PDU Session which contains the QoS flow |
| S-NSSAI | SMF | S-NSSAI for the PDU Session which contains the QoS flow |
| Application ID | SMF | Used by NWDAF to identify the application service provider and application for the QoS flow |
| IP filter information | SMF | Provided by the SMF, which is used by NWDAF to identify the service data flow for policy control and/or differentiated charging for the QoS flow |
| QFI | SMF | QoS Flow Identifier |
| QoS flow Bit Rate | UPF | The observed bit rate for UL direction; and The observed bit rate for DL direction |
| QoS flow Packet Delay | UPF | The observed Packet delay for UL direction; and The observed Packet delay for the DL direction |
| Packet transmission | UPF | The observed number of packet transmission |
| Packet retransmission | UPF | The observed number of packet retransmission |

NOTE 1:	How NWDAF collects QoS flow Bit Rate, QoS flow Packet Delay, Packet transmission and Packet retransmission information from UPF is not defined in this Release of the specification.

NOTE 2:	Care shall be taken with regards to load and major signaling caused when requesting Any UE. This could be achieved via mandating some event filters (e.g. Area of Interest for AMF), Analytics Reporting Information (e.g. SUPImax), or sampling ratio as part of Event Reporting Information.

NWDAF subscribes to the network data from 5GC NF(s) in the Table 6.4.2-2 by invoking Nnf_EventExposure_Subscribe service operation with the following Event IDs as input parameters:

-	AMF Source: Namf_EventExposure_Subscribe (Event IDs = Location Changes, Area of Interest).

-	SMF Source: Nsmf_EventExposure_Subscribe (Event ID = QFI allocation).

Table 6.4.2-3: UE level Network Data from OAM related to the QoS profile

| Information | Source | Description |
| --- | --- | --- |
| Timestamp | OAM | A time stamp associated with the collected information. |
| Reference Signal Received Power | OAM | The per UE measurement of the received power level in a network cell, including SS-RSRP, CSI-RSRP as specified in clause 5.5 of TS 38.331 [14] and E-UTRA RSRP as specified in clause 5.5.5 of TS 36.331 [15] |
| Reference Signal Received Quality | OAM | The per UE measurement of the received quality in a network cell, including SS-RSRQ, CSI-RSRQ as specified in clause 5.5 of TS 38.331 [14] and E-UTRA RSRQ as specified in clause 5.5.5 of TS 36.331 [15] |
| Signal-to-noise and interference ratio | OAM | The per UE measurement of the received signal to noise and interference ratio in a network cell, including SS-SINR, CSI-SINR, E-UTRA RS-SINR, as specified in clause 5.1 of TS 38.215 [12] |

NWDAF subscribes the network data from OAM in the Table 6.4.2-3 by using the services provided by OAM as described in clause 6.2.3.

The Event Filters for the service data collection from SMF, AMF and AF are defined in TS 23.502 [3].

The timestamps are provided by each NF to allow correlation of QoS and traffic KPIs. The clock reference is able to know the accuracy of the time and correlate the time series of the data retrieved from each NF.

* * * Next Change * * * *

### 6.4.4	Procedures to request Service Experience for an Application

Figure 6.4.4-1: Procedure for NWDAF providing Service Experience for an Application

This procedure allows the consumer to request Analytics ID "Service Experience" for a particular Application. The consumer includes both the Application ID for which the Service Experience is requested and indicates that the Target of Analytics Reporting is "any UE". At the same time, for an Application ID, a set of initial QoS parameter combinations per service experience window (e.g. one is for 3<Service MOS<4 and another is for 4<Service MOS<5) is defined in PCF (e.g. by configuration of operator policies) that may be updated based on the Service Experience reported by NWDAF.

1.	Consumer NF sends an Analytics request/subscribe (Analytics ID = Service Experience, Target of Analytics Reporting = any UE, Analytics Filter information = (Application ID, Analytics target period S-NSSAI, DNN, Area of Interest)) to NWDAF by invoking a Nnwdaf_AnalyticsInfo_Request or a Nnwdaf_AnalyticsSubscription_Subscribe.

2a. NWDAF subscribes the service data from AF in the Table 6.4.2-1 by invoking Nnef_EventExposure_Subscribe or Naf_EventExposure_Subscribe service (Event ID = Service Experience information, Application ID, Event Filter information), Target of Event Reporting = Any UE) as defined in TS 23.502 [3].

NOTE 1:	In the case of external AF, NEF translates the requested Area of Interest provided as event filter by NWDAF into a list of geographic zone identifier(s) that act as event filter for AF.

2b.	NWDAF subscribes the network data from 5GC NF(s) in the Table 6.4.2-2 by invoking Nnf_EventExposure_Subscribe service operation.

2c. With these data, the NWDAF estimates the Service experience for the application.

NOTE 2:	QoE measurements from the applications are based on outcome of the ongoing SA5 Rel-16 WID "Management of QoE measurement collection" which addresses how to collect the QoE measurements from the applications in the UE.

3.	The NWDAF provides the data analytics, i.e. the observed Service Experience (which can be a range of values) to the consumer NF by means of either Nnwdaf_AnalyticsInfo_Request response or Nnwdaf_AnalyticsSubscription_Notify, depending on the service used in step 1, indicating how well the used QoS Parameters satisfy the Service MoS agreed between the MNO and the end user or between the MNO and the external ASP.

NOTE 3:	The call flow only shows a request-response model for the interaction of NWDAF and consumer NF for simplicity instead of both request-response model and subscription-notification model.

If the consumer NF is a PCF and it determines that the application SLA is not satisfied, it may take into account the Observed Service Experience and the operator policies including SLA and required Service Experience (which can be a range of values) to determine new QoS parameters to be applied for the service, as defined in clause 6.1.1.3 and clause 6.2.1.2, TS 23.503 [4].

NOTE 4:	The non-real time data information from AF includes the service experience data (see Table 6.4.2-1), which indicates the service quality during the service lifetime.

### 6.4.5	Procedures to request Service Experience for a Network Slice

Figure 6.4.5-1: Procedure for NWDAF providing Service Experience for a UE or a group of UEs in a Network Slice

This procedure is similar to the procedure in clause 6.4.4, with the following differences. The consumer needs to request the Analytics ID "Service Experience" for all UEs or a group of UEs or a UE on a Network Slice, identified by an S-NSSAI. If multiple Network Slice instances of the same Network Slice are deployed, associated NSI ID(s) may be used in addition to S-NSSAI. If 'any UE' is the Target of Analytics Reporting, NWDAF may subscribe to UE mobility event notifications of AMF as described in clause 5.3.4.4 of TS 23.501 [2] using event ID "Area of Interest" and Event Filters as described in Table 5.2.2.3.1-1 of TS 23.502 [3] if it is needed to retrieve the list of SUPIs (and GPSIs if available). The event exposure service request may also include the immediate reporting flag as Event Reporting Information as described in Table 4.15.1-1 of TS 23.502 [3].

In addition, service experience data may need to be collected from multiple Applications. If each Application is hosted in different AFs, NWDAF subscribes the service data in the Table 6.4.2-1 from the different AFs by invoking Nnef_EventExposure_Subscribe or Naf_EventExposure_Subscribe services for each Application (Event ID = Service Experience information, Event Filter information, Application ID) as defined in TS 23.502 [3]. Figure 6.4.5-1 shows an example procedure with two AFs.

If one AF provides the service experience data of multiple Applications, the set of Application IDs is provided by NWDAF to the AF with the Naf_EventExposure_Subscribe service operation, as defined TS 23.502 [3].

The Observed Service Experience for a Network Slice when consumed by OAM could be used as described in Annex H of TS 28.550 [7].

* * * * End of Changes * * * *