3GPP SA WG2 Meeting #S2-138E	S2-2002765

20 – 24 April, 2020, Electronic meeting          (merge of S2-2002765 and S2-2002878)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0152 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Any UE for Abnormal Behavior Analytics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA |  |  |  |  |  | Date: |  |  | 2020-04-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Introduce Any UE subscription or request into Abnomal Behavior Analytics ID The data collection for Any UE may have major impact on load and signalling. Therefore care shall be taken with regards to load and major signaling caused when requesting Any UE. To optimize the data collection, different Filters may be used to lower the load and signalling. This contribution introduces a two-step method to lower load and signalling. If data from different types of NFs will be subscribed to, while assuring that data from the same UEs are retrieved, NWDAF may decide to optimize the data collection. If the Analytics Filter relevant to AMF (e.g. area of interest) is used in the subscription/request, the NWDAF first subscribes to data from AMF within the area of interest as in step 2. From the response from AMF it builds a list of SUPIs. Then it subscribes to data from SMF using this retrieved list of SUPIs as in step 3. If the Analytics Filter relevant to SMF (e.g. DNN and S-NSSAI combination) is used in the subscription/request, the NWDAF first subscribes to data from SMF within the DNN and S-NSSAI combination as in step 3. From the response from SMF it builds a list of SUPIs. Then it subscribes to data from AMF using this retrieved list of SUPIs as in step 2. When to perform this optimimization is NWDAF internal decision, as well as how NWDAF treats UEs that either start being served or end being served by either AMF or SMF during subscription. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of Any UE as input parameter when requesting data from AMF and SMF. Also introduced an optimization to lower load and signalling utilizing Filter in the subscription or request to NWDAF. In the Moderated controversial issues email discussion only Area of interest has been brought up as a Filter. That optimization utilizes the Any UE subscription described for Observed Service Experience related Analytics to retrieve a list of SUPIs. Which is used as input for the subscription to SMF. But in this contribution also the same mechanism is used when DNN in the input to NWDAF, and the list of SUPIs need to come from SMF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Any UE will not be included as a possible parameter in subscription or request for Abnormal Behavior analytics |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7.5.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR…. CR.. |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** BEGIN CHANGES ***

#### 6.7.5.4	Procedure

Figure 6.7.5.4-1: Procedure for NWDAF assisted misused or hijacked UEs identification

1a.	A consumer NF subscribes to/requests NWDAF using Nnwdaf_AnalyticsSubscription_Subscribe/ Nnwdaf_AnalyticsInfo_Request (Analytics ID set to "Abnormal behaviour", Target of Analytics Reporting = Internal-Group-Identifier or SUPI).

A consumer NF may subscribe to/request abnormal behaviour notification/response from NWDAF for a group of UEs, Any UE or a specific UE. The Analytics ID indicates the NWDAF to identify misused or hijacked UEs through abnormal behaviour analytic.

1b.	Consumer AF to NWDAF: Nnwdaf_AnalyticsSubscription_Subscribe or Nnwdaf_AnalyticsInfo_Request (Analytics ID, Target of Analytics Reporting = External-group identifier or External UE ID or Any UE, Analytics Filter Information= DNN, S-NSSAI).

For untrusted AFs, the AF sends the subscription via a NEF, where the AF invokes NEF service Nnef_AnalyticsExposure_Subscribe or Nnef_AnalyticsExposure_Fetch (Analytics ID, Target of Analytics Reporting = External-group-identifier or External UE ID or Any UE, Analytics Filter Information= Application ID).

An AF may also subscribe to/request abnormal behaviour notification/response from NWDAF for a group of UEs or a specific UE or Any UE, where the subscription/request message may contain expected UE behaviour parameters identified on the application layer. If an External-Group-Identifier is provided by the AF, the NEF interrogates UDM to map the External-Group-Identifier to the Internal-Group-Identifier and obtain SUPI list corresponding to the Internal-Group-Identifier. The NEF may also map AF-Identifier (e.g. Application ID) into DNN and S-NSSAI combination based on local configuration.

1c. [Conditional] This step applies if the NWDAF determines that data is required to be collected from AMF(s) and SMF(s) for the same set of UEs.

NWDAF decides the order in which steps 2 and 3 are executed

If the Analytics Filter relevant to AMF (e.g. area of interest) is used in the subscription/request, the NWDAF first subscribes to data from AMF within the area of interest as in step 2. From the response from AMF, it builds a list of SUPIs. Then it subscribes to data from SMF using this retrieved list of SUPIs as in step 3.

If the Analytics Filter relevant to SMF (e.g. DNN and S-NSSAI combination) is used in the subscription/request, the NWDAF first subscribes to data from SMF within the DNN and S-NSSAI combination as in step 3. From the response from SMF, it builds a list of SUPIs. Then it subscribes to/requests data from AMF using this retrieved list of SUPIs as in step 2.

NOTE 0: How NWDAF treats UEs that either start being served or end being served by either AMF or SMF during subscription is NWDAF internal decision.

2.	NWDAF to AMF (Conditional): Namf_EventExposure_Subscribe (Event ID(s), Event Filter Information =Area of Interest, Internal-Group-Identifier, Any UE, SUPI or list of SUPIs).

The NWDAF sends subscription requests to the related AMF to collect UE behavioural information if it has not subscribed such data.

NOTE 1:	The NWDAF determines the AMF serving the UE as described in clause 6.2.2.1.

The AMF sends event reports to the NWDAF based on the report requirements contained in the subscription request received from the NWDAF.

If requested by NWDAF via Event Filter(s), the AMF checks whether the UE's behaviour matches its expected UE behavioural information. In this case, the AMF sends event reports to the NWDAF only when it detects that the UE's behaviour deviated from its expected UE behaviour.

3.	NWDAF to SMF (Conditional): Nsmf_EventExposure_Subscribe (Event ID(s), Event Filter Information =(DNN, S-NSSAI), Internal-Group-Identifier, Any UE, SUPI or list of SUPIs).

The NWDAF sends subscription requests to the related SMF if it has not subscribed such data.

NOTE 2:	The NWDAF determines the SMF serving the UE as described in clause 6.2.2.1.

The SMF sends event reports to the NWDAF based on the report requirements contained in the subscription request received from the NWDAF.

If requested by NWDAF via Event Filter(s), the SMF checks whether the UE's behaviour matches its expected UE behavioural information. In this case, the SMF sends event reports to the NWDAF only when it detects that the UE's behaviour deviated from its expected UE behaviour.

4.	The NWDAF performs data analytics for misused or hijacked UEs identification. Based on the analytics and operator's policies the NWDAF determines whether to send a notification to 5GC NFs or the AF.

5a.	NWDAF to consumer NF (AMF or PCF or SMF depending on the subscription) (Conditional): Nnwdaf_AnalyticsSubscription_Notify or Nnwdaf_AnalyticsInfo_Response (Analytics ID, (Exception ID, Internal-Group-Identifier, Any UE or SUPI, Exception level)) (which is used depending on the service used in step 1a).

If the NWDAF determines to send a notification/response to the consumer 5GC NFs, the NWDAF invokes Nnwdaf_EventSubscription_Notify or Nnwdaf_AnalyticsInfo_Response services. Based on the notification/response, the 5G NFs adopt configured actions to resolve/mitigate/avoid the risks as described in the Table 6.7.5.3-1.

5b/5c.	NWDAF to AF (Conditional): Nnwdaf_AnalyticsSubscription_Notify or Nnwdaf_AnalyticsInfo_Response (Analytics ID, (Exception ID, Exception Level, External UE ID)) (which is used depending on the service used in step 1b).

If the NWDAF determines to send a notification/response to the consumer AF, the NWDAF needs to include external UE ID of the identified UE into the notification/response message.

NOTE 3:	Based on the notification, the AF can adopt corresponding actions, e.g. adjusting recommended TCP Window Size, adjusting recommended Service Start and End.

NOTE 4:	The call flow only shows a subscribe-notify model for the interaction of NWDAF and consumer NF for simplicity instead of both request-response model and subscription-notification model.

*** END OF CHANGES ***