SA WG2 Meeting #S2-138E	S2-2002926r05

20 - 24 April, 2020, Electronic, Elbonia

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0160 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Further corrections to Observed Service Experience analytics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA |  |  |  |  |  | Date: |  |  | 2020-04-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | For slice service experience and slice instance service experiences in Service Experience analytics: - Spatial validity and valid period are also needed as part of the statistics. - Confidence (i.e. probability assertion), spatial validity and valid period are also needed as part of the predictions information. - With the introduction of slice instance service experience, the slice service experience can be defined as service experience across Network Slice instances over the Analytics target period (average, variance). And the slice service experience is provided if multiple Network Slice instances are deployed. |  |  |  |  |  |  |  |  |
| FO |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Add spatial validity and valid period to the statistics information for slice service experience and each slice instance service experience 2. Add spatial validity, valid period and probability assertion to the predictions information for slice service experience and each slice instance service experience. 3. Clarify that the slice service experience is service experience across Network Slice instances and is provided only when multiple Network Slice instances are deployed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete statistics/predictions information for slice service experience and slice instance service experience. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.3, 6.4.3 |  |  |  |  |  |  |  |  |
| “ |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

******************** 1st Change ********************

6.1.3	Contents of Analytics Exposure

The consumers of the Nnwdaf_AnalyticsSubscription or Nnwdaf_AnalyticsInfo service operations described in clause 7 provide the following input parameters listed below.

-	A list of Analytics ID(s): identifies the requested analytics.

-	Analytics Filter Information: indicates the conditions to be fulfilled for reporting Analytics Information. This set of optional parameter types and values enables to select which type of analytics information is requested. Analytics Filter Information are defined in procedures.

-	Target of Analytics Reporting: indicates the object(s) for which Analytics information is requested, entities such as specific UEs, a group of UE(s) or any UE (i.e. all UEs).

-	(Only for Nnwdaf_AnalyticsSubscription) A Notification Target Address (+ Notification Correlation ID) as defined in TS 23.502 [3] clause 4.15.1, allowing to correlate notifications received from NWDAF with this subscription.

-	Analytics Reporting Information with the following parameters:

-	(Only for Nnwdaf_AnalyticsSubscription) Analytics Reporting Parameters as per Event Reporting parameters defined in Table 4.15.1-1, TS 23.502 [3].

-	(Only for Nnwdaf_AnalyticsSubscription) Reporting Thresholds, which indicate conditions on the level of each requested analytics that when reached shall be notified by the NWDAF. A matching direction may be provided such as below, above, or crossed. If no matching direction is provided, the default direction is crossed.

-	Analytics target period: time interval [start..end], either in the past (both start time and end time in the past) or in the future (both start time and end time in the future). An Analytics target period in the past is a request or subscription for statistics. An Analytics target period in the future is a request or subscription for predictions. The time interval is expressed with actual start time and actual end time (e.g. via UTC time). When the Analytics Reporting Parameters indicate a periodic reporting mode, the time interval can also be expressed as positive or negative offsets to the reporting time. By setting start time and end time to the same value, the consumer of the analytics can request analytics or subscribe to analytics for a specific time rather than for a time interval.

-	Preferred level of accuracy of the analytics (e.g. Low/High).

-	(Only for Nnwdaf_AnalyticsInfo_Request) Time when analytics information is needed (if applicable). If the time is reached the consumer does not need to wait for the analytics information any longer, yet the NWDAF may send an error response to the consumer.

-	[OPTIONAL] Maximum number of objects requested by the consumer (max) to limit the number of objects in a list of analytics per Nnwdaf_AnalyticsSubscription_Notify or Nnwdaf_AnalyticsInfo_Request response.

-	[OPTIONAL] Maximum number of SUPIs (SUPImax) requested by the consumer to limit the number of SUPIs in an object. When SUPImax is not provided, the NWDAF shal return all SUPIs concerned by the analytics object. When SUPImax is set to 0, the NWDAF shall not provide any SUPI.

NOTE:	The feasibility of the parameter "Time when analytics are needed" will be checked by stage 3.

The NWDAF provides to the consumer of the Nnwdaf_AnalyticsSubscription or Nnwdaf_AnalyticsInfo service operations described in clause 7, the output information listed below:

-	(Only for Nnwdaf_AnalyticsSubscription) The Notification Correlation Information.

-	For each Analytics ID the analytics information in the requested Analytics target period.

-	In addition, the following additional information:

-	Timestamp of analytics generation, which allows consumers to decide until when the received information shall be used. For instance, an NF can deem a received notification from NWDAF for a given feedback as invalid based on this timestamp;

-	Validity period, which defines the time period for which the analytics information is valid.

-	Probability assertion: level of certainty, confidence in statistics/prediction.

-	(Optional) Spatial validity: area where the estimated Service Experience applies. If Area of Interest information is provided in Analytics Filter Information, spatial validity should be the requested Area of Interest.

******************** 2nd Change ********************

6.4.3	Output Analytics

The NWDAF services as defined in the clause 7.2 and 7.3 are used to expose the analytics.

-	Service Experience statistics information is defined in Table 6.4.3-1.

-	Service Experience predictions information is defined in Table 6.4.3-2.

Table 6.4.3-1: Service Experience statistics

| Information | Description |
| --- | --- |
| S-NSSAI | Identifies the Network Slice for which analytics information is provided. |
| Slice service experience | Service experience across Network Slice instances over the Analytics target period (average, variance). |
| Slice instance service experiences (1…max) | List of observed service experience information for each Network Slice instance. |
| > NSI ID | Identifies the Network Slice instance within the Network Slice. |
| > Slice instance service experience | Service experience across Applications on a Network Slice instance over the Analytics target period (average, variance). |
| > SUPI list (0..SUPImax) | List of SUPI(s) for each slice instance service experience. |
| > Ratio | Estimated percentage of UEs with similar service experience (in the group, or among all UEs). |
| > Application service experiences (1..max) | List of observed service experience information for each Application. |
| >> Application ID | Identification of the application. |
| >> Service Experience Type | Type of Service Experience analytics, e.g. on voice, video, other. |
| >> Service Experience | Service Experience over the Analytics target period (average, variance). |
| >> SUPI list (0..SUPImax) | List of SUPI(s) for each application service experience. |
| >> Ratio | Estimated percentage of UEs with similar service experience (in the group, or among all UEs). |

Table 6.4.3-2: Service Experience predictions

| Information | Description |
| --- | --- |
| S-NSSAI | Identifies the Network Slice for which analytics information is provided. |
| Slice service experience | Service experience across Network Slice instances |
| > Slice service experience | Service experience across Network Slice instances over the Analytics target period (average, variance). |
| > Probability assertion | Confidence of this prediction. |
| Slice instance service experiences (1…max) | List of observed service experience information for each Network Slice instance. |
| > NSI ID | Identifies the Network Slice instance within the Network Slice. |
| > Slice instance service experience | Service experience across Applications on a Network Slice instance over the Analytics target period (average, variance). |
| > SUPI list (0..SUPImax) | List of SUPI(s) for each slice instance service experience. |
| > Ratio | Estimated percentage of UEs with similar service experience (in the group, or among all UEs). |
| > Probability assertion | Confidence of this prediction. |
| > Application service experiences (1..max) | List of predicted service experience information for each Application. |
| >> Application ID | Identification of the application. |
| >> Service Experience Type | Type of Service Experience analytics, e.g. on voice, video, other. |
| >> Service Experience | Service Experience over the Analytics target period (average, variance). |
| >> SUPI list (0..SUPImax) | List of SUPI(s) for each application service experience. |
| >> Ratio | Estimated percentage of UEs with similar service experience (in the group, or among all UEs). |
| >> Probability assertion | Confidence of this prediction. |

NOTE 1:	The spatial validity and validity period, as defined in clause 6.1.3, are provided for slice service experiences, slice instance service experiences or application service experiences if needed, in Service Experience statistics and predictions information.

NOTE 2:	If multiple Network Slice instances are not deployed for the S-NSSAI or NSI IDs are not available, only one Slice instance service experience entry is provided. In that case, the NSI ID is not provided and the Slice instance service experience indicates the service experience for the S-NSSAI.

NOTE 3:	The SUPI list and Ratio in the service experience information for an application can be omitted, if the corresponding parameter(s) is/are provided and with the same value(s) in the service experience information for the slice instance which the application belongs to.

The number of Service Experiences and SUPIs are limited respectively by the maximum number of objects and the Maximum number of SUPIs provided  in Analytics Reporting Information by the NWDAF Service Consumer.

N

F

(

consumer

)

NWDAF

NEF

AF

1

NF

(

network data

provider

)

4

.

NWDAF derives

requested analytics

for a Network Slice

AF

2

3

c

.

N

a

f

_

EventExposure

_

Subscribe

(

Event ID

=

Service Experience information)

2

a

.

Nnf

_

EventExposure

_

Subscribe

Request

(

Event ID

,

Event Filters

,

Event Reporting Information

)

2

c

.

Nnf

_

EventExposure

_

Notify

3

b

.

Naf

_

EventExposure

_

Notify

3

a

.

N

a

f

_

EventExposure

_

Subscribe

(

Event ID

=

Service Experience information)

1

.

Nnwdaf

_

Analytic

s

Info

_

Request

/

Nnwdaf

_

Analytic

s

Subscription

_

Subscribe

(

Analytics

ID

=

Service Experience

)

5

.

Nnwdaf

_

Analytic

s

Info

_

3d. Naf_EventExposure_Notify

Request Response

/

Nnwdaf

_

Analytic

s

Subscription

_

Notify

(

estimated Service Experience

)

2

b

.

Nnf

_

EventExposure

_

Subscribe

Response

******************** End of Changes ********************