3GPP TSG-WG SA2 Meeting #138E e-meeting  	S2-2002976

# Elbonia, April 20 – 23, 2020	(revision of S2-200xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 2241 | rev |  | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updating NRF Status Notify service operation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA |  |  |  |  |  | Date: |  |  | 2020-004-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | When a consumer request the UE abnormal behaviour analytics ID with analytics target set to “Any UE”, NWDAF has to determine all the AMFs and SMFs that are serving in the AoI. Currently TS 23.288 defines the NFs and services that need to be invoked in order for NWDAF to determine the NFs serving an UE. This definition is incomplete when it comes to NWDAF discovering NFs serving an AoI. The same mechanisms of discovering at NRF the UDM and BSF and then discovering the AMFs, SMFs, PCFs serving the UEs would lead to increase in data collection, because, for instance, NWDAF would have to subscribe to UDM UECM services with the filter set to any UE. The usage of NRF Discovery services allows NWDAF to discover at a point in time “T” the NFs serving an area of interest, by invoking the discovery service and using as filter the TAI. However, if in a new point in time “T+n” new NFs, (e.g., AMFs) are included or NFs are removed from an AoI, NWDAF would not be aware of such changes and will provide imprecise UE abnormal analytics IDs. Therefore, we propose that the discovery of AMFs and SMFs serving an area of interest for the generation of UE abnormal behaviour for “Any UE” to be based on the NRF NF Management Status service, which can keep NWDAF updated of any changes in NFs related to an AoI. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new input parameter in the Nnrf_NFManagement_StatusSubscribe service operation to allow the subscription to status of NFs to be filtered by area of interest. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NWDAF will receive updates of the NF profiles via status notify that are unnecessary for analytics generation and will just create load in the system without any benefit. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.7.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

##### 5.2.7.2.5	Nnrf_NFManagement_NFStatusSubscribe service operation

Service Operation name: Nnrf_NFManagement_NFStatusSubscribe.

Description: Consumer can subscribe to be notified of the following:

-	Newly registered NF along with its NF services.

-	Updated NF profile.

-	Deregistered NF.

Inputs, Required: NF type (if NF status of a specific NF type is to be monitored), NF instance ID (if NF status of a specific NF instance is to be monitored), NF service (if NF status for NF which exposes a given NF service is to be monitored).

Inputs, Optional:

-	PLMN ID of the target NF/NF service, in the case of the subscription to the status of NF/NF service instance(s) in home PLMN from the serving PLMN.

-	For the UPF Management defined in clause 4.17.6: UPF Provisioning Information as defined in that clause.

-	For AMF, Consumer may include list of GUAMI(s).

-	S-NSSAI(s) and the associated NSI ID(s) (if available).

-	For AMF and SMF, Consumer may include a list of TAIs.

-	For NWDAF, Consumer may include Analytics ID(s) and TAI(s). Details about NWDAF discovery and selection are described in clause 6.3.13, TS 23.501 [2].

-	For NEF, Consumer may include Event ID(s) provided by AF.

Outputs, Required: When the subscription is accepted: Subscription Correlation ID (required for management of this subscription).

Outputs, Optional: None.

NOTE:	Alternatively, other means such as OA&M can also be used to subscribe for NF status.

# * * * * End of changes * * * *