3GPP TSG-WG SA2 Meeting #138E e-meeting  	S2-2002989

# Elbonia, April 20 – 23, 2020	(revision of S2-200xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 2245 | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on I-NEF event exposure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_CIoT |  |  |  |  |  | Date: |  |  | 2020-04-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - In the I-NEF service operations information flow is incorrect. - Step 2d is incorrect, as the AMF has already received the I-NEF routing address in the Ninef_EventExposure_Subscribe request. - Step 3: notification events detected by SMF do not require sent to the AMF, the SMF notifies the I-NEF directly. - Event notifications should be conditional, depending on the detection of the subscribed monitoring event. - As answered in the reply to LS S2-2002645/C3-201494 from CT3, Ninef_EventExposure_Subscribe operation parameters are incorrect. - There is no use case to require "Target of Event Reporting (SUPI or Internal Group Identifier)". - As defined in TS 29.128 clause 6.4.2, "Maximum Number of Reports" and/or "Monitoring Duration" are optionally sent from the MME/SGSN to IWK-SCEF. Hence, the "Event Reporting Information" may be provided in interworking scenarios. - Event filtering is not an I-NEF task. - The notification target address to receive notifications sent to the NEF needs to be provided in the Ninef_EventExposure_Subscribe response. - Change the Ninef_EventExposure service, rename Ninef_EventExposure subscribe/unsubscribe to Ninef_EventRoutingConfiguration Add/Delete, and remove the Ninef_EventExposure Notify service operation to keep NEF unimpacted. - Namf_EventExposure_Notify operations, the “AMF ID (GUAMI)” parameter is not mandatory. In TS 29.518, the AMF ID is only needed for UE reachability. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - I-NEF call flow and service operation parameters are corrected: - If I-NEF is needed, the SMF sends notifications to that I-NEF. - In clause 4.15.3.2.3a steps 2-3, the I-NEF need not subscribe to notifications from the AMF or the SMF. - Monitoring event notifications are conditional, subject to the notification event detection. - Ninef_EventExposure_Subscribe operation parameters are corrected: - Remove "Target of Event Reporting (SUPI or Internal Group Identifier)" and "Event Filter" and make "Event Reporting Information" optional in the Ninef_EventExposure_Subscribe operation. - Add notification of target address to receive notifications to be sent to the NEF in the output of Ninef_EventNotification_Subscribe. - Change the Ninef_EventExposure service, rename Ninef_EventExposure subscribe/unsubscribe to Ninef_EventRoutingConfiguration Add/Delete, and remove the Ninef_EventExposure Notify service operation to keep NEF unimpacted. - Move “AMF ID (GUAMI)” to optional in Ninef and Namf EventExposure_Notify. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect operation, missing functionality, lack of clarity for stage 3 and misunderstanding. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.3.2.3a, 5.2.2.3.4, 5.2.6A.1, 5.2.6A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** 1st Change ***

4.15.3.2.3a	I-NEF service operations information flow

The procedure is used by the NF (AMF, SMF) in VPLMN to provision monitoring configurations and to explicitly cancel a previous monitoring configuration at I-NEF. Cancelling is done by sending Ninef_EventRoutingConfiguration_Delete request identifying the monitoring configuration to cancel. The notification steps 3 to 5 are not applicable in cancellation case.

Figure 4.15.3.2.3a-1: Ninef_EventRoutingConfiguration_Add and Delete operations

1.	NEF or UDM subscribe or unsubscribe to the notification of monitoring events at SMF or AMF as refered to steps 1 to 2 in clause 4.15.3.2.1, steps 1 to 3 in clause 4.15.3.2.2, and steps 1 to 5 in clause 4.15.3.2.3.

2a-b.	SMF determines that the UE is in roaming and the I-NEF is needed, and it provisions the monitoring configuration in the I-NEF using a Ninef_EventRoutingConfiguration_Add request. The monitoring configuration on the I-NEF includes the notification endpoint information of the NEF that receives the normalized notification reports. When the SMF provisions the monitoring configuration  to the I-NEF, the I-NEF includes a target address to receive the SMFs notifications in the Ninef_EventRoutingConfiguration_Add Response. The SMF updates the monitoring configuration created in step 1 with the notification target address but also keeps the previously stored NEF address in case the need to remove the I-NEF arises later. The I-NEF Monitoring Event routing configuration is removed by sending Ninef_EventRoutingConfiguration_Delete Request.

3a-b.	AMF determines that the UE is in roaming and the I-NEF is needed, and it provisions the monitoring configuration in the I-NEF using a Ninef_EventRoutingConfiguration_Add request. The monitoring configuration on the I-NEF includes the notification endpoint information of the NEF that receives the normalized notification reports. When AMF provisions the monitoring configuration to the I-NEF, the I-NEF includes a target address to receive the AMFs notifications in the Ninef_EventRoutingConfiguration_Add Response. The AMF updates the monitoring configuration created in step 1 with the notification target address but also keeps the previously stored NEF address in case it needs to be restored after inter-AMF mobility that removes the need for I-NEF. The I-NEF Monitoring Event routing configuration is removed by sending Ninef_EventRoutingConfiguration_Delete Request.

4.	[Conditional- depending on the Event] The SMF detects the related event occurs and sends the event report by means of Nsmf_EventExposure_Notify message to the I-NEF.

5.	[Conditional- depending on the Event] The I-NEF performs normalisation of reports and generation of charging/accounting information as defined in clause 6.2.5a of TS 23.501 [2]. The I-NEF sends the event report by routing Nsmf_EventExposure_Notify message to the NEF.

6.	[Conditional- depending on the Event]

The AMF detects the related event occurs and sends the event report by means of Namf_EventExposure_Notify message to the I-NEF.

7.	[Conditional- depending on the Event] The I-NEF performs normalisation of reports and generation of charging/accounting information as defined in clause 6.2.5a of TS 23.501 [2]. The I-NEF sends the event report by routing Nsmf_EventExposure_Notify message to the NEF.

*** Next Change ***

5.2.2.3.4	Namf_EventExposure_Notify service operation

Service operation name: Namf_EventExposure_Notify.

Service operation description: Provides the previously subscribed event information to the NF Consumer which has subscribed to that event before.

Input, Required: Notification Correlation Information, Event ID, corresponding UE(s) (SUPI(s) and if available GPSI(s)), time stamp.

Input, Optional: AMF ID (GUAMI), Event specific parameter list.

Output, Required: None.

Output, Optional: None.

When the AMF detects a UE access and mobility event corresponding to a Subscription, it invokes Namf_EventExposure_Notify service operation to the NF consumer(s) which has subscribed to the UE mobility event before. The event is notified towards the consumers for which the Event filters (which may include "AN type(s)") match. The Notification Target Address (+ Notification Correlation ID) indicates to the Event Receiving NF the specific event notification subscription. The event specific parameter indicates the type of mobility event and related information, e.g. Registration Area Update/new Registration Area.

The optional event specific parameter list provides the values that matched for generating the event notification. The parameter values to match are specified during the event subscription (see clause 5.2.2.3.2). For example if the event type reported is "AN change", the event specific parameter list contains the value of the new AN.

*** Next Change ***

5.2.6A.1	General

The following table shows the I-NEF Services and Service Operations:

Table 5.2.6A.1-1: NF Services provided by the I-NEF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Ninef_EventRoutingConfiguration | Add | Request/Response | AMF, SMF |
|  | Delete |  | AMF, SMF |

5.2.6A.2	Ninef_EventRoutingConfiguration service

5.2.6A.2.1	General

See clause 4.15.3.1.

*** Next Change ***

5.2.6A.2.2	Ninef_EventRoutingConfiguration_Add operation

Service operation name: Ninef_EventRoutingConfiguration_Add

Description: the consumer asks I-NEF to authorize the monitoring event and provide with routing configuration.

Inputs (required): NF ID, (Set of) Event ID(s) as specified in clause 4.15.3.1, Notification Target Address (+ Notification Correlation ID), Subscription Correlation ID.

Inputs (optional): Event Reporting Information defined in Table  4.15.1-1, Expiry time, Chargeable Party Identifier.

Outputs (required): Expiry time (required if the subscription can be expired based on the operator's policy), cause.

Outputs (optional): Notification target address to receive notifications to be sent to the NEF.

*** Next Change ***

5.2.6A.2.3	Ninef_EventRoutingConfiguration_Delete service operation

Service operation name: Ninef_EventRoutingConfiguration_Delete

Description: the NF consumer deletes routing configuration for an event if already defined in I-NEF.

Inputs (required): Subscription Correlation ID.

Outputs (required): Operation execution result indication.

*** Next Change ***

*** End of Changes ***