SA WG2 Meeting #S2-138E	S2-2003028r04

20 - 24 April, 2020, Electronic, Elbonia	(revision of S2-200xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 2257 | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | clarification on CN tunnel for EPS during the I-SMF insertion |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE, Huawei, Ericsson, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | ETSUN |  |  |  |  |  | Date: |  |  | 2020-04-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The I-SMF handling is simliar to V-SMF in home routed roaming. For home routed roaming scenario, if the EBI is assigned successfully, the PGW-C+SMF prepares the CN Tunnel Info for each EPS bearer and provides it to V-SMF. Thus when the UE moves to EPC, the V-SMF does not need to interact with the PGW-C+SMF to get the EPS bearer context(s). (see step 8 in clause 4.11.1.4.1) This also applied to ETSUN case. There is still a scenario, in which how to handle is not clear. In the LBO or non-roaming scenario, there is no I-SMF and the PDU session has been allocated the EBI. It means the PGW-C+SMF does not prepare the CN Tunnel info for each bearer. Later if there is a need to insert an I-SMF, there are several options: - PGW-C+SMF prepares the CN Tunnel Info for each EPS bearer and provides it to I-SMF; - Do nothing in I-SMF insertion. When the next EBI assignment is successfully, the PGW-C+SMF prepares the CN Tunnel Info for each EPS bearer and provide it to I-SMF This CR proposes the option 1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | During I-SMF insertion, if the PGW-C+SMF has not prepared the CN Tunnel Info for each EPS bearer, the PGW-C+SMF requests the PGW-U+UPF to allocate the CN Tunnel for each EPS bearer for the PDU Session. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not clear, when the I-SMF is inserted, whether the PGW-C+SMF prepares the CN Tunnel Info for each EPS bearer or not. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.23.12.44.23.12.x (new), 4.23.12.x.1 (new), 4.23.12.2 (new), 4.23.12.3 (new), 4.23.12.4 (new), 4.23.12.5 (new), 4.23.12.6 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

#### 4.23.12.4	Procedures for EPS bearer ID allocation

The EPS Bearer ID allocation procedure, EPS Bearer ID transfer procedure and EPS Bearer ID revocation procedure are perfomed as described for the home-routed roaming case in clause 4.11.1.4, with the following differences:

-	H-SMF is replaced by SMF.

-	V-SMF is replaced by I-SMF.

-	In Step 8 of clause 4.11.1.4.1, in addition to home routing roaming scenario, for scenario of deployments topologies with specific SMF Service Areas, if EBI is assigned successfully, the PGW-C+SMF also prepares CN Tunnel Info for each EPS bearer and provides this information to the I-SMF.

NEXT CHANGE

#### 4.23.12.X	Impact to 5GC procedures

##### 4.23.12.X.1	General

This clause captures impact to 5GC procedures in other subclauses of clause 4.23 to support interworking with N26.

##### 4.23.12.X.2	UE Triggered Service Request with I-SMF insertion/change/removal

The following impact is applicable for UE triggered Service Request with I-SMF insertion in clause 4.23.4.3:

-	In step 20a, when an I-SMF is inserted, if EBI(s) have been allocated before but the PGW-C+SMF has not prepared the CN Tunnel Info for each EPS bearer, the PGW-C+SMF requests the PGW-U+UPF to allocate the CN Tunnel for each EPS bearer for PDU Session(s). PGW-U+UPF allocates the PGW-U tunnel info for the EPS bearer and sends it to the PGW-C+SMF.

-	In step 20c, the PGW-C+SMF provides also CN Tunnel Info for each EPS bearer to the I-SMF.

NOTE:	The CN Tunnel Info for each EPS bearer provided to the I-SMF is to prepare forUE mobility to EPC network so that the I-SMF does not need interact with the PGW-C+SMF to get the EPS bearer context(s) at mobility to EPC.

##### 4.23.12.X.3	PDU Session establishment procedure

The following impact is applicable for PDU Session establishment in clause 4.23.5.1:

##### -	If I-SMF is involved in the PDU Session establishment, after EBI has been  successfully allocated, the PGW-C+SMF also prepares CN tunnel info for each EPS beare and provides it to the I-SMF r, see clause 4.23.12.4.

##### 4.23.12.X.4	PDU Session modification procedure

The following impact is applicable for PDU Session modification in clause 4.23.5.3:

-	If I-SMF is involved in the PDU Session modification, if EBI needs to be allocated and the allocation is successful, the PGW-C+SMF also prepares CN tunnel info for each EPS bearer and provides it to the I-SMF, see clause 4.23.12.4.

##### 4.23.12.X.5	Inter NG-RAN node N2 based handover with I-SMF insertion/change/removal

The following impact is applicable for Inter NG-RAN node N2 based handover with I-SMF insertion in clause 4.23.7.3.2:

-	In step 7c, at I-SMF insertion, if EBI(s) have been allocated before but the PGW-C+SMF has not prepared the CN Tunnel Info for each EPS bearer, the PGW-C+SMF requests the PGW-U+UPF to allocate the CN Tunnel for each EPS bearer for PDU Session(s). PGW-U+UPF allocates the PGW-U tunnel info for the EPS bearer and sends it to the PGW-C+SMF.

-	In step 7f, the PGW-C+SMF provides also the CN Tunnel Info for each EPS bearer to the I-SMF.

##### 4.23.12.X.6	Xn based handover with insertion of I-SMF

The following impact is applicable Xn based handover with insertion of I-SMF in clause 4.23.11.2:

-	In step 6, at I-SMF insertion, if EBI(s) have been allocated before but the PGW-C+SMF has not prepared the CN Tunnel Info for each EPS bearer, the PGW-C+SMF requests the PGW-U+UPF to allocate the CN Tunnel for each EPS bearer for PDU Session(s). PGW-U+UPF allocates the PGW-U tunnel info for the EPS bearer and sends it to the PGW-C+SMF.

-	In step 9, the PGW-C+SMF provides also the CN Tunnel Info for each EPS bearer to the I-SMF.

END OF CHANGE