SA WG2 Meeting #S2-138E	S2-2003035

20 - 24 April, 2020, Electronic, Elbonia	(revision of S2-200xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2321 | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on the bridge delay |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Vertical_LAN |  |  |  |  |  | Date: |  |  | 2020-04-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The residence time in the UPF has been covered by PDB/CN-PDB, so the Residence times only may vary among UEs. According to the moderated email discussion, it is endorsed PSFP based Hold and Forward Buffering rule is not supported in the Rel-16. So the values of independentDelayMax and independentDelayMin for 5GS Bridge can not be same to avoid the sequence issue. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removing the Residence times vary among UPF. Clarify the independentDelayMax and independentDelayMin for 5GS Bridge can be different. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misunderstanding on the Residence time in the UPF. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.27.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

5.27.5	5G System Bridge delay

In order for the 5G System to participate as a TSN bridge according to gate schedules specified, the 5GS Bridge is required to provide Bridge Delays as defined in IEEE 802.1Qcc [95] for each port pair and traffic class of the 5GS bridge to an IEEE TSN system. In order to determine 5GS Bridge Delays, the following components are needed:

1.	UE-DS-TT Residence Time: the time taken within the UE and DS-TT to forward a packet between the UE and DS-TT port. UE-DS-TT Residence Time is provided at the time of PDU Session Establishment by the UE to the network.

NOTE 1:	UE-DS-TT Residence Time is the same for uplink and downlink traffic and applies to all traffic classes.

2.	Per traffic class delay between the UE and the UPF/NW-TT that terminates the N6 interface (including UPF and NW-TT residence times), independent of frame length that a given 5GS deployment supports. The per-traffic class delay between the UE and the UPF/NW-TT is pre-configured in the TSN AF (see clause 5.28.4).

The TSN AF calculates the 5GS independentDelayMin and independentDelayMax values for each port pair and for each traffic class using the above components.

NOTE 2:	With supporting the hold and forward buffering mechanism in UE-DS-TT and NW-TT, the values of independentDelayMax and independentDelayMin for 5GS Bridge can be the same.

The dependentDelayMin and dependentDelayMax for 5GS Bridge specify the time range for a single octet of an Ethernet frame to transfer from ingress to egress and include the time to receive and store each octet of the frame, which depends on the link speed of the ingress Port as per IEEE 802.1Qcc [95].

NOTE 3:	Further details how TSN AF determines dependentDelayMin and dependentDelayMax and is up to implementation.

Since Residence times may vary among UEs, the 5GS Bridge Delay is determined after the PDU Session Establishment for the corresponding UPF and the UE by the TSN AF. The TSN AF deduces the related port pair(s) from the port number of the DS-TT Ethernet port and port number of the serving NW-TT Ethernet port(s) when the TSN AF receives the 5GS Bridge information for a newly established PDU Session and caculates the bridge delays per port pair.

END OF CHANGES