3GPP TSG-WG SA2 Meeting #138E e-meeting  	S2-2003090

# Elbonia, April 20 – 23, 2020	(revision of S2-200xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 2278 | rev | - | Current version: | 15.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on Deregistration procedures for SMS over NAS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1 |  |  |  |  |  | Date: |  |  | 2020-04-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This CR proposes the following changes: - Consider the Access Type over which the SMS is deactivated. According to the clause 4.13.3.2, the SMS deactivation is triggered by Deregistration or Registration with UE’s capability change for SMS over NAS. The general Registration or Deregistration is associated with Access Type (i.e., 3GPP Access or non-3GPP Access). But current SMS deactivation in clause 4.13.3.2 didn’t differ different accesses. Hence once UE deregister from one access type, the SMS will be deactivated from all access types, which results in that the SMS is no longer allowed to be used by the UE via another available access type. - Some editorial changes to correct the Nudr_DM_Unsubscribe service operation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to consider the Access Type over which the SMS is deactivated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Deregistration on one of the access types will cause SMS service become completely unusable to the UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.13.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

4.13.3.2	Deregistration procedures for SMS over NAS

If UE indicates to AMF that it no longer wants to send and receive SMS over NAS for specific Access Type(s) (e.g., not including "SMS supported" indication in subsequent Registration Request message) or AMF considers that UE is deregistered on specific Access Type(s) or AMF receives Deregistration Notification from UDM for specific Access Type(s) indicating UE Initial Registration, Subscription Withdrawn or 5GS to EPS Mobility as specified in clause 5.2.3.2.2, then:

-    AMF may, if the UE is not registered at other Access Type for SMS over NAS service at the AMF any more, unsubscribe from SMS Subscription data changes notification with the UDM by means of the Nudm_SDM_Unsubscribe service operation. If that, the UDM may remove the corresponding subscription of data change notification in UDR by Nudr_DM_Unsubscribe service operation.

-    AMF invokes, for the impacted Access Type(s), Nsmsf_SMService_Deactivate service operation to trigger the release of UE Context for SMS on SMSF based on local configurations.

-    AMF may, if the UE is not registered at other Access Type for SMS over NAS service at the AMF anymore, delete or deactivate the stored SMSF address in its UE Context.

-    The SMSF unsubscribes from SMS Management Subscription data changes notification with the UDM by means of the Nudm_SDM_Unsubscribe service operation if the UE is not registered at other Access Type for SMS over NAS service  at the SMSF anymore. If that, the UDM may remove the corresponding subscription of data change notification in UDR by Nudr_DM_Unsubscribe service operation.

-    The SMSF shall invoke Nudm_UECM_Deregistration (SUPI, NF ID, Access Type) service operation from UDM to trigger UDM to delete SMSF address of the UE for the impacted Access Type(s).

-    The UDM may update UE context in SMSF in UDR by Nudr_DM_Update (SUPI, Subscription Data, SMS Subscription data, SMSF address). The SMSF also removes the UE Context for SMS for the impacted Access Type(s), including AMF address.

# * * * * End of changes * * * *