3GPP TSG-SA2 Meeting #138-E	S2-2003181

# Online, , 20th Apr 2020 - 23rd Apr 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.287 | CR | DUMMY | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | V2X server discovery for PC5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo Mobile Communication Co. LTD, Qualcomm Incorporated, Tencent, LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Dummy |  |  |  |  |  | Date: |  |  | 2020-04-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | V2X services can be either IP or non IP based (unstructured). The same V2X service can run over Uu or PC5, both Uu and PC5 allow for IP and non-IP based transport. V2X services that consist of an application server component and use IP as a transport e.g. J2945/3 (https://www.sae.org/standards/content/j2945/3/) the client (typically the car/UE) needs to discover the application server. Section 5.3 of TS 23.287 describes how Application Server discovery is performed. However this section only describes how this is performed when Uu is used. PC5 also allows IP transport but it supports no application server discovery in TS 23.287 which does not fulfil requirement R.5.1-024 from TS 22.186. Furthermore in order to use an FQDN other IP configuration information needs to be provisioned e.g. DNS server address. To summarises there is no server discovery capabilities when PC5 is used. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the PC5 provisioning section to allow an application server FQDN and or IP address’s to be defined per service. Add ability to define default gateway and DNS address so that the FQDN can be resolved. Update section on Application Server discovery to reference new PC5 provisioning information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | PC5 based IP services are unable to perform V2X application server discovery, resulting in no service being possible. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.2.1, 5.1.3.1, 5.3.1, 5.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 5.1.2	Authorization and Provisioning for V2X communications over PC5 reference point

#### 5.1.2.1	Policy/Parameter provisioning

The following sets of information for V2X communications over PC5 reference point is provisioned to the UE:

1)	Authorization policy:

-	When the UE is "served by E-UTRA" or "served by NR":

-	PLMNs in which the UE is authorized to perform V2X communications over PC5 reference point when "served by E-UTRA" or "served by NR".

For each above PLMN:

-	RAT(s) over which the UE is authorized to perform V2X communications over PC5 reference point.

-	When the UE is "not served by E-UTRA" and "not served by NR":

-	Indicates whether the UE is authorized to perform V2X communications over PC5 reference point when "not served by E-UTRA" and "not served by NR".

-	RAT(s) over which the UE is authorized to perform V2X communications over PC5 reference point.

2)	Radio parameters when the UE is "not served by E-UTRA" and "not served by NR":

-	Includes the radio parameters per PC5 RAT (i.e. LTE PC5, NR PC5) with Geographical Area(s) and an indication of whether they are "operator managed" or "non-operator managed". The UE uses the radio parameters to perform V2X communications over PC5 reference point when "not served by E-UTRA" and "not served by NR" only if the UE can reliably locate itself in the corresponding Geographical Area. Otherwise, the UE is not authorized to transmit.

Editor's note:	The radio parameters (e.g. frequency bands) are to be defined by RAN WGs. The reference to RAN specification will be added when defined in RAN WGs.

NOTE 1:	Whether a frequency band is "operator managed" or "non-operator managed" in a given Geographical Area is defined by local regulations.

3)	Policy/parameters per RAT for PC5 Tx Profile selection:

-	The mapping of V2X service types (e.g. PSIDs or ITS-AIDs) to Tx Profiles.

Editor's note:	The Tx Profiles are to be defined by RAN WGs. The reference to RAN specification will be added when defined in RAN WGs.

4)	Policy/parameters related to privacy:

-	The list of V2X service types, e.g. PSIDs or ITS-AIDs of the V2X applications, with Geographical Area(s) that require privacy support.

5)	Policy/parameters when LTE PC5 is selected:

Same as specified in TS 23.285 [8] clause 4.4.1.1.2 item 3) Policy/parameters except for the mapping of V2X service types to Tx Profiles and the list of V2X services with Geographical Area(s) that require privacy support.

6)	Policy/parameters when NR PC5 is selected:

-	The mapping of V2X service types (e.g. PSIDs or ITS-AIDs) to V2X frequencies with Geographical Area(s).

-	The mapping of Destination Layer-2 ID(s) and the V2X service types, e.g. PSIDs or ITS-AIDs of the V2X application for broadcast.

-	The mapping of Destination Layer-2 ID(s) and the V2X service types, e.g. PSIDs or ITS-AIDs of the V2X application for groupcast.

-	The mapping of default Destination Layer-2 ID(s) for initial signalling to establish unicast connection and the V2X service types, e.g. PSIDs or ITS-AIDs of the V2X application.

NOTE 2:	The same default Destination Layer-2 ID for unicast initial signalling can be mapped to more than one V2X service types. In the case where different V2X services are mapped to distinct default Destination Layer-2 IDs, when the UE intends to establish a single unicast link that can be used for more than one V2X service types, the UE can select any of the default Destination Layer-2 IDs to use for the initial signalling.

-	PC5 QoS mapping configuration:

-	Input from V2X application layer:

-	V2X service type (e.g. PSID or ITS-AID).

-	(Optional) V2X Application Requirements for the V2X service type, e.g. priority requirement, reliability requirement, delay requirement, range requirement.

NOTE 3:	Details of V2X Application Requirements for the V2X service type is up to implementation and out of scope of this specification.

-	Output:

-	PC5 QoS parameters defined in clause 5.4.2 (i.e. PQI and conditionally other parameters such as MFBR/GFBR, etc).

-	SLRB configurations, i.e. the mapping of PC5 QoS profile(s) to SLRB(s), when the UE is "not served by E-UTRA" and "not served by NR".

-	The PC5 QoS profile contains PC5 QoS parameters described in clause 5.4.2, and value for the QoS characteristics regarding Priority Level, Averaging Window, Maximum Data Burst Volume if default value is not used as defined in Table 5.4.4-1.

6a)	The parameters as defined in clause 5.1.3.1 items a) and b). These parameters shall be made available to the UE in the following ways:

-	pre-configured in the ME;

-	configured in the UICC;

-	preconfigured in the ME and configured in the UICC; or

-	provided/updated by the V2X Application Server via V1 reference point.

NOTE 4:	The values used for V2X communications over PC5 reference point could be different to the values for V2X communications over Uu reference point.

7)	Validity timer indicating the expiration time of the V2X Policy/Parameter.

Editor's note:	The SLRB configurations will be determined by RAN WGs. The reference to RAN specification will be added when defined in RAN WGs.

Editor's note:	For the PC5 QoS profile, coordination with RAN WGs is needed.

Editor's note:	The V2X frequencies with Geographical Area(s) will be determined by RAN WGs. The reference to RAN specification will be added when defined in RAN WGs.

The above parameter sets from bullet 2) to 6a) may be configured in the UE through the V1 reference point by the V2X Application Server.

****NEXT CHANGE****

#### 5.1.3.1	Policy/Parameter provisioning

The following set of information may be provisioned to the UE for V2X communications over Uu reference point:

1)	Mapping of the V2X service types (e.g. PSIDs or ITS-AIDs of the V2X application) to:

-	PDU Session Type (i.e. IP type or Unstructured type);

-	Transport layer protocol (i.e. UDP or TCP, only applicable for IP PDU Session type);

-	SSC Mode;

-	S-NSSAI(s);

-	DNN(s).

NOTE:	Above listed information elements are optional and used by UE as UE Local Configuration specified in TS 23.503 [16].

2)	Validity timer indicating the expiration time of the V2X Policy/Parameter.

The following sets of information may be provisioned to the UE and is applicable for V2X communications over both LTE-Uu and Uu reference points:

a)	Mapping of the V2X service types (e.g. PSIDs or ITS-AIDs of the V2X application) to V2X Application Server address information (consisting of IP address/FQDN and transport layer port#) for unicast.

b)	List of FQDNs or IP addresses of the V2X Application Servers, associated with served geographical area information and list of PLMNs that the configuration applies to.

****NEXT CHANGE****

## 5.3	V2X Application Server discovery

### 5.3.1	General

A UE needs to discover the V2X Application Server(s), when V2X communication over Uu reference point or V2X communication over PC5 reference point is used. The V2X Application Server address information may be configured on the UE or provisioned over N1 reference point, as specified in clause 5.1.3.1 when Uu reference point is used, or as specified in clause 5.1.2.1 when PC5 reference point is used.

When the configuration contains the FQDN(s), the UE shall perform DNS to resolve the address(es) of the V2X Application Server. The UE may use the configured V2X Application Server information only in the designated geographical area. When the UE changes serving PLMN or crosses configured geographic areas, it should perform address resolution again.  When using PC5 Unicast as specified in clause 5.2.1.4, the DNS address used to resolve the FQDN shall be that of UE-B.

NOTE 1:	When the V2X Application Server is notified by SMF indicating the DNAI change, as specified in clause 5.6.7 of TS 23.501 [6], the application layer can trigger the UE to perform DNS to resolve the address(es) of the V2X Application Server.

NOTE 2:	Annex X provides deployment example scenario related to communicating with V2X Application Server over PC5 reference point.

### 5.3.2	Multiple V2X Application Server and Localized V2X Application Server discovery and routing

Multiple V2X Application Servers may be involved in the V2X communication, each providing particular V2X services and/or serving a particular geographical region. Therefore, the V2X Application Server address information as specified in clause 5.1.3.1 for V2X communication over Uu reference point or clause 5.1.2.1 for V2X communication over  PC5 reference point can contain multiple servers' information. When multiple V2X Application Servers are configured, the application layer will choose the proper V2X Application Server to use.

When localized V2X Application Servers are deployed, Anycast may be used to conceal the server change from the UE. In this case, a FQDN is configured for a large region, e.g. the entire PLMN, and the UE only needs to resolve it once to an Anycast address. The UPF is responsible for routing the traffic to the appropriate local V2X Application Servers based on Anycast address.

* * * * End of  Change * * * *