SA WG2 Meeting #139E	S2-2003651

Elbonia, June 1- 12, 2020	(revision of S2-2000733)

Source:	Apple

Title:	KI #1, New Sol: Using Non-3GPP access to deliver NAS Notification message

Document for:	Approval

Agenda Item:	8.10

Work Item / Release:	FS_MUSIM / Rel-17

Abstract of the contribution: This solution proposes to use the procedure in TS 23.502 clause 4.2.3.3 step 4c when the Multi-USIM UE is in CM-CONNECTED state for non-3GPP access, the network needs to signal with the Multi-USIM UE, and the Multi-USIM UE has left the current 3GPP system for 3GPP access.

# 1	Background

This contribution proposes a solution to Key Issue #1: Handling of Mobile Terminated service with Multi-USIM device.

* * * * Start of Changes * * * *

## 6.0	Mapping Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 |
| 1 | X |  |  |  |
| 2 | X |  |  |  |
| 3 | X |  |  |  |
| 4 |  |  | X |  |
| 5 |  |  | X |  |
| 6 |  |  | X |  |
| 7 | X | X |  |  |
| 8 | X |  |  |  |
| X | X |  |  |  |

* * * * Next Change (all new text) * * * *

## 6.X	Solution #X: Using Non-3GPP access to deliver NAS Notification message

### 6.X.1	Introduction

This solution addresses the Key Issue #1 "Handling of Mobile Terminated service with Multi-USIM device".

The current specification TS 23.502 [6] clause 4.2.3.3 step 4c covers the case when the UE is in CM-IDLE state in 3GPP access and in CM-CONNECTED state in non-3GPP access, the network may initiate a Network Triggered Service Request procedure for 3GPP access via non-3GPP access.

### 6.X.2	Functional Description

When the network needs to signal (e.g. N1 signalling to UE, Mobile-terminated SMS, User Plane connection activation for PDU Session(s) to deliver mobile terminating user data) with the Multi-USIM UE, and if the Multi-USIM UE:

1.	is simultaneously registered over 3GPP and non-3GPP accesses in the same PLMN;

2.	is in CM-CONNECTED state for non-3GPP access;

3.	has left the current 3GPP system for 3GPP access; and

4.	the PDU Session ID is associated with 3GPP access:

the AMF shall notify the UE through non-3GPP access, i.e. by sending a NAS Notification message containing the 3GPP Access Type to the UE over non-3GPP access and set a Notification timer.

### 6.X.3	Procedures

Network Triggered Service Request procedure in TS 23.502 [6] clause 4.2.3.3 step 4c applies.

### 6.X.4	Impacts on existing entities and interfaces

UE: does not need to simultaneously monitor paging on all 3GPP RATs.

### 6.X.5	Evaluation

Editor's note:	This clause provides an evaluation of the solution.

* * * * End of Changes * * * *