3GPP TSG-WG SA2 Meeting #139E e-meeting  	S2-2003990

# Elbonia, June 1 - 12, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.737 | CR |  | rev |  | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Conclusion on solutions for Key Issue #2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | TNO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_5GSAT_ARCH |  |  |  |  |  | Date: |  |  | 2020-05-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Key Issue #2 conclusions are updated to account for the responses from 3GPP TSG RAN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Conclusion for Key Issue #2 is updated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete conclusion. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** Begin of Change ***

## 8.2	Conclusion on solutions for Key Issue #2

Key Issue #2 studied Mobility Management with moving satellite coverage area. 3GPP system should adapt to being usable over variety of satellite types without putting pre-requisite conditions on the capabilities of the satellites, or the satellite constellation to be used. The system impact on 3GPP system depends on what kind of satellites are used, and therefore, the following conclusions are made.

Both Solutions #1 and #9 have been identified as having no impacts on the existing 3GPP specifications of the 5G-CN. It is thus concluded that no normative work is needed in SA2 for these solutions. Feedback and evaluation of these solutions has also been requested to the RAN Working Group.

Solutions #7 and #8 have both impacts on the 5G-CN. Dependency with respect to Moving Tracking Areas in the case of Satellite based RAN has been identified for these solutions.

RAN2 WG recommends tracking areas as being fixed on Earth (see TR 38.821, clause 7.3.1.3.3 ‘Tracking Area recommendation’). Consequently, Solution#7 and Solution#8 are not needed in Rel. 17.

*** End of Change ***