SA WG2 Meeting #S2-139-e	S2-2003635r09

01 - 12 June, 2020, Electronic meeting	(revision of S2-200xxxx)

Source:	Intel

Title:	KI #1, Sol #1: Update to remove ENs

Document for:	Approval

Agenda Item:	8.10

Work Item / Release:	FS_MUSIM / Rel-17

Abstract of the contribution: Proposes an update to Sol #1 to remove ENs.

# 1	Discussion

Solution #1 (Paging Cause) has several Editor’s notes.

Editor's note:	The use of PLMN-specific Paging Cause values is FFS.

For simplicity it is proposed in Rel-17 to agree at least a set of standardized values for the Paging Cause. An example of possible values is provided in the table below:

Table 1: Paging cause values

| Paging cause Value | Type of downlink traffic |
| --- | --- |
| 1 | Voice service |
| 2 | SMS |
| 3 | IMS service other than voice or SMS |
| 4 | CP signaling |
| 5 | Other-data |

In a future release the meaning of cause value “5” (Other data) can be further expanded into “6” (Other data 2), “7” (Other data 3), etc. and the meaning of these values can be agreed between the UE and the network.

Proposal 1: Delete the EN on PLMN-specific Paging Cause values and replace it with a NOTE stating that only standardized values are used in Rel-17.

Editor's note:	It is FFS whether the SMF takes into account additional information (e.g. the source IP address in the IP packet) to determine the Paging Cause.

Assuming that the standardized Paging Cause values are limited to those listed in Table 1, there is no need for the SMF to use additional information for determining the Paging Cause value. If in future release the set of Paging Cause values is extended to include non-standardized values negotiated by the UE and the network, then the use of additional information (e.g. source IP address) may be required.

Proposal 2: Delete the EN on the use of additional information for determining the Paging Cause.

Editor's note:	It needs to be confirmed with SA3 whether exposing the Paging Cause in clear poses a security issue.

This EN is clearly in the scope of SA WG3.

Proposal 3: It is proposed to convert the EN into a NOTE.

Editor's note:	It is FFS how to support per service DSCP and whether the use of DSCP for determination of the Paging Cause by NG-RAN can provide sufficient granularity for all Paging Cause values.

Editor's note:	It is FFS whether there is a need to support the Paging Cause for UE in RRC_Inactive. There is a dependency with the key issue on "coordinated leaving".

These two ENs are related to support of Paging Cause in RRC_Inactive state. There are two questions to resolve:

Q1.	Whether Paging Cause in RRC_Inactive needs to be supported.

Q2.	Which alternative is used for support of RRC_Inactive: DSCP or inclusion of Paging Cause in the N3 header.

Regarding Q1 there is a dependency on the “coordinated leaving” solution. Consider the case where UE A (of a Multi-USIM device) is paged while UE B (of the same Multi-USIM device) is in Connected state. UE B requests “coordinated leaving” in system B and UE A responds to paging in system A. Depending on the selected solution for “coordinated leaving”, UE B can remain in either RRC_Idle or RRC_Inactive, or only in RRC_Idle:

a.	If the solution for “coordinated leaving” allows UE B to remain in RRC_Inactive state then the Paging Cause needs to be supported in RRC_Inactive state in order to have a consistent feature. Namely, in the example above, if UE B is now paged in system B while UE A is communicating in system A, the multi-USIM device needs to make a new decision whether to respond to the paging in system B. Without a Paging Cause, it will not be able to do so.

b.	If the solution for “coordinated leaving” always leaves UE B in RRC_Idle state then the Paging Cause does not need to be supported in RRC_Inactive state.

At this point it is not possible to answer Q1 (i.e. not possible to resolve the second EN above). However, assuming that Paging Cause is supported in RRC_Inactive, we propose to answer Q2 by simplifying the solution and keeping only one of the two alternatives.

The RRC_Inactive alternative based on DSCP assumes that the DSCP code of N3 GTP-U header is used by the RAN to determine the Paging Cause. In our view this approach has issues in that it requires that certain “3GPP level” information (i.e. the Paging Cause) be reflected in the DSCP marking of the transport layer. It also requires consistent pre-configuration across RAN and CN nodes. It may also not be future-proof if the Paging Cause value range is extended in future releases.

We propose to keep the solution based on inclusion of Paging Cause in the N3 header. If this is agreeable, then the first EN above can be removed.

Proposal 4: Remove the first EN (DSCP related). Remove the corresponding alternative for carrying Paging Cause as part of the DSCP marking. Keep the second EN (dependency with “coordinated leaving”).

The proponents of the Paging Cause solution(s) may have different views on how the Paging Cause value should map to specific MT services. To progress the discussion on theis point it is proposed to add an Annex for collection of Paging Cause mapping proposals. NOTE 1 currently states that the granularity of the Paging Cause values will be determined in coordination with SA1, however, to avoid lengthy liaison exchanges, SA2 should attempt to resolve this point on its own and involve SA1 only if needed.

Proposal 5: Add an Annex for collection of Paging Cause mapping proposals. Add “if needed” in NOTE 1.

# 2	Proposals

It is proposed to agree the text proposal below.

* * * * Start of Change * * * *

## 6.1	Solution #1: Handling of MT service with Paging Cause

### 6.1.1	Introduction

The solution applies to Key Issue #1 "Handling of MT service".

The solution applies to both 5GS (UE in either CM_IDLE or RRC_Inactive state) and EPS (UE in CM_IDLE state only).

### 6.1.2	Functional Description

The solution is based on a Paging Cause that is delivered to the UE as part of the [Uu] Paging message.

NOTE 1:	The granularity of the paging information in the Paging Cause will be coordinated with SA WG1 input, if needed.

NOTE 2:	Based on the Paging Cause and the service preferences configured by the user or a pre-configured logic specific to the Multi-USIM device, the Multi-USIM device that is actively engaged in communication associated with another USIM can decide whether to present the mobile terminated service that triggered the paging to the user. Alternatively, the Multi-USIM device can systematically present the mobile terminated service that triggered the paging to the user, in which case it is up to the user to decide whether to respond to the paging request.

NOTE y:	In this release, only the operator managed services, e.g. IMS voice, is considered to be indicated in paging cause and only standardized values are used for the Paging Cause. This does not preclude the use of a specific Paging Cause value for “Other” services.

For a UE in CM_IDLE state:

-	For MT user plane traffic as part of the Network Triggered Service Request procedure, and if Paging Policy Differentiation (PPD) applies, the SMF determines Paging Policy Indicator (PPI) and optionally determines a Paging Cause value based on the DSCP received from the UPF. The SMF includes the Paging Cause, along with the PPI, the ARP and the 5QI of the corresponding QoS Flow, in the N11 message sent to the AMF. The AMF uses this information to derive a paging strategy and sends paging messages to NG-RAN over N2. The AMF shall forward the Paging Cause in the PAGING message to NG-RAN if it was received from the SMF.

Editor’s note:	Whether exposing the Paging Cause in clear poses as security issue will be determined by SA WG3.

NOTE:	It will be determined whether the Paging Cause can be used only for UEs that have requested MUSIM assistance or unconditionally.

-	For MT control plane traffic (e.g. MT SMS over NAS, or NAS signaling) the AMF derives the paging strategy and Paging Cause based on the type of MT control plane traffic and forwards the Paging Cause in the PAGING message to NG-RAN.

For a UE in RRC_Inactive state:

-	For MT user plane traffic the SMF instructs the UPF to detect the DSCP in the TOS (IPv4) / TC (IPv6) value in the IP header of the DL PDU and to transfer the corresponding PPI and optionally the Paging Cause in the CN tunnel header (by using a FAR with the PPI and Paging Cause value). The NG-RAN can then utilize the PPI received in the CN tunnel header of an incoming DL PDU in order to apply the corresponding paging policy for the case the UE needs to be paged when in RRC Inactive state. If the Paging Cause was included in the CN tunnel header of an incoming DL PDU the NG-RAN forwards the Paging Cause to the UE for the case the UE needs to be paged when in RRC Inactive state.

NOTE 3:	The Paging Cause is included in the CN tunnel header in all data packets.

-	Alternatively, the NG-RAN determines the Paging Cause based on specific 5QI (i.e. 5QI=1 or 5) of the QoS flows for the downlink data packet.

Editor's note: Since all the IMS signalling is transmited in specific QoS Flow, it is FFS whether specifc 5QI value can be used to determine the paging cause for voice service.

Editor's note:	It is FFS whether there is a need to support the Paging Cause for UE in RRC_Inactive. There is a dependency with the key issue on "coordinated leaving".

-	For MT control plane traffic (e.g. MT SMS over NAS, or NAS signaling) the AMF derives the Paging Cause based on the type of MT control plane traffic and forwards the Paging Cause in the DOWNLINK NAS TRANSPORT message to NG-RAN.

The solution can also be used in EPS with the following changes:

-	It applies to UE in CM_IDLE only.

-	AMF, SMF and UPF in the description above are replaced with MME, SGW-C and SGW-U, respectively.

6.1.3	Procedures

#### 6.1.3.1	Handling of MT service with Paging Cause for UE in CM_Idle in 5GS

The solution has impact on the Network Triggered Service Request procedure in TS 23.502 [6] clause 4.2.3.3. The changes relative to the existing procedure are indicated in bold underlined text. Only the impacted steps are shown.

Figure 6.1.3.1-1: Network Triggered Service Request (based on TS 23.502 [6] Figure 4.2.3.3-1)

2c.	The UPF forwards the downlink data packets towards the SMF if the SMF instructed the UPF to do so (i.e. the SMF will buffer the data packets).

-	If the Paging Policy Differentiation feature is supported by the SMF and if the PDU Session type is IP, the SMF determines the Paging Policy Indicator and optionally a Paging Cause based on the DSCP in TOS (IPv4) / TC (IPv6) value from the IP header of the received downlink data packet and identifies the corresponding QoS Flow from the QFI of the received DL data packet.

3a.	[Conditional] SMF to AMF: Namf_Communication_N1N2MessageTransfer (SUPI, PDU Session ID, N1 SM container (SM message), N2 SM information (QFI(s), QoS profile(s), CN N3 Tunnel Info, S-NSSAI), Area of validity for N2 SM information, ARP, Paging Policy Indicator, Paging Cause, 5QI, N1N2TransferFailure Notification Target Address, Extended Buffering support), or NF to AMF: Namf_Communication_N1N2MessageTransfer (SUPI, N1 message).

[…]

When supporting Paging Policy Differentiation, the SMF determines the Paging Policy Indicator and may also determine a Paging Cause related to the downlink data that has been received from the UPF or triggered the Data Notification message, based on the DSCP as described in TS 23.501 [4] clause 5.4.3, and indicates the Paging Policy Indicator and the Paging Cause in the Namf_Communication_N1N2MessageTransfer.

4b.	[Conditional] If the UE is in CM-IDLE state in 3GPP access and the PDU Session ID received from the SMF in step 3a has been associated with 3GPP access and based on local policy the AMF decides to notify the UE through 3GPP access even when UE is in CM-CONNECTED state for non-3GPP access, the AMF may send a Paging message to NG-RAN node(s) via 3GPP access including the Paging Cause.

4c.	If the UE is simultaneously registered over 3GPP and non-3GPP accesses in the same PLMN, and the UE is in CM-CONNECTED state for non-3GPP access and in CM-IDLE for 3GPP access, the AMF may decide to send the NAS Notification message containing the 3GPP Access Type to the UE over non-3GPP access including the Paging Cause.

4d.	If the UE is simultaneously registered over 3GPP and non-3GPP accesses in the same PLMN, and the UE is in CM-CONNECTED state for non-3GPP access and in CM-IDLE for 3GPP access and if the UE decides to not accept the incoming service the UE shall respond with NAS Notification response message over the non-3GPP access to indicate the same to the network.

6.	The UE may choose to respond to paging or NAS notification message based on paging cause value by executing service request procedure.

#### 6.1.3.2	Handling of MT service with Paging Cause in RRC_Inactive mode

Figure 6.1.3.2-1is the call flow of handling of MT service with Paging Cause in RRC_Inactive mode.

Figure 6.1.3.2-1 Handling of MT service with Paging Cause in RRC_Inactive mode

1.	NG-RAN receives the DL data (control plane data and/or user plane data) in RRC_Inactive mode. If handling of MT service with Paging Cause is supported by NG-RAN, NG-RAN determines the Paging Cause based on the Paging Cause field included in the CN tunnel header of an incoming DL PDU. Alternatively, the NG-RAN determines the Paging Cause  based on specific 5QI (i.e. 5QI=1 or 5) of the QoS flows for the downlink data packet.

Editor's note: Since all the IMS signalling is transmited in specific QoS Flow, it is FFS whether specifc 5QI value can be used to determine the paging cause for voice service.

NG-RAN sends the paging message with the Paging Cause.

#### 6.1.3.3	Handling of MT service with Paging Cause in EPS

Figure 6.1.3.3-1 is handling of MT service with Paging Cause in EPS.

Figure 6.1.3.3-1: Handling of MT service with Paging Cause in EPS

1.	If the handling of MT service with Paging Cause is supported by Serving GW, Serving GW determines the Paging Cause based on DSCP in TOS (IPv4)/TC (IPv6) value from the IP header of the downlink data packet. Alternatively, if the Serving GW supports the Paging Policy Differentiation feature, then the Serving GW unconditionally, for each bearer and for each packet of PDN type IPv4, IPv6 or IPv4v6 that triggers a Downlink Data Notification, sends the DSCP in TOS (IPv4) / TC (IPv6) information received in the IP payload of the GTP-U packet from the PDN GW in the Paging Policy Indication in the Downlink Data Notification.

2.	SGW includes the Paging Cause in the DDN message sent from SGW to MME. If the Paging Cause is not received, but the Paging Policy Indication is received, the MME determines the Paging Cause taking the configuration for that HPLMN and/or APN and/or QCI into account.

For mobile terminating signalling and SMS over NAS, the MME determines an appropriate Paging Cause.

3.	MME sends S1 paging message including the Paging Cause information.

4.	RAN sends the paging message with Paging Cause.

### 6.1.4	Impacts on services, entities and interfaces

For 5G:

AF:

-	P-CSCF sets the DSCP value in the IP header to indicate the traffic type.

SMF:

-	optionally, determines Paging Cause based on DSCP value from IP header and HPLMN/APN/QCI configuration and includes the Paging Cause in DDN sent to AMF..

AMF:

-	determines the Paging Cause for NAS SMS and MT control plane traffic . For user plane traffic the AMF either receives the Paging Cause from the SMF or determines a Paging Cause  based on HPLMN/APN/QCI configuration (e.g. 5QI=1 or 5) received from the SMF.

-	sends the N2 paging signalling with Paging Cause.

NG-RAN:

-	sends the paging message with Paging Cause.

-	in RRC_Inactive mode, NG-RAN determines the Paging Cause based on the Paging Cause field included in the CN tunnel header of an incoming DL PDU. Alternatively, the NG-RAN determines the Paging Cause based on specific 5QI (i.e. 5QI=1 or 5) of the QoS flows for the downlink data packet.

UE:

-	receives paging message with the Paging Cause information.

For EPS:

SGW

-	optionally, determines Paging Cause based on DSCP value from IP header and HPLMN/APN/QCI configuration and includes the Paging Cause in DDN sent to MME. Alternatively implements existing, optional, Paging Policy Differentiation feature.

MME

-	determines the Paging Cause for NAS SMS and MT control plane traffic. . For user plane traffic the MME either receives the Paging Cause from the SGW or alternatively, determines a Paging Cause  based on HPLMN/APN/QCI configuration (e.g. QCI=1 or 5) received in DNN.

-	sends the S1 paging signalling with Paging Cause.

eNB

-	sends the paging message with Paging Cause.

UE

-	receives paging message with the Paging Cause information.

* * * * Next Change * * * *

* * * * End of Change * * * *