3GPP TSG-WG SA2 Meeting #139E e-meeting 	S2-200xxxx

Elbonia, June 01 - 12, 2020	(revision of S2-2004114, 3710, 3756, 3801 and 4154)

Title:	[DRAFT] Reply LS on provisioning "EDN connection info" by Edge Configuration Server

Response to:	LS (S2-2003201/S6-200617) on provisioning "EDN connection info" by Edge Configuration Server from SA6

Release:	Release 17

Work Item:	FS_enh_EC

Source:	3GPP TSG SA WG2

To:	3GPP TSG SA WG6

Cc:

Contact Person:

Name:	Hui Ni

Tel. Number:

E-mail Address:	hui.ni@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

SA2 thanks SA6 about the LS on provisioning "EDN connection info" by Edge Configuration Server (S2-2003201/S6-200617). SA2 discussed the questions and agreed the following answers.

Q1: SA6 kindly requests that SA2 comment on whether SA6’s understanding that a UE Application may provide a DNN is correct?

SA2 answer: SA2 confirms that the application layer in the UE can provide a DNN that can be used at the NAS layers by matching it against the DNN in a Traffic descriptor for matching a in the URSP rules. It is also possible that the Traffic descriptor does not contain a DNN, but the other components of the same Traffic descriptor match the corresponding information provided from the application (e.g. destination IP address or FQDN). In this case the DNN provided by the application can be replaced with the DNN in the Route Selection Descriptor (if present).

Q2: SA6 kindly requests that SA2 comment on how to handle the scenario when parameters provisioned by Edge Configuration Server create a conflict with URSP rules, ANDSP rules or UE local configuration?

SA2 answer:  Per current Stage 2 design, the association of an application to a PDU Session is based on either URSP or UE Local Configuration.

.

SA2 would like to point out that any parameters that could be used for DNN and/or S-NSSAI selection in the application that might be directly provisioned to UE by the ECS will be treated as UE Local Configuration. As described in , TS 23.503 clause 6.1.2.2 and 6.6.2handling of the URSP rules takes precedence URSP and  over UE Local Configuration.

If there is a need for application layer to provide some input for URSP generation at PCF, this is currently only possible for specific service like V2X.  Possibility to extend this to any other service is currently studied in SA2 in TR 23.748 for Rel-17.

Q3: SA6 kindly requests that SA2 comment on whether S-NSSAI can be provisioned by 3rd party via the application layer?

SA2 answer:

Per Stage 2 design, the association between application and S-NSSAI associated to a PDU Session (by evaluating either URSP or UE Local Configuration) should be controlled by the operator to ensure that only expected service traffic flows are transferred via a specific slice.

As explained in the answer to Q2, TR 23.748 is currenly studying means for a 3rd party to influence to S-NSSAI information in URSP rules under control of the PCF.

2. Actions:

To SA6 group.

ACTION: 	SA2 kindly asks SA6 to take above information into consideration

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #140e		19 Aug – 2 Sep 2020		Elbonia

TSG-SA2 Meeting #141                                  TBD			             TBD