3GPP TSG-SA2 Meeting #139E	S2-200xxxx

Elbonia, June 1 - 12, 2020

Source:	Samsung, Convida Wireless

Title:	Solution for unicast-multicast delivery mode switch

Document for:	Approval

Agenda Item:	8.9

Work Item / Release:	FS_5MBS / Rel-17

Abstract of the contribution: This contribution proposes a solution for KI#7 on unicast-multicast delivery mode switch.

# 1	Introduction

This paper presents a solution to Key Issue #7 making use of Baseline Architecture #2, as described in TR 23.757.

# 2	Proposal

The following changes are proposed to TR 23.757:

- * * * * First change * * * *

# 6	Solutions

## 6.0	Mapping of solutions to key issues

Editor's note:	This clause describes the mapping between solutions and key issues.

Table 6.0-1: Mapping of solutions to key issues

|  | Key Issues |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 MBS session management | 2 Service levels definition | 3 Levels of authorization for MC | 4 QoS for MC and BC | 5 BC TV and Radio services | 6 Local MBS | 7 MC-UC delivery mode switch | 8 BC-UC delivery method switch | 9 IWK with EPC/eMBMS for Public Safety |
| 1 |  | x |  |  |  |  |  |  |  |
| 2 | x |  |  |  |  |  |  |  |  |
| 3 | x |  |  |  |  |  |  |  |  |
| 4 | x |  |  |  |  |  |  |  |  |
| 5 | x |  |  |  |  |  |  |  |  |
| 6 | x |  |  |  |  |  |  |  |  |
| 7 |  |  |  |  |  | x |  |  |  |
| X |  |  |  |  |  |  | x |  |  |

- * * * * Second change (all new text) * * * *

## 6.X	Solution #X: Solution for reliable delivery mode switching between unicast and to multicast

### 6.X.1 Functional description

This solution addresses Key Issue #7 and assumes the baseline architecture described in Annex 2 TR 23.757 with dedicated NFs for MBS, namely MBSF for control plane and MBSU for user plane.

The solution describes a delivery mode switching method between unicast and multicast. The goal of this solution is for the system to be able to dynamically provide such a switch whenever the right triggers are given. Given the variety of use cases that could benefit from 5MBS switching functionality (e.g. for media, V2X, public safety, etc.), the triggers can be of a diverse nature, and this solution tries to flexibly support a variety of them. For example, the unicast to multicast switching trigger may differ if the switch is performed to distribute the same content over multicast instead of unicast for network resource efficiency purposes or if the switch is performed to guarantee service continuity due to UE mobility across different NG-RAN nodes with support of 5MBS and lack thereof.

Figure 6.X.1-1 shows an example scenario of the switching solution. The figure shows the provisioning of a multicast communication service, i.e. can be delivered via multicast, from an application/content server to one or more UEs. The content/application server transmits the signalling to the MBSF and the data to the MBSU. Two examples of the switching operation are summarized in the following:

- In an example of a switching operation, a UE establishes a multicast session, but may be receiving the data via point-to-point (PTP) byfrom the NG-RAN if the NG-RAN node is not point-to-multipoint (PTM) capable. After a mobility event (e.g. handover) the UE may be served by a PTM-capable NG-RAN node, and a switching procedure from unicast to multicast is performed.

- In another example of switching operation, the UE may be served via PTP by an NG-RAN node that is PTM capable. But, the UE population in the area consuming the same content from the same application server may grow, and the MBSF or the NG-RAN may decide to perform a switch to PTM.

Figure 6.X.1-1: Switch scenario examples

### 6.X.2	Procedures

Figure 6.X.2-1: Unicast to multicast switch solution

0a.	The availability of the multicast service is announced by the content server e.g. via application layer signaling. The announcement may provide information such as the multicast IP address and port number of the multicast group that the UE can join.

0b.	The UE and the 5GC establish an MBS session.

- MBS data transmission may start, and data may be exchanged between content/application server and the UE. When the delivery mode of MBS data is unicast, the UE may periodically transmit consumption reports to the network. These reports might be used to trigger the switch between unicast and multicast. In addition, a mobility event (e.g. handover) may also require a switch from unicast to multicast.

2.	Step 2 shows three potential alternatives to trigger the MBS switch from unicast to multicast in 5GS. Only one of them may trigger each switch, and the subsequent steps of the solution are shared.

2a.	Alternative 1: RAN triggered switch. Steps 2.1.a - 2.1.c. show the set of request messages originated by NG-RAN that need to be forwarded by the network to modify the MBS unicast PDU session already established. Since the NG-RAN triggers the switch, it may have done so e.g. due to mobility reasons, due to consumption reports sent by the UE in step 1, etc.

2.1.a. N2_Session_Modification Request is sent from NG-RAN to M-AMF.

2.1.b. Nsmf_Session_Modification Request is sent from M-AMF to MB-SMF, in response to the M-AMF receiving the N2_Session_Modification Request.

2.1.c. N6mb-C Session Modification Request is sent from the MB-SMF to the MBSF, in response to the MB-SMF receiving the Nsmf_Session_Modification Request.

2.1.d. MBS Session Notification is optionally sent from the MBSF to the content/application server; this only needs to take place in case the content server is required to be aware of the switch from unicast to multicast.

2b.	Alternative 2: MBSF triggered switch. This alternative assumes MBSF is the entity deciding to perform a switch from unicast to multicast once the MBS service has been initiated. The decision may be based on different optimization criteria (e.g. consumption reports, etc.).

2c.	Alternative 3: Content/application server triggered switch: This alternative accounts for the possibility of the content/application server deciding to switch from unicast to multicast.

3.	MBSF update comprising:

3a.	The MBSF requests the update of the UE’s session from the MB-SMF, using an Nsmf_PDUSession_Update Request. The MBSF may provide several parameters, e.g. SUPI, S-NSSAI, MBS ID/TMGI, DNN, etc.

3b.	The MB-SMF responds, using an Nsmf_PDUSession_Update Response, to acknowledge the update of the session and may provide its multicast IP address and port number for NG-RAN, among other parameters.

4.	[OPTIONAL] The MB-SMF may register the multicast session with the UDM, using a Nudm_UECM_Update message, which may acknowledge.

5.	[OPTIONAL] MBSF and MBSU may exchange internal messages.

NOTE:	The MBSF-MBSU communication is out of the scope of SA2.

6.	The MB-SMF transmits Namf_Communication_N1N2MessageTransfer message to the M-AMF to request the transfer of the message to the NG-RAN nodes(s). The message may contain parameters such as the multicast IP address, the port number, the APN, the NSAPI, etc. of the multicast MBS session. The M-AMF may acknowledge this transmission.

7.	Step comprising (7.a) The M-AMF sending the session creation request to the NG-RAN via an N2_PDU Session request message, and (7.b) the session creation being later acknowledged (after step 8) by the NG-RAN to the M-AMF with an N2_PDU Session response message that may include downlink tunnel information.

8.	The NG-RAN establishes the required resources for the UE by e.g. setting up the relevant bearers.

9.	[OPTIONAL] The NG-RAN node may join the relevant multicast group.

10.	Step comprising (10.a) the M-AMF transmitting an Nsmf_PDUSession_UpdateSMContext request to the MB-SMF to update the multicast session management context with the information from the NG-RAN (e.g. tunnel information), and (10.b) the MB-SMF confirming the reception with an Nsmf_PDUSession_UpdateSMContext response message.

11.	[OPTIONAL] Step comprising the following optional messages:

11a.	The MB-SMF may request the MBSF to update the multicast session context (e.g. the downlink tunnel information that was received in step 7) with the information from the NG-RAN via the N6mb-C interface.

11b. Responding to 11a, the MBSF may acknowledge the context update via the same interface.

12.	[OPTIONAL] The MBSF may need to internally communicate with the MBSU regarding the modification of the session parameters.

NOTE:	The MBSF-MBSU communication is out of the scope of SA2.

13.	UPF session modification step, comprising:

13a. The MB-SMF sends a N4 session modification request to the MB-UPF.

13b. The MB-UPF then acknowledges the session update.

14.	Data transmission step takes place comprising:

14a. Initially, the content is transmitted from the content provider to the MBSU.

14b. The MBSU processes the received data (e.g. applies FEC, raptor codes, etc.).

14c. The MBSU forwards the multicast data (i.e. the processed data) to the MB-UPF.

14d. The MB-UPF sends the forwarded data using IP multicast to the NG-RAN nodes.

14e. The NG-RAN node multicasts the data over the air using over-the-air multicast bearers, which is received by the UE.

Editor's note: A corresponding multicast to unicast switching solution is FFS.

### 6.X.3	Impacts on services, entities and interfaces

Editor's note: This clause describes impacts to services, entities and interfaces.

- * * * * End of changes * * * *