- SA WG2 Meeting #139E (e-meeting)	S2-2004188

- 1-12 June 2020, Elbonia

Title:		[DRAFT] Reply LS on early UE capability retrieval for eMTC

Reply to:	LS on early UE capability retrieval for eMTC (S2-2003575/R2-2003935)

Release:	Release 16

Work Item:	LTE_eMTC5-core, TEI16, 5G_CIoT

Source:	SA2

To:		RAN2, RAN3, CT1

Cc:		SA3

Contact person:	Sebastian Speicher

sspeiche@qti.qualcomm.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	[TS 23.401 CR 3603]

# 1	Overall description

SA2 thanks RAN2 for their LS.

Regarding the requested action to SA2:

RAN2 respectfully asks SA2, CT1 and RAN3

- To consider feasibility of introducing enhancements to enable the (ng-)eNB for early UE capability retrieval for eMTC UEs connected to EPC and 5GC in Rel-16, and if confirmed feasible, specify the required changes based on truncated 5G-S-TMSI solution in their specifications.

SA2 confirms that there are no issues to support early UE capability retrieval for EPS from system architecture perspective. SA2has agreed the attached related CR to TS 23.401.

For 5GS SA2 would like to provide the following feedback:

- SA2 has concerns to enable use of truncated 5G-S-TMSI for all UEs accessing ng-eNBs connected to 5GC as this would reduce the available AMF Set ID number space, AMF Pointer number space and TMSI number space (due to truncation) for all those UEs.

- As an alternative, SA2 can confirm that the AMF could also send the truncated Truncated 5G-S-TMSI Configuration to eMTC UEs to enable eMTC UEs to provide a Truncated 5G-S-TMSI as the UE identifier to an ng-eNB.

- This alternative would however require ng-eNBs to be able to determine whether the UE identifier signaled in Msg3 is a truncated 5G-S-TMSI or not.

SA2 would like to kindly request RAN2 to evaluate whether it is feasible to enable ng-eNBs to determine whether the UE identifier signaled in Msg3 is a truncated 5G-S-TMSI or not and to inform SA2 accordingly.

2	Actions

To RAN2

ACTION: 	SA2 asks RAN2 to take the above response into consideration and to provide feedback as described above.

# 3	Dates of next TSG SA WG2 meetings

TSG SA WG2 Meeting 140E		TBD

TSG SA WG2 Meeting 141		TBD