SA WG2 Meeting #139E 	S2-2003667

1 - 12 June 2020, Elbonia	(merge with S2-2003866, revision of)

Title:	DRAFT Reply LS on Clarification on eNA

Response to:	LS on Clarification on eNA (C3-202507/S2-2003544)

Release:	Rel-16

Work Item:	eNA

Source:	SA2

To:	CT3

Cc:	-

Contact person:

Name:	Yannick Lair

E-mail Address:	yannick.lair@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments: S2-2003339 (SA2#138E), S2-2003668 (SA2#139E)

1. Overall Description:

SA2 would like to provide the following answers to CT3 questions on eNA raised in their LS in C3-202507/S2-2003544.

Q1:  Is the network slice instance also applicable for other analytics types which may provide the S-NSSAI during the analytics request, e.g. QoS Sustainability Analytics?

For QoS Sustainability analytics in particular, since the main use case is for an external V2X AS to get QoS sustainability analytics, the NSI ID is unknown to the V2X AS acting as an external AF. Also, the QoS sustainability analytics rely on collecting data related to RAN UE throughput and RAN has no knowledge on NSI. The NSI ID consequently cannot be an analytics filter for QoS sustainability analytics. The same would apply to user data congestion analytics.

In addition, SA2 has not identified any use case for adding NSI ID as a possible filter to any of the other analytics for which S-NSSAI is a possible filter, i.e. NF load analytics, UE communication analytics and abnormal behaviour analytics.

Therefore, as currently specified in TS 23.288, NSI ID as analytics filter only applies to slice load analytics and to observed service experience analytics.

Q2: 	Does the maximum number of objects and/or the maximum number of SUPIs can be provided as both Analytics Reporting Information and Analytics Filter Information in one Nnwdaf_AnalyticsSubscription_Subscribe or Nnwdaf_AnalyticsInfo_Request request sent from the NF consumer to the NWDAF?

Maximum number of objects and Maximum number of SUPIs are defined as part of analytics reporting information, in TS 23.288 clause 6.1.3. But, as CT3 pointed out, TS 23.288, for some specific analytics, specifies that these maximum values are part of analytics filter information. SA2#139E agreed a CR to solve this inconsistency.

Maximum number of objects can be provided by the analytics consumer for each analytics defined in TS 23.288. Maximum number of SUPIs currently only applies to Observed service experience analytics and Abnormal behaviour analytics.

Also, related to these Maximum number of objects and Maximum number of SUPIs, SA2#138E agreed in S2-2003339 corrections on NWDAF services which provide further clarification on the inputs for NWDAF service operations, and, in particular, it clarifies that these maximum values can be provided per analytics IDs.

2. Actions:

To CT3 group.

ACTION: 	SA2 kindly ask CT3 to take the above answers into account for stage 3 work for eNA.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #140E		19 August – 2 September 2020		Elbonia

TSG-SA2 Meeting #141		12 – 16 October 2020			Tallinn, EE