3GPP SA WG2 Meeting #139E	S2-2003981r05

Elbonia, 1 – 12  June 2020            	(Revision of S2-2000xxx)

Source:	NTT DOCOMO, Orange, Telecom Italia, Deutsche Telekom,Verizon, AT&T

Title:	Architectural Assumption for MUSIM support

Document for:	Approval

Agenda Item:	8.10

Work Item / Release:	FS_MUSIM/ Rel-17

Abstract of the contribution: This contribution proposes an architecture assumption if a serving network does not support the MUSIM specific procedure.

# 1. Background

The study assumed that MUSIM feature (procedures) is supported in the serving PLMN, but, the serving PLMN shall not have to support this feature, which is like any other study in Rel-17. Hence, it is important to clarify that if a serving network does not support the MUSIM procedure, then UE shall not trigger any MUSIM specific procedures in the serving network. It is proposed that the UE will learn the network support of MUSIM procedure, and shall not trigger the procedure if the network is not supported.

# 2. Text Proposal

It is proposed to adopt the following text within the TR.

*** Start of the change ***

## 4.2	Architectural Assumptions

The following architectural assumptions apply:

-	The study shall focus on Dual-USIM devices, the expectation being that the enablers for Dual-USIM devices will also apply to Multi-USIM devices.

-	The study shall consider all of the following cases:

-	UE connected over 3GPP Access with EPS on one USIM and 5GS on the other USIM.

-	UE connected over 3GPP Access with EPS on both USIMs.

-	UE connected over 3GPP Access with 5GS on both USIMs.

NOTE 1:	The use of "3GPP Access" above is related to issues that can occur due to concurrent transmission and/or reception in Multi-USIM device via two or more 3GPP RATs.

-	Specific to Dual-USIM devices the study shall focus on Single Rx / Single Tx UEs and Dual Rx / Single Tx UEs.

NOTE 2:	Dual Rx allows the Dual-USIM device to simultaneously receive traffic from two networks. Single Rx allows the Dual-USIM device to receive traffic from one network at one time. Single Tx allows the Dual-USIM device to transmit traffic to one network at one time.

-	The Multi-USIM device shall handle Emergency calls using TS 22.101 [5] clause 10.9 as a basis.

-	The problem statement is common to 5GS and EPS, but the solutions for 5GS and EPS need not be the same.

-	The system enablers for Multi-USIM devices are expected to apply for the cases where the multiple USIMs are owned by the same or by different MNOs.

NOTE 3:	While the solutions developed as part of this study might also be applicable to DR-mode (single USIM) interworking between 5GS and EPS, 5GS-EPS interworking use cases with DR-mode (single USIM) are considered outside the scope of this study.

-	The solutions shall not require network coordination for the case where the multiple USIMs in the Multi-USIM device are served by different serving networks.

-	A multi-USIM device with different USIMs may be camping with all USIMs on the same serving network RAN node, or it may be camping on different serving networks RAN nodes.

-	For a multi-USIM device, the solutions may require in-device co-ordination between the UEs represented by each USIM within the multi-USIM device. The mechanism for in-device co-ordination shall be left for the multi-USIM device implementation.

-	MUSM features requiring new UE-network interactions are optional, hence the UE may use MUSIM features requiring new UE-network interactions in one PLMN when it has learnt that this PLMN supports these MUSIM features.

- *** End of the change ***