SA WG2 Meeting #139E e-meeting	S2-2004017r04

Elbonia, June 01 – 12, 2020	(revision of S2-2000492, S2-1911554)

Source:	Huawei, HiSilicon, Mediatek Inc.

Title:	KI#3, New Solution for service continuity via UE-to-Network Relay

Document for:	Approval

Agenda Item:	8.8

Work Item / Release:	FS_5G_ProSe / Rel-17

Abstract: This contribution proposes a solution for service continuity via L2 UE-to-Network Relay.

# 1. Introduction/Discussion

Key Issue 3 proposes the Support of UE-to-Network Relay. This contribution focus on the sub-issue of service continuity:

- How to perform communication path switch between a direct Uu path and an indirect Uu path via a UE-to-Network Relay, or between two indirect Uu paths via different UE-to-Network Relays. And how to guarantee service continuity during these communication path switch procedures.

This proposal focuses on the handover between direct Uu communication and communication via UE-to-Network Relay under the same NG-RAN node.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.752.

- * * * * First change * * * *

## 6.0	Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| 1 | X |  |  |  |  |  |  | X |
| 2 | X |  |  |  |  |  |  | X |
| 3 | X |  |  |  |  |  |  |  |
| 4 | X | X |  |  |  |  |  |  |
| 5 |  | X |  |  |  |  |  |  |
| 6 |  |  | X |  |  |  |  |  |
| 7 |  |  | X |  |  |  |  | X |
| 8 |  |  |  | X |  |  |  |  |
| 9 |  |  |  | X |  |  |  |  |
| 10 |  |  |  | X |  |  |  |  |
| 11 |  |  |  | X |  |  |  |  |
| 12 |  |  |  |  | X |  |  |  |
| 13 |  |  |  |  |  |  | X |  |
| 14 |  |  |  |  |  |  | X |  |
| 15 |  |  |  |  |  |  | X |  |
| 16 |  |  |  |  |  |  |  | X |
| 17 |  |  |  |  |  |  |  | X |
| 18 | X |  |  |  |  |  |  |  |
| 19 | X |  | X |  |  |  |  |  |
| 20 |  | X |  |  |  |  |  |  |
| 21 |  | X |  |  |  |  |  |  |
| 22 | X | X |  |  |  |  |  |  |
| X |  |  | X |  |  |  |  |  |

- 6.X	Solution #X: Service continuity via L2 UE-to-Network Relay

- 6.X.1	Description

This solution, based on Solution 7, addresses Key Issue #3 and proposes a Layer-2 based UE-to-Network relay solution to ensure the service continuity between the remote UE and the network, which covers the path switch between a direct Uu path and an indirect Uu path via a UE-to-Network Relay for the intra-NG-RAN case.

NOTE:	This solution is documented for illustration purpose only. It will be updated to highlight the SA2 impact if identified any, or will be removed and replaced by referencing to RAN WG conclusion.

- 6.X.2	Procedures

#### 6.X.2.1	From direct to indirect path under the same NG-RAN

Figure 6.X.2.1-1: Service continuity from direct to indirect under the same NG-RAN

- It is assumed that the Remote-UE has been authorized to access to the network via a UE-to-Network Relay.

- The Remote UE receives the measurement configuration via RRC message from the NG-RAN.

- Based on the measurement configuration as provided by NG-RAN, the Remote UE performs the UE-to-Network Relay Discovery procedure including necessary radio measurements according to the measurement configuratoin in Step 1. This procedure will be executed in parallel with other UE-to-Network Relay Discovery procedure(s).

Editor's note: The details of steps 1 and 2 will be decided by RAN WG2.

Editor's note: The detailed solution depends on the output of the discussion on solutions for KI#1.

3.	If the measurement reporting trigger is satisfied, the Remote UE sends an RRC message to the NG-RAN.

4.	Based on the received RRC message, the NG-RAN decides to handover the Remote UE from a direct 3GPP connection to an indirect 3GPP connection. The NG-RAN selects the UE-to-Network Relay.

Editor's note: The details of how the NG-RAN selects target UE-to-Network Relay will be decided by RAN WGs.

5.	The NG-RAN decides to handover the Remote UE within the same NG-RAN. The NG-RAN sends RRCReconfiguration message to the Remote UE.

6.	The Remote UE sets up the one-to-one communication connection with the UE-to-Network Relay.

7.	The Remote UE sends RRCReconfigurationComplete message via the UE-to-Network Relay to the NG-RAN.

8.	The NG-RAN sends RRCReconfiguration message to the UE-to-Network Relay about the AN resource setup.

9.	The NG-RAN may trigger the AN initiated PDU Session Modification as specified in TS 23.502 [xx].

#### 6.X.2.2	From indirect to direct path under the same NG-RAN

Figure 6.X.2.1-2: Service continuity from indirect to direct under the same NG-RAN

1. 	RRC message (e.g. Measurement Report) is triggered and sent to the NG-RAN via the UE-to-Network Relay.

2.	Based on the received RRC message, the NG-RAN decides to handover the Remote-UE from the UE-to-Network Relay to the serving NG-RAN.

3.	The NG-RAN sends RRCReconfiguration message via the UE-to-Network Relay to the Remote UE.

4.	The Remote UE sends RRCReconfigurationComplete message to the NG-RAN.

5.	AN initiated PDU Session Modification (same as Step 9 of clause 6.X.2.1).

6.	RRCConnectionReconfiguration is performed between the NG-RAN and the UE-to-Network Relay.

7.	Release side link connection between the Remote UE and the UE-to-Network Relay.

- 6.X.3	Impacts on Existing Nodes and Functionality

This solution needs to use the RAN specified L2 relay functionality for forwarding the signalling and user data between the Remote UE and the network.

Remote UE

-	Sends RRC message to report UE-to-Network Relay UE information.

NG-RAN

-	RRC message from Remote UE to report UE-to-Network Relay UE information.

-	During handover between direct and indirect communication, NG-RAN needs to reconfigure Relay UE.

AMF/SMF

-	No impact.

- * * * * End of changes * * * *