3GPP TSG-SA WG2 Meeting #139E	S2-2003629r02

01 - 12 June, 2020, Electronic, Elbonia

Source:	NEC

Title:	KI#7 New Sol#X: Steering the UE to a network slice in a different frequency band

Document for:	Approval

Agenda Item:	8.4

Work Item / Release:	FS_eNS_Ph2 / Rel-17

Abstract: A new solution for steering the UEs to a network slice in a different frequency band. It is backword compatible with Rel-15/16 UEs.

# 1. Background

This contribution proposes the following solution #x ‘Steering the UE to a network slice in a different frequency band’ for Key Issue #7 "Support of 5GC assisted cell selection to access network slice" in TR23.700-40 v0.3.0

******* start of 1st change ********

## 6.0	Mapping Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

| Solution#'s | Solution Titles | Key Issue#'s |
| --- | --- | --- |
| 1 | PCF measurement based Network Slice SLA control for Maximum Number of UEs parameter | 1 |
| 2 | Max number of UEs per Network Slice control at registration | 1 |
| 3 | AMF/NSSF based counting of UEs in a Network Slice | 1 |
| 4 | NWDAF enhancements for supporting of network slice quota on the maximum number of UEs | 1 |
| 5 | NWDAF enhancements for supporting of network slice quota on the maximum number of PDU Sessions | 2 |
| 6 | PCF-based counting of PDU Sessions in a Network Slice | 2 |
| 7 | Support of Network Slice SLA for Maximum Number of PDU sessions parameter | 2 |
| 8 | AMF and O&M based solution | 1, 2 & 4 |
| 9 | Monitoring multiple quotas of number of UEs/PDU Sessions per S-NSSAI at NWDAF | 1, 2 & 4 |
| 10 | Max number of PDU Sessions per Network Slice control via NSQ function | 2 |
| 11 | Handling maximum number of sessions using NF status | 2 |
| X | Steering the UE to a network slice in a different frequency band | 7 |

******* end of 1st change ********

******* start of 2nd change ********

## 6.X	Solution #X: Steering the UE to a network slice in a different frequency band

### 6.X.1	Introduction

This solution addresses the below requirements from Key Issue #7: "Support of 5GC assisted cell selection to access network slice".

-	How does 5GS steer UEs to a 5G-AN (e.g. a specific frequency band) that can support the network slices that the UE can use.

-	What information does 5GS need to take a decision to steer UE to a proper 5G-AN.

### 6.X.2	High Level Description

The solution is based on the following high level assumptions and principles:

- The UE is allocated Allowed NSSAI which can contain S-NSSAIs supported in different frequency bands.

- No mandatory UE impact. The solution equally works for legacy UEs Rel-15 and Rel-16.

### 6.X.3	Procedures

Figure 6.X.3.1-1 – Steering the UE to a network slice in different frequency band

1).	The UE is in idle mode, registered via RAN-1 for S-NSSAI-1 in frequency band 1 (FB-1)  and S-NSSAI-2 in frequency band 2 (FB-2).

2).	An application in the UE needs to establish service on S-NSSAI-2 in FB-2.

3).	The UE establishes RRC connection with RAN-1.

4). The UE triggers PDU Session Establishment Request on S-NSSAI-2 via RAN-1.

5)  AMF is aware that RAN-1, via which the UE is connected, does not support S-NSSAI-2 and the AMF is aware that S-NSSAI-2 is supported by another RAN Node.

6) AMF requests RAN-1 for UE handover to a RAN Node supporting S-NSSAI-2.

7) RAN-1 triggers inter-frequency cell change to RAN-2 which supports S-NSSAI-2 and is in UE’s location.

Note: It is for RAN Working Groups to define the inter-frequency cell change in connected mode e.g. Handover, Cell Change Order, RRC release with re-direction or RRC Connection reconfiguration.

7) AMF continues with the PDU Session establishment procedure on S-NSSAI-2 via RAN-2.

8) After PDU session on S-NSSAI-2 is released, the UE falls-back (e.g. re-selects back) to a cell in FB-1. This step is optional. If not implemented, the UE will stay on FB-2 until it is steered back to FB-1 due to a new service request on FB-2.

### 6.X.4	Impacts on existing entities and interfaces

-	UE cell fall-back to a cell on the initial frequency band (optional).

- 	AMF decision and request for handover to a cell from another frequency band.

******* end of 2nd change *******