3GPP TSG-SA WG2 Meeting #139E	S2-2003629r05

01 - 12 June, 2020, Electronic, Elbonia

Source:	NEC

Title:	KI#7 New Sol#X: Steering the UE to a network slice in a different frequency band

Document for:	Approval

Agenda Item:	8.4

Work Item / Release:	FS_eNS_Ph2 / Rel-17

Abstract: A new solution for steering the UEs to a network slice in a different frequency band. It is backword compatible with Rel-15/16 UEs.

# 1. Background

This contribution proposes the following solution #x ‘Steering the UE to a network slice in a different frequency band’ for Key Issue #7 "Support of 5GC assisted cell selection to access network slice" in TR23.700-40 v0.3.0

******* start of 1st change ********

## 6.0	Mapping Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

| Solution#'s | Solution Titles | Key Issue#'s |
| --- | --- | --- |
| 1 | PCF measurement based Network Slice SLA control for Maximum Number of UEs parameter | 1 |
| 2 | Max number of UEs per Network Slice control at registration | 1 |
| 3 | AMF/NSSF based counting of UEs in a Network Slice | 1 |
| 4 | NWDAF enhancements for supporting of network slice quota on the maximum number of UEs | 1 |
| 5 | NWDAF enhancements for supporting of network slice quota on the maximum number of PDU Sessions | 2 |
| 6 | PCF-based counting of PDU Sessions in a Network Slice | 2 |
| 7 | Support of Network Slice SLA for Maximum Number of PDU sessions parameter | 2 |
| 8 | AMF and O&M based solution | 1, 2 & 4 |
| 9 | Monitoring multiple quotas of number of UEs/PDU Sessions per S-NSSAI at NWDAF | 1, 2 & 4 |
| 10 | Max number of PDU Sessions per Network Slice control via NSQ function | 2 |
| 11 | Handling maximum number of sessions using NF status | 2 |
| X | Steering the UE to a network slice in a different frequency band | 7 |

******* end of 1st change ********

******* start of 2nd change ********

## 6.X	Solution #X: Steering the UE to a network slice in a different frequency band

### 6.X.1	Introduction

This solution addresses the below requirements from Key Issue #7: "Support of 5GC assisted cell selection to access network slice".

-	How does 5GS steer UEs to a 5G-AN (e.g. a specific frequency band) that can support the network slices that the UE can use.

-	What information does 5GS need to take a decision to steer UE to a proper 5G-AN.

### 6.X.2	High Level Description

The solution is based on the following high level assumptions and principles:

- The UE is allocated Allowed NSSAI which can contain S-NSSAIs supported in different frequency bands.

Editor's note:	it is FFS whether it is acceptable to change the current design principle by which the Allowed NSSAI contains all the S-NSSAI the UE can use concurrently. It is also FFS to see if the scenario by which an operator does not support services on common frequency bands, that are supposed to be run concurrently.

- No mandatory UE impact. The solution equally works for legacy UEs Rel-15 and Rel-16.

Editor's note:	it is FFS whether the solution can work for pre-Rel-17 UEs, as these UEs would expect to be able to have the ability to have PDU Sessions established on all S-NSSAIs concurrently.

### 6.X.3	Procedures

Figure 6.X.3.1-1 – Steering the UE to a network slice in different frequency band

1).	The UE is in idle mode, registered via RAN-1 for S-NSSAI-1 in frequency band 1 (FB-1)  and S-NSSAI-2 in frequency band 2 (FB-2).

2).	An application in the UE needs to establish service on S-NSSAI-2 in FB-2.

3).	The UE establishes RRC connection with RAN-1.

4). The UE triggers PDU Session Establishment Request on S-NSSAI-2 via RAN-1.

5)  AMF is aware that RAN-1, via which the UE is connected, does not support S-NSSAI-2 and the AMF is aware that S-NSSAI-2 is supported by another RAN Node.

6) AMF requests RAN-1 to steer the UE  to a RAN Node supporting S-NSSAI-2.

7) RAN-1 triggers inter-frequency cell change to RAN-2 which supports S-NSSAI-2 and is in UE’s location.

Note: It is for RAN Working Groups to define the inter-frequency cell change in connected mode e.g. Handover, Cell Change Order, RRC release with re-direction or RRC Connection reconfiguration.

Editor's note: it is FFS what happens to PDU Sessions already established over S-NSSAI1 in that case, and whether it is acceptable for them to be affected.

Editor’s Note: It is FFS how to proceed the PDU session establishment procedure if frequency cell change is failed.

8) AMF continues with the PDU Session establishment procedure on S-NSSAI-2 via RAN-2.

9) After PDU session on S-NSSAI-2 is released, the UE falls-back (e.g. re-selects back) to a cell in FB-1. This step is optional. If not implemented, the UE will stay on FB-2 until it is steered back to FB-1 due to a new service request on FB-2.

### 6.X.4	Impacts on existing entities and interfaces

-	UE cell fall-back to a cell on the initial frequency band (optional).

- 	AMF decision and request for handover to a cell from another frequency band.

******* end of 2nd change *******