3GPP TSG-SA WG2 Meeting #139E	S2-2003630r1

01 - 12 June, 2020, Electronic, Elbonia

Source:	NEC

Title:	KI#6 New Sol#X: Network slices simultaneous usage incompatibility support

Document for:	Approval

Agenda Item:	8.4

Work Item / Release:	FS_eNS_Ph2 / Rel-17

Abstract: A new solution to address the requirements in KI#6. It proposes a solution for network slices simultaneous usage incompatibility support.

# 1. Background

This contribution proposes a new solution #X ‘Network slices simultaneous usage incompatibility support’ for Key Issue #6 "Constraints on simultaneous use of the network slice" in TR23.700-40 v0.3.0

******* start of 1st change ********

## 6.0	Mapping Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

| Solution#'s | Solution Titles | Key Issue#'s |
| --- | --- | --- |
| 1 | PCF measurement based Network Slice SLA control for Maximum Number of UEs parameter | 1 |
| 2 | Max number of UEs per Network Slice control at registration | 1 |
| 3 | AMF/NSSF based counting of UEs in a Network Slice | 1 |
| 4 | NWDAF enhancements for supporting of network slice quota on the maximum number of UEs | 1 |
| 5 | NWDAF enhancements for supporting of network slice quota on the maximum number of PDU Sessions | 2 |
| 6 | PCF-based counting of PDU Sessions in a Network Slice | 2 |
| 7 | Support of Network Slice SLA for Maximum Number of PDU sessions parameter | 2 |
| 8 | AMF and O&M based solution | 1, 2 & 4 |
| 9 | Monitoring multiple quotas of number of UEs/PDU Sessions per S-NSSAI at NWDAF | 1, 2 & 4 |
| 10 | Max number of PDU Sessions per Network Slice control via NSQ function | 2 |
| 11 | Handling maximum number of sessions using NF status | 2 |
| X | Network slices simultaneous usage incompatibility support | 6 |

******* end of 1st change ********

******* start of 2nd change ********

## 6.X	Solution #X: Network slices simultaneous usage incompatibility support

### 6.X.1	Introduction

This solution addresses the below requirements from Key Issue #6: “Constraints on simultaneous use of the network slice”:

1)	How to enforce the constraints related to simultaneous usage of Network Slices in the UE and in the network, both in roaming and non-roaming scenarios.

2)	How to ensure that the identified enforcement solution does not negatively impact the network operations of Rel-15 and Rel-16 5GS deployments.

### 6.X.2	High Level Description

This solution proposes network slices simultaneous usage incompatibility support by both, the UE and the network. It is based on the following high level assumptions and principles:

- The UDM holds information about subscribed S-NSSAI’s incompatibility for simultaneous usage attribute;

Editor's note: it is FFS why UDM should store information regarding network deployment. Mutual slice exclusivety is a network-deployment consideration, not a user-specific consideration.

- The AMF retrieves the subscribed S-NSSAI’s incompatibility for simultaneous usage attribute from the UDM and stores it as part of the UE context in the AMF for the duration of the UE registration with that AMF. At mobility re-registrations, the network slices incompatibility for simultaneous usage attributes are passed to the new AMF with the UE context;

Editor's note: it is FFS why the AMF should store network deployment considerations as part of the UE context. Mutual slice exclusivety is a network-deployment consideration, not a user-specific consideration.

- If the UE supports the network slices simultaneous usage incompatibility feature, the UE indicates its support for it in the Registration Request message.

- The AMF provides to the UE the allowed S-NSSAI’s simultaneous usage incompatibility attributes in the Registration Accept message or UE Configuration Update message only if the UE has indicated its support for it.

Editor's note: The Allowed NSSAI can only contain network slices that can be used simultaneously, which this solution does not comply with. Moreover, as the GST parameter is handled by the OAM, there will be no AMF that belongs simultaneously to two network slices which cannot be used simultaneously, therefore there can be no Allowed NSSAI that contain these incompatible slices.

- The solution does not negatively impact the network operations of Rel-15 and Rel-16 5GS deployments.

### 6.X.3	Procedures

#### 6.X.3.1	Network slices simultaneous usage incompatibility control by the UE.

Figure 6.X.3.1-1 – Network slices simultaneous usage incompatibility control by the UE

- The UE requests registration with the network and if the UE supports the network slices simultaneous usage incompatibility feature, the UE also includes in the Registrations Request message a network slice incompatibility support indication.

- If the AMF supports the network slices simultaneous usage incompatibility feature, the AMF includes the network slices incompatibility support indication to the UDM when retrieving UE’s subscription information.

- In return, if the AMF indicated a support for network slices simultaneous usage incompatibility, the UDM returns the network slices incompatibility attributes per each subscribed S-NSSAI which can have one of the following values:

0 – The network slice can be used simultaneously with any network slice;

1 – The network slice can be used simultaneously with any network slice with same SST value;

2 - The network slice can be used simultaneously with any network slice with same SD value;

3 - The network slice cannot be used simultaneously with another network slice.

- If the UDM provided the network slices incompatibility attribute for the subscribed S-NSSAIs, the AMF stores the network slices incompatibility attributes in the UE context.

- When confirming the UE registration procedure, if the UE indicated its support for network slices incompatibility in the Registration Request message in step 1, then the AMF includes in the Registration Accept message a parameter with the network slices incompatibility attributes per each S-NSSAI from the allowed NSSAI.

- The UE behaviour after receiving network slices incompatibility attributes per allowed S-NSSAI - The UE shall not allow for simultaneous PDU sessions on network slices that are incompatible for simultaneous use. For example, if the UE has already established a PDU Session(s) on one or more network slices and the UE wants to establish another PDU Sessions on a new network slice, the UE shall first check for simultaneous usage incompatibility between the new network slice and each of the network slices on which there is an already establish PDU Session and vice-versa. The UE shall not trigger a PDU Session establishment on a new network slice if (the new network slice is incompatible for simultaneous use with any other network slice on which a PDU Session has already been established) OR (any network slice on which a PDU Session has already been established is incompatible for simultaneous use with the new network slice on which the UE wants to establish a PDU Session).

#### 6.X.3.2	Network slice simultaneous usage incompatibility control by the network

While the network slices simultaneous usage incompatibility supporting UE manages the restrictions for network slices simultaneous usage incompatible at its origin (i.e. by checking for simultaneous usage incompatibility before the UE triggers a PDU session establishment on a new network slice as described in Figure 6.X.3.1-1), it is still needed for the network to be able to control the UE behaviour, i.e. whether the UE respects the network slices simultaneous usage incompatibility attributes. The Figure 6.X.3.2-1 bellow demonstrates the network control for network slices simultaneous usage incompatibility.

Figure 6.X.3.2-1 – Network slices simultaneous usage incompatibility control by the network

1). There are already established PDU Session(s) on one or more network slices.

2). The UE triggers a PDU Session Establishment on a new network slice S-NSSAI_X.

3). The AMF checks for network slice simultaneous usage incompatibility between the S-NSSAI_X and the network slices on which there are already established PDU Session(s). The AMF already holds in the UE context the incompatibility attributes for each allowed for the UE network slices acquired from the UDM during the UE registration or via the UE context transfer from a previous AMF. If (the new network slice S-NSSAI_X is incompatible for simultaneous use with any other network slice on which a PDU Session has already been established) OR (any network slice on which a PDU Session has already been established is incompatible for simultaneous use with the new network slice S-NSSAI_X on which the UE wants to establish a PDU Session), the PDU Session Request on S-NSSAI_X shall be rejected for simultaneous usage incompatibility reason.

Otherwise, there is no simultaneous usage incompatibility between S-NSSAI_X and the network slices already in use and the procedure continues to step 5.

4) AMF rejects the PDU Session Establishment Request on S-NSSAI_X. The AMF may include in the PDU Session Establishment Reject message a reject cause for network slice incompatibility. The AMF may also include the list of the network slices with which the rejected network slice (i.e. S-NSSAI_X) is incompatible for simultaneous use and the procedure ends here.

If the UE received a list of network slices that are incompatible for simultaneous use with the rejected network slice, the UE shall not trigger another PDU Session establishment on the rejected network slice until all PDU Sessions on the reported incompatible network slices are released.

5) The AMF continues with the PDU Session establishment procedure on S-NSSAI_X as per TS23.502.

### 6.X.4	Impacts on existing entities and interfaces

UE/AMF:

- Network slice incompatibility indication in the Registration Request message;

- Network slice incompatibility attribute per allowed network slice in the Registration Accept and UE Configuration Update message;

Editor's note: The Allowed NSSAI can only contain network slices that can be used simultaneously, which this solution does not comply with. Moreover, as the GST parameter is handled by the OAM, there will be no AMF that belongs simultaneously to two network slices which cannot be used simultaneously, therefore there can be no Allowed NSSAI that contain these incompatible slices.

- A new PDU Session rejection cause related to the network slice incompatibility;

Editor's note: The Allowed NSSAI only contain S-NSSAIs that can be used simultaneously, therefore there should not be any PDU Session rejection due to network slice exclusivity.

- Logic for network slices simultaneous usage incompatibility check both in the UE and the AMF.

UDM:

- Holds network slices simultaneous usage incompatibility attributes per subscribed network slice

Editor's note: it is FFS why UDM should store information regarding network deployment. Mutual slice exclusivety is a network-deployment consideration, not a user-specific consideration.

******* end of 2nd change *******