SA WG2 Meeting #139E	S2-2003752r01

Elbonia, June 1- 12, 2020	(revision of S2-200xxxx)

Source:	Apple

Title:	KI #5, New Sol: AF providing survival time as part of the TSC Assistance container

Document for:	Approval

Agenda Item:	8.5

Work Item / Release:	FS_IIoT / Rel-17

Abstract: This contribution proposes a solution in which survival time is provided by AF as part of the TSC Assistance container.

# 1. Introduction

IIoT applications as defined in TS 22.104 Annex A are classified into four major verticals and the Table A.2.1-. Survival time for the IIoT applications are defined in Tables 5.2-1, 5.2-2, 5.3-1 and 5.5-1.

START of CHANGE

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

| Solutions | Key Issues |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | #1 Uplink Time Synchronization | #2 UE-UE TSC communication | #3A Exposure of TSC services: Exposure of deterministic QoS | #3B Exposure of TSC services Exposure of Time Synchronization | #4 supporting the fully distributed configuration model for TSN | #5 |
| #1 | X |  |  |  |  |  |
| #2 |  | X |  |  |  |  |
| #3 |  | X |  |  |  |  |
| #4 |  | X |  |  |  |  |
| #5 |  |  | X |  |  |  |
| #6 |  |  | X |  |  |  |
| #7 |  |  |  | X |  |  |
| #8 |  |  |  | X |  |  |
| #X |  |  |  |  |  | X |

NEXT CHANGE (All text is new)

## 6.X	Solution #X: IIoT application specific AF providing survival time as part of the TSC Assistance container

### 6.X.1	Introduction

This solution addresses Key issue #5: Use of Survival Time for Deterministic Applications in 5GS.

### 6.X.2	High Level Description

This solution is based on the assumption that AF supports a wide range of IIoT application areas or verticals.

-	During PDU Session Establishment procedure from an IIoT UE, SMF will contact AF via PCF to provide to establish/modify the 5GS Bridge. Procedure is in accordance with 23.502, Annex F.1

-	AF will be pre-loaded with the survival time information for the IIoT applications as defined in TS 22.104 [4], Tables 5.2-1, 5.2-2, 5.3-1 and 5.5-1.

-	AF will provide the survival time as part of the TSC Assistance Container.

-	PCF provides the survival time to SMF, as part of PCC rules. SMF delivers the survival time to the NG-RAN and UE (via AMF) as part of the TSC Assistance Container.

### 6.X.3	Procedures

The procedure in Figure 6.X.3-1 shows a signalling flow for the procedure to deliver the survival time for an IIoT application as part of the TSC Assistance container:

Figure 6.X.3-1: Procedures to deliver survival time as part of the TSC Assistance container

1.	UE+DS-TT initiates a PDU Session Establishment procedure with SMF via AMF. UE includes following information in this message: UE DS-TT Residence time, DS-TT MAC address, Port Management capabilities, DNN and optionally S-NSSAI.

2.	SMF selects a suitable UPF for the TSC session.

3.	UPF assigns port number and provides the following information to SMF: port number for DS-TT, port number for NW-TT, Bridge ID.

4.	SMF encapsulates the information received in step 1 and 3 into "5GS Bridge Information" and sends them to PCF requesting PCC rules for setting up QoS for this session.

5.	PCF contacts the AF via Npcf_PolicyAuthorisation_Create_Request to provide the QoS parameters for the TSC session. PCF provides the "5GS Bridge Information" including port management capabilities of the DS-TT and NW-TT. This is in accordance with TS 23.502, Annex F.1 – 5GS Bridge Information reporting.

6.	AF provides the survival time for the requested TSC session as part of the TSC Assistance container. This container contains the survival time along with the QoS parameters.

7.	PCF creates the PCC rules and send them to SMF. The PCC rules contain the TSC Assistance container: including  survival time.

8.	SMF retrieves survival time from TSC Assistance container provides the CN Tunnel Info, QoS profile (5QI, QFI), TSCAI and SMF derived CN assisted RAN parameters tuning to the AMF.

9.	AMF forwards the information received in step 8 to NG-RAN for setting up the N3 tunnel. This message contains the following parameters: PDU Session ID, CN Tunnel Info, QoS profile with QFI, TSCAI, NAS PDU, UL NG-U UP TNL Information.

10.	NG-RAN sends a PDU Session Setup Response to the AMF.

11.	AMF forwards the PDU Session Establishment Accept to UE. This message contains the QoS flow with 5QI values and survival time information.

### 6.X.4	Impacts on existing entities and interfaces

AF

Provide Survival Time to PCF

PCF

Create PCC rules containing the TSC Assistance container which includes survival time

SMF

Retrieve survival time from TSN Assistance Container, and provide to AMF

### 6.X.5 	Evaluation

Editor's note:	This clause provides an evaluation of the solution.

End of CHANGES