SA WG2 Meeting #139E	S2-2003826r01

Elbonia, June 1- 12, 2020		      (revision of S2-200xxxx)

Source:	Apple

Title:	KI #16, New Sol: UP optimization for Edge Computing

Document for:	Approval

Agenda Item:	8.1

Work Item / Release:	FS_eNA_ph2 / Rel-17

Abstract of the contribution: This contribution proposes a solution for Key Issue #16: UP optimization for Edge Computing

# 1. Introduction

Edge computing enables to locate resources near the consumers to provide low latency and high data volume. In order to support tremendous services/applications running in edge computing efficiently, 5GC need to consider edge computing deployment and hosted application/service characteristics. For most efficient utilization of resources and best experience for end user, NWDAF can be used to provide analytics to automate certain functions of Edge Computing application.

The following aspects are considered here:

-	Analytics which could be exposed to AF (edge computing) and internal NFs to improve UP optimization and support edge computing operations efficiently.

-	Data to be collected by the NWDAF to provide analytics information to support edge computing.

- 	Assisting UP path optimization considering DN characteristics.

The proposed new inputs to analytics provide insights which can then be used to improve the user experience of UE application in Edge Computing environment.

This solution is proposed for Key Issue #16.

# 2. Proposal

The following changes are proposed for TR 23.700-91.

* * * * Start of Changes * * * *

## 6.0	Mapping Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

| Solutions | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| 1 | 11 |  |  |  |
| 2 | 4 |  |  |  |
| 3 | 15 |  |  |  |
| 4 | 12 |  |  |  |
| X | 16 |  |  |  |

* * * * Next Change (All new text) * * * *

## 6.X	Solution #X: UP optimization for Edge Computing

### 6.X.1 	Functional Description

#### 6.X.1.1 	General

NWDAF supporting Edge Computing analytics and predictions shall be able to collect information about UE and EAS via NFs (e.g. AF, AMF, SMF, UPF, etc.) and perform data analytics to optimize the utilization and relocation of Edge computing resources for respective application instances.

The consumer of these analytics may indicate in the request the target of analytics reporting (a group of UEs and/or application instances) and the analytics filter information containing parameters such as:

-	UE group mobility patterns

-	QoS Parameters

-	Application status and state

-	Traffic state and rules

-	Edge server resource status

If the consumer of these analytics and predictions is an AF, then AF may influence traffic routing following procedures in TS 23.502 [3] clause 4.3.6, and if SMF or PCF are consumers, they shall utilize procedures in TS 23.502 [3] clause 4.16.5 for user plane traffic routing and updating N4 information for local traffic handling, routing and steering corresponding to N4 rules (PDR, FAR, URR, QER, etc.) with respective UPF(s).

#### 6.X.1.2 	Input data

The information collected via 5GC NFs for UE group mobility behaviour is defined in Table 6.X.1.2-1:

Table 6.X.1.2-1: Input Data related to for UE mobility behaviour analytics

| Information | Source | Description |
| --- | --- | --- |
| UE/Group ID | AMF | UE Group IDs |
| UE locations (1..max) | AMF | UE positions |
| > UE location | AMF | TA or cells that the UE enters |
| > Timestamp | AMF | A timestamp when the AMF detects the UE enters this location |
| Type Allocation code (TAC) | AMF | To indicate the model and vendor information of the UE. The UEs with the same TAC may have similar mobility behaviour. The UE whose mobility behaviour is unlike other UEs with the same TAC may be an abnormal one. |
| Frequent mobility re-registration | AMF | A UE (e.g. a stationary UE) may re-select between neighbour cells due to radio coverage fluctuations. This may lead to multiple re-registrations if the cells belong to different Registration Areas. The number of re-registrations N within a period M may be an indication for abnormal ping-pong behaviour, where N and M are operator's configurable parameters. |

Most of the information is already provisioned to be available from AMF for NWDAF, the new inclusion of Frequent Mobility Re-Registration for UE on border of Registration Areas. This will help to allocate respective resources for this group of UE/UE application.

UE group/application session ID, EAS measurements and EAS capability are new data inputs required to provide analytics and predictions for resource allocation, relocation and steering depending upon usage and current state, thus mitigating impacts of congestion and outages while also handling the traffic/server switching predictions for future sessions depending upon resource usage and UE application statistics. These analytics can be useful to influence UP traffic routing with help of NF, forwarding request to respective UPF(s).

Table 6.X.1.2-2: Input Data related to for EAS application UP analytics

| Information | Source | Description |
| --- | --- | --- |
| UE ID | AF | GPSI or external UE ID |
| Application IDs | AF | Identifying Edge application providing this service |
| Trajectory | AF | Edge Relocations |
| > UE location | AF | Tracking Area Mapping to Edge Service Area |
| > Timestamp | AF | A timestamp when UE enters this Edge Service Area |
| > UE group Application Session ID | AF | UE group Application ID served by respective EAS |
| List of resource status (1..n) | AF | List of observed load information for each EAS instance along with the corresponding EAS ID / EAS Set ID (as applicable) |
| > EAS type | AF | Type of the EAS instance |
| > EAS instance ID | AF | Identification of the EAS instance |
| > EAS status | AF | The availability status of the EAS on the Analytics target period, expressed as a percentage of time per status value (registered, suspended, undiscoverable) |
| > EAS resource usage | AF | The average usage of assigned resources (CPU, memory, disk) |
| > EAS load | AF | The average load of the EAS instance over the Analytics target period |
| > EAS peak load | AF | The maximum load of the EAS instance over the Analytics target period |
| > EAS Measurements | AF | RTT and other information related to measurements |
| > EAS Capability | AF | Capability of EAS processor, GPU, memory, etc. |
| QoS flow Bit Rate | UPF | The observed bit rate for UL direction; and The observed bit rate for DL direction |
| QoS flow Packet Delay | UPF | The observed Packet delay for UL direction; and The observed Packet delay for the DL direction |
| Packet transmission | UPF | The observed number of packet transmission |
| Packet retransmission | UPF | The observed number of packet retransmission |

NOTE:  How the UPF contacts NWDAF is left to network implementation.

#### 6.X.1.3 	Output Analytics

The NWDAF supporting UP optimization for edge computing analytics shall be able to provide analytics and prediction to consumer NFs. With this analytics information, NFs taking decision for UP optimization should be able to subscribe periodically or on event basis (threshold based) or a one-time request for run time decision making. This analytics information can be used to improve the user experience of UE(s) with respect to EAS application, may include traffic steering/routing rules and updates for respective application session and UE group. These analytics and predictions are provided for user plane resource allocation, relocation and traffic steering for edge computing session, following steps from TS 23.502 [3] clause 4.3.6 or 4.16.5 based on consumer NF.

Table 6.X.1.3-1: NWDAF provided statistics and analytics

| Information | Target Node | Description |
| --- | --- | --- |
| Resource Status | NF | List of predicted information for each EAS instance along with the corresponding EAS id / EAS Set ID with respect to UE groups served per application |
| > EAS resource relocation | NF | Relocation of assigned resources (CPU, memory, disk) for EAS application to respective UE groups |
| > EAS load distribution | NF | The average load of the EAS instance over the Analytics target period for different application aspects |
| > EAS TAC mapping | NF | Mapping of EAS area against mobility of same TAC(Type Allocation Code) for UE within different Tracking Areas |
| > EAS Applications’ state | NF | State of applications’ run on different EAS with respect to UE groups |
| Traffic Rule Update | NF | Traffic steering rules for respective application session, to be forwarded via PCF for rules update and forwarding to respective UPF(s) via SMF(s) |
| > Confidence | NF | Confidence of this prediction |

Editor’s Note: 	Whether EAS-TAC mapping is needed from NWDAF, or whether this is already available at SMF or PCF is FFS.

Editor’s Note: 	Whether NWDAF needs to provide traffic steering rules and if yes, what are the recommended values is FFS.

Predictions shall be forwarded to respective NF (finally SMF and UPF) making decision for EAS resource relocation which can be on basis of past UE group mobility/activity behaviour and application server status over a period of time.

Table 6.X.1.3-2: NWDAF provided predictions

| Information | Target Node | Description |
| --- | --- | --- |
| Resource Update | NF | List of predicted information for each EAS instance along with the corresponding EAS id / EAS Set ID with respect to UE groups served per application |
| > EAS resource management | NF | Based upon Analytics model, prediction for whether resource for particular EAS need to be incremented or reduced / optimisation |
| > UE Group mobility | NF | Predicting the UE group mobility on basis of subscription data model & congestion period predictions |
| > Application state & load | NF | Application state and load prediction on basis of time of usage and number of users at particular instance |
| > Confidence | NF | Confidence of this prediction |

## 6.X.2 	Procedures

The NFs may request analytics from NWDAF for respective Edge computing user plane optimization. The analytics request may be subscription based on periodic or aperiodic with threshold defined for requesting the analytics. Further NWDAF may request information data points from different NFs (AMF, AF, SMF, UPF, etc.) to provide precise analytics and predictions for Edge resources relocation and allocations.

The following flow can be used as illustration of the request and response in the network:

EAS-LADN

NWDAF

NF

OAM

UPF

AF

AMF

1. Nnwdaf_AnalyticsInfo_Request/

Nnwdaf_AnalyticsSubscription_Subscribe

2. Naf_EventExposure_Subscribe/

UPF Traffic Usage Report

3. Naf_EventExposure_Notify/

Traffic Usage Response

4. Namf_EventExposure_Subscribe

5. Namf_EventExposure_Notify

6. Subscribe

EAS/UE(s) Information

7. Notification

8. NWDAF derives Analytics

9. Nnwdaf_AnalyticsInfo_Response/

Nnwdaf_AnalyticsSubscription_Notify

10. Trigger for new analytics/prediction Request or Periodicity

11. Naf_EventExposure_Notify/

Traffic Usage Response

12. Namf_EventExposure_Notify

13. Nnwdaf_AnalyticsInfo_Response/

Nnwdaf_AnalyticsSubscription_Notify

14. Analytics & Predictions for optimized UP performance between UE(s) & EAS

Figure: 6.X.1.4-1: NWDAF requested analytics/predictions for Edge computing

Initially UE groups are accessing multiple applications through EAS (via UPF) with respective DN(s). AMF has the information of UE(s) mobility within its Registration Area and AF has information about the resources with UPF closely monitoring the QoS of the User Plane connection.

1. 	NF sends Analytics Info Request or Subscription request to NWDAF with respect to UE group and application session information.

2. 	NWDAF initiates request and subscription to EAS resources and QoS with respective AF & UPF with respect to application session ID and UE group served by the applications.

NOTE 1:	This request to UPF/AF will be forwarded with the same UE group ID & application ID for which analytics were requested by NF.

3.	UPF and AF respond with the data requested by NWDAF for the analytics as per the table 6.X.1.2-2.

4.	NWDAF initiates Event Exposure to AMF for mobility parameters and UE group mobility within the Registration Area and also requests the counter to check UE group mobility.

5. 	AMF responds with the data required by NWDAF as per table 6.X.1.2-1 with a Notify message.

6. 	NWDAF requests subscription for EAS/UE group SLA information from OAM, considering privacy, this may be limited to group information instead of specific UE.

NOTE 2:	Request to OAM data would depend upon the precision required for analytics with respect to granularity in conditions of UE and EAS.

7.	OAM responds with notification for the information regarding the request NWDAF subscribed to.

8.	NWDAF derives analytics and also predictions with the available data with an appropriate confidence level.

9.	These analytics and predictions are provided for user plane resource allocation, relocation and traffic steering for Edge computing session, following steps from TS 23.502 [3] clause 4.3.6 or 4.16.5 based on consumer NF.

10.	Trigger or periodicity is defined to get these results in initial step. This trigger may be (but not limited to) group UE mobility, change of EAS serving area, EAS server status, etc.

11.	Notifications are received by NWDAF from UPF and AF, until the end/termination of the request session from NF and later on NWDAF.

12. Notifications may further be received from AMF in case the mobility of UE group patter is concerned and trigger for the event.

13.	Periodic or trigger based information is shared with NF with new session ID and analytics ID for respective application session with EAS.

14. UE group and EAS resources are migrated and relocated, and traffic routing/steering rules are updated by NF (e.g. AF) on basis of analytics for best User Plane performance for edge computing sessions.

NOTE 3:	The EAS service area and tracking/registration area can be mapped as follows for ease of management of resources and trigger conditions for relocations as in Figure 6.x.1.4-2:

Figure: 6.X.1.4-2: EAS and TA mapping

### 6.X.3 	Impacts on existing entities and interfaces

-	NF: SMF/PCF/AF should be able to request information from NWDAF to make decision of edge relocation and then update traffic rules for UP optimization of the session based on TS 23.502 [3] clause 4.3.6 or 4.16.5 based on consumer NF

-	AF/UPF: AF/UPF shall be able to share information of the application sessions ongoing, without disclosing privacy information, and should be connected to multiple Edge resources for UP & CP information respectively.

### 6.X.4	Evaluation

Editor's note:	This clause provides an evaluation of the solution.

* * * * End of Changes * * * *