SA WG2 Meeting #139E	S2-2003836r01

Elbonia, June 1- 12, 2020	(revision of S2-200xxxx)

Source:	Apple

Title:	KI #5, New Sol: Functional entities and triggers for initiating policy update for ProSe path selection

Document for:	            Approval

Agenda Item:	8.8

Work Item / Release:	FS_5G_ProSe / Rel-17

Abstract: This contribution proposes a solution for Key Issue #5:  Direct communication path selection.

This solution is proposed for Key Issue #5 to study Support direct communication path selection between PC5 and Uu.

* * * * Start of Changes * * * *

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| 1 | X |  |  |  |  |  |  | X |
| 2 | X |  |  |  |  |  |  | X |
| 3 | X |  |  |  |  |  |  |  |
| 4 | X | X |  |  |  |  |  |  |
| 5 |  | X |  |  |  |  |  |  |
| 6 |  |  | X |  |  |  |  |  |
| 7 |  |  | X |  |  |  |  | X |
| 8 |  |  |  | X |  |  |  |  |
| 9 |  |  |  | X |  |  |  |  |
| 10 |  |  |  | X |  |  |  |  |
| 11 |  |  |  | X |  |  |  |  |
| 12 |  |  |  |  | X |  |  |  |
| 13 |  |  |  |  |  |  | X |  |
| 14 |  |  |  |  |  |  | X |  |
| 15 |  |  |  |  |  |  | X |  |
| 16 |  |  |  |  |  |  |  | X |
| 17 |  |  |  |  |  |  |  | X |
| 18 | X |  |  |  |  |  |  |  |
| 19 | X |  | X |  |  |  |  |  |
| 20 |  | X |  |  |  |  |  |  |
| 21 |  | X |  |  |  |  |  |  |
| 22 | X | X |  |  |  |  |  |  |
| Y |  |  |  |  | X |  |  |  |

* * * * Next Change (all new text) * * * *

## 6.Y	Solution #Y: Functional entities and triggers for initiating policy update for ProSe path selection

UPF

N4

N2

N3

N6

UE-A

UE-C

ProSe Application

ProSe Application

ProSe Application

UE-B

Uu

PC5

PC5

UDM

UDR

SMF

PCF

DDNMF

NEF

ProSe Application

DN

AF

AMF

Namf

Nsmf

Nnef

Npcf

Naf

Nudr

Nudm

RAN

UPF

N4

N2

N3

N6

UE-A

UE-C

ProSe Application

ProSe Application

ProSe Application

UE-B

Uu

PC5

PC5

UDM

UDR

SMF

PCF

DDNMF

NEF

ProSe Application

DN

AF

AMF

Namf

Nsmf

Nnef

Npcf

Naf

Nudr

Nudm

RAN

### 6.Y.1	Functional entities and roles to initiate policy update

Functional entities and triggers at these entities shall be responsible for direct communication path selection and impact on the corresponding interfaces. Triggers and initiation of policy provisioning for path selection may be initiated by UE or AF.

UE and AF shall use these policies for path selection between PC5 and Uu interface with the help of RAN node (QoS parameters, AMBR, authorization etc., required for RAN resource management, roaming, network scheduled operation modes and Xn handover).

Editor's note:	RAN impacts shall be studied in RAN WGs.

#### 6.Y.1.1	UE triggered request

- 	UE triggers may include (but not limited to) relay operation change, policy expiry, PDU session establishment, change in location and/or application instance, new application requesting ProSe (like MBS), etc.

-	UE shall initiate ProSe policy provisioning request with policy container for UE triggered update.

-	UE shall configure parameters and proceed with ProSe procedures over PC5 interface and select between PC5 and Uu path based on policies provided by PCF.

-	UE shall establish PDU session to ProSe Function according to UE configuration.

#### 6.Y.1.2 	AF initiated request

-	Application changes may cause policy update requirement and request is generated at AF which may be (but not limited to) triggered by application request, content provider request, UE initiated request, application request, etc.

-	AF may request analytics and predictions from NWDAF to trigger changes in UE mode of operation and policy provisioning (provide better user experience and service with help of proximity services).

-	These nodes support PC1 (direct UP link), PC5 link unavailability requests from UE(s), content transfer request between unicast and multicast and many other applications.

-	NFs such as multicast function CP (MSF/MBSF), content provider or emergency service may initiate request similar to AF to enable commercial broadcast, 3rd party public safety provider application related policy changes.

### 6.Y.2 	Policy change procedures

#### 6.Y.2.1	 	UE triggered policy update

UE may initiate the selection policy update based on UE triggers. The procedure is similar to V2X UE requesting policy provisioning:

UE

RAN

AMF

PCF

4. UE policy delivered as described in TS 23.502 clause 4.2.4.3

1.Triggers from UE

2. UE Policy Provisioning Request

UE ProSe Policy Container

3. Npcf_UEPolicyControl Update

ProSe Policy update requested

Figure 6.Y.2.1-1: UE requested policy update

1.		Based on UE triggers, UE may request policy change from PCF.

2.		The UE sends the UE Policy provisioning request including the UE Policy Container (ProSe PSS Policy Provisioning Request) to the AMF.

3.		The AMF sends the Npcf_UEPolicyControl_Update request to the PCF including the UE Policy Container received from UE.

4.		UE Policy delivery/update procedure defined in TS 23.502 [8] clause 4.2.4.3.

### 6.Y.2.2 	AF initiated policy update

Application Server

Application Server

UE

UE

RAN

RAN

AMF

AMF

SMF

SMF

PCF

PCF

NEF

NEF

AF

AF

7. N4 Session update to UPF

5. ProSe Policy Provisioning

1.UE request PC3 message (Discovery/Group Communication Request)

Application ID, Session Information

2. Session Update Request

Update Type, Trigger request

3. Nnef_TrafficInfluence_Create/Update/Delete

ProSe Update, Trigger Cause

4. Npcf_PolicyAuthorization_Create/Update/Delete

Policy Update, Trigger, Application ID, Session ID

6. Npcf_SMPolicyControl Update

Policy Update, Session Information

8. Namf_Communication_N1N2MessageTransfer

Policy Update, Session Information

9. N2 Message for RAN Policy

10. Service request to deliver UE policies

7. N4 Session update to UPF

5. ProSe Policy Provisioning

1.UE request PC3 message (Discovery/Group Communication Request)

Application ID, Session Information

2. Session Update Request

Update Type, Trigger request

3. Nnef_TrafficInfluence_Create/Update/Delete

ProSe Update, Trigger Cause

4. Npcf_PolicyAuthorization_Create/Update/Delete

Policy Update, Trigger, Application ID, Session ID

6. Npcf_SMPolicyControl Update

Policy Update, Session Information

8. Namf_Communication_N1N2MessageTransfer

Policy Update, Session Information

9. N2 Message for RAN Policy

10. Service request to deliver UE policies

Figure 6.Y.2.2-1: AF initiated policy update

1. 	UE may send Discovery Request or Group Communication update to Application Server.

NOTE 1: 	This step may be skipped depending on which node generated trigger. In case the generating node is the AF, this request shall be skipped. This request may further be initiated by single or multiple UEs

2. 	Session Update Request is generated from Application Server and is forwarded to AF using PC2 interface with Application ID, User ID, Session ID and other ProSe session parameters.

3. 	AF invokes the Traffic Influence request for the Application and Session ID with request type ProSe discovery or Selection or Policy Change depending upon the trigger type and node at which the trigger was generated.

4. 	NEF requests for Policy Authorization Create/Modify/Delete at the relevant PCF depending upon the nature of the request and access type, forwarding the information received from the AF.

5.	UE Policy delivery/update procedure defined in TS 23.502 [8] clause 4.2.4.3.

6. 	PCF initiates the ProSe policy update including application information and other session information to SMF related to the PDU session using SMPolicyControl message.

7.	N4 session is modified at UPF with required policy changes and UP tunnel is created/modified/torn down depending upon the request type, trigger type or application type and prepares for buffering for seamless mobility.

8.	PCF initiates N1N2MessageTransfer to AMF to initiate policy distribution to RAN and UE.

9. 	N2 update message is sent by AMF to RAN for updating the policy provisioning at RAN with UE policy container (which may also include, but not limited to PC5 QoS parameters, AMBR, authorization etc., required for RAN resource management, roaming, network scheduled operation modes and Xn HO) using Namf_Communication_N1N2MessageTransfer message

Editor’s Note:  RAN impacts shall be studied in RAN WGs.

10. Depending upon the UE(s) states, connections, RAN initiates paging or network-initiated Service Request or RRC Reconfiguration with the policy update container delivered from AMF.

NOTE 2:	As per the policy, state and trigger, the selection of interface (PC5 or Uu) is performed at UE(s). Discovery Request may be initiated at this step for PC5.

#### 6.Y.3 	Network analytics and predictions to trigger ProSe selection

AF may be subscribed to NWDAF to receive analytics and predictions for PC5 interface for ProSe application. AF may request for the analytics and NWDAF can collect information from AMF and PCF for respective policies.

### 6.Y.4 	Impacts on existing entities and interfaces

-	AMF: Selects PCF supporting ProSe provisioning from NRF, forwards the policies to UE and RAN.

-	PCF: Determines and provisions UE policy for ProSe application on request from UE or ProSe AF

-	AF: Requests policy update from PCF and ProSe parameter provisioning depending upon triggers.

-	UE: Requests policy update depending upon triggers and utilizes the policy for selection between PC5 & Uu interfaces

Editor’s Note: RAN impacts shall be studied in RAN WGs.

### 6.Y.5	Evaluation

Editor's note:	This clause provides an evaluation of the solution.

* * * * End of Changes * * * *