SA WG2 Meeting #139E e-meeting	S2-2003882r02

Elbonia, June 1 - 12, 2020	(revision of S2-2xxxxxx)

Source:	Huawei, HiSilicon

Title:	KI #10 & #16, New Sol: Solution for NWDAF assisted UP optimization for edge computing

Document for:	Discussion / Approval

Agenda Item:	8.1

Work Item / Release:	FS_eNA_ph2 / Rel-17

Abstract of the contribution: This contribution proposes a solution for KI#10 and #16 that how NWDAF can provide service experience analytics per UP path including UPF and DNAI to the SMF to assist UP optimization for edge computing.

- Discussion

TR 23.700-91 has discussed and given requirements as following for KI#10 and KI #16:

//Requirements extracted from KI #10//:

In order to support diverse usage scenarios of 5G such as edge computing and AR/VR, the applications or services deployed in cloud or edge computing could be run on multiple external ENs, and/or they can be relocated into another location. It means that the service experience can be affected by both internal and external UP selections. To improve the service experience, this key issue needs to consider service/application traffic characteristics, mobility of UE and frequent relocation of service/application and the end to end service/application performance analytics among the multiple data paths.

//Requirements extracted from KI #16//:

Edge computing enables to locate resources near the consumers to provide low latency and high data volume. In order to support tremendous services/applications running in edge computing efficiently, 5GC need to consider edge computing deployment and hosted application/service characteristics. In this context, it is necessary to consider 1) native edge service/application characteristics, 2) mobility of both UE mobility and service/application mobility, 3) the characteristics and performance of DN between UPF and edge computing, 4) edge computing infrastructure deployment.

This contribution is to provide solution for at least the following the issues:

-	What information the NWDAF outputs will contain.

-	Study whether new type of analytics are needed for UP optimization.

-	How the NFs can make use of analytics for its actions.

-	What kind of data should be collected by the NWDAF to provide new type of UP optimization analytics.

-	What kind of analytics should be exposed to AFs or NFs to select optimal UP to improve end-to-end service experience.

2.	Proposal

It is propose a solution for KI#10 and KI #16 into the TR 23.700-91.

* * * * First Change * * * *

6.0	Mapping Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

| Solutions | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| 6.X | KI#10 | KI#16 |  |  |

* * * * Second Change * * * *

6.X	Solution for Key Issue #10 and #16: <NWDAF Assisted UP Optimization for edge computing>

6.X.1	Description

This solution is proposed to address Key Issue #10: NWDAF assisted UP optimization and KI #16: UP optimization for edge computing.

This solution specifies for how NWDAF can provide service experience analytics per UP path including UPF and DNAI to the SMF to assist UP optimization for edge computing.

6.X.2	Input Data

The service data collected from the AF/NEF, the network data from other 5GC NFs and the network data from OAM for  service experience analytics are defined in Table 6.X.2-1, Table 6.X.2-2 and Table 6.X.2-3, respectively.

Table 6.X.2-1: Service Data from AF related to the Service Experience analytics as defined in Table 6.4.2-1, clause 6.4.2, TS 23.288 [x]

| Information | Source | Description |
| --- | --- | --- |
| Application ID | AF | To identify the service and support analytics per type of service (the desired level of service) |
| Locations of Application | AF/NEF | Locations of application represented by a list of DNAI(s). The NEF may map the AF-Service-Identifier information to a list of DNAI(s) when the DNAI(s) being used by the application are statically defined. |
| Service Experience | AF | Refers to the QoE as established in the SLA and during on boarding. It can be either e.g. MOS or video MOS as specified in ITU-T P.1203.3 [11] or a customized MOS |
| Timestamp | AF | A time stamp associated to the observed level of Service Experience provided by the AF, mandatory if the observed Service Experience is provided by the ASP. |

Table 6.X.2-2: QoS flow level Network Data from 5GC NF as defined in Table 6.4.2-2, clause 6.4.2, TS 23.288 [x]

| Information | Source | Description |
| --- | --- | --- |
| Timestamp | 5GC NF | A time stamp associated with the collected information. |
| Location Info | AMF | The UE location information when the service is delivered. |
| DNN | SMF | DNN for the PDU Session which contains the QoS flow |
| S-NSSAI | SMF | S-NSSAI for the PDU Session which contains the QoS flow |
| Application ID | PCF/SMF | Used by NWDAF to identify the application service provider and application for the QoS flow |
| UPF info | SMF | UPF ID/address/FQDN information for the UPF serving the UE |
| IP filter information | SMF | Provided by the SMF, which is used by NWDAF to identify the service data flow for policy control and/or differentiated charging for the QoS flow |
| QFI | SMF | QoS Flow Identifier |
| QoS flow Bit Rate | UPF | The observed bit rate for UL direction; and The observed bit rate for DL direction |
| QoS flow Packet Delay | UPF | The observed Packet delay for UL direction; and The observed Packet delay for the DL direction |
| Packet transmission | UPF | The observed number of packet transmission |
| Packet retransmission | UPF | The observed number of packet retransmission |

NOTE 1: The text highlighted in yellow is a new input from SMF, comparing to the ones for the Service Experience analytics defined in clause 6.4.2, TS 23.288.

Table 6.X.2-3: UE level Network Data from OAM as defined in Table 6.4.2-3, clause 6.4.2, TS 23.288 [x]

| Information | Source | Description |
| --- | --- | --- |
| Reference Signal Received Power | OAM | The per UE measurement of the received power level in a network cell, including SS-RSRP, CSI-RSRP as specified in clause 5.5 of TS 38.331 [14] and E-UTRA RSRP as specified in clause 5.5.5 of TS 36.331 [15] |
| Reference Signal Received Quality | OAM | The per UE measurement of the received quality in a network cell, including SS-RSRQ, CSI-RSRQ as specified in clause 5.5 of TS 38.331 [14] and E-UTRA RSRQ as specified in clause 5.5.5 of TS 36.331 [15] |
| Signal-to-noise and interference ratio | OAM | The per UE measurement of the received signal to noise and interference ratio in a network cell, including SS-SINR, CSI-SINR, E-UTRA RS-SINR, as specified in clause 5.1 of TS 38.215 [12] |

6.X.3	Output Analytics

The NWDAF services as defined in the clause 7.2 and 7.3, TS 23.288 [X], are used to expose the analytics.

-	Service Experience per UP path statistics information is defined in Table 6.X.3-1.

-	Service Experience per UP path predictions information is defined in Table 6.X.3-2.

Table 6.X.3-1: Service Experience per UP path statistics

| Information | Description |
| --- | --- |
| ServiceExperience per UP path (1..n) | List of observed service experience information for each Application per UP path (including UPF and DNAI). |
| > Application ID | Identification of the application. |
| > Service Experience Type | Type of Service Experience analytics, e.g. on voice, video, other. |
| >>UE location | Indicating the UE location information (e.g. cell ID, gNB ID, etc) when the UE service is delivered. |
| >>UPF Info | Indicating UPF serving the UE |
| >>DNAI | Indicating which DNAI the UE service uses/camps on |
| >> Service Experience | Service Experience over the Analytics target period (average, variance), e.g. service MOS. |
| > Spatial validity | Area where the estimated Service Experience applies. If Area of Interest information was provided in the request or subscription, spatial validity may be a subset of the requested Area of Interest. |
| > Validity period | Validity period as defined in clause 6.1.3,TS 23.288. |

NOTE 2: The text highlighted in yellow is new output information for the Service Experience statistics as defined in clause 6.4.3, TS 23.288.

Table 6.X.3-2: Service Experience per UP path predictions

| Information | Description |
| --- | --- |
| ServiceExperience per UP path (1..n) | List of observed service experience information for each Application per UP path (including UPF and DNAI). |
| > Application ID | Identification of the application. |
| > Service Experience Type | Type of Service Experience analytics, e.g. on voice, video, other. |
| >>UE location | Indicating the UE location information (e.g. cell ID, gNB ID, etc)when the UE service is delivered. |
| >>UPF Info | Indicating which UPF the UE service uses/camps on |
| >>DNAI | Indicating which DNAI the UE service uses/camps on |
| >> Service Experience | Service Experience over the Analytics target period (average, variance), e.g. service MOS. |
| > Spatial validity | Area where the estimated Service Experience applies. If Area of Interest information was provided in the request or subscription, spatial validity may be a subset of the requested Area of Interest. |
| > Validity period | Validity period as defined in clause 6.1.3, TS 23.288. |
| > Probability assertion | Confidence of this prediction. |

NOTE 3: The text highlighted in yellow is new out information for the Service Experience predictions as defined in clause 6.4.3, TS 23.288.

6.X.4	Procedures

Figure 6.X.4-1: Procedure for subscription to network performance analytics

- The SMF sends Nnwdaf_AnalyticsSubscription_Subscribe or Nnwdaf_AnalyticsInfo_Request (Analytics ID=Service Experience per UP path (UPF and DNAI), Target of Analytics Reporting=any UE, Analytics Filter=Application ID, Area of Interest, and Analytics target Period(s)) to the NWDAF.

- The NWDAF provides the data analytics, i.e. the observed Service Experience per UP path (UPF and DNAI) to the consumer SMF by means of either Nnwdaf_AnalyticsInfo_Request response or Nnwdaf_AnalyticsSubscription_Notify.

- An AF may send a request to influence SMF routing decisions for traffic of PDU Session as defined in clause 5.6.7, TS 23.501 and clause 4.3.6.4, TS 23.502.

- The PCF updates the SMF with corresponding new PCC rule(s) with PCF initiated SM Policy Association Modification procedure as described in clause 4.16.5.2, TS 23.502.

- During the procedure that AF influence on traffic routing as defined in 4.3.6.4, TS 23.502,  besides the PCC rules from the PCF, the SMF can also take the Service Experience analytics per UP path from the NWDAF into account to:

    - (re)select UP paths ,including UPF and DNAI e.g. as described in clause 4.3.5, TS 23.502;

    - (re)configure traffic steering, updating the UPF regarding the target DNAI with new traffic steering rules.

6.X.5	Impacts on services, entities and interfaces

Editor's note:	This clause lists impacts to services and interfaces.

* * * * End of change * * * *