3GPP TSG-WG SA2 Meeting #139E e-meeting  	S2-2003907

# Elbonia, June 1 – 12, 2020	(revision of S2-200xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.287 | CR | 0140 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | On Subscription to V2X services |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eV2XARC |  |  |  |  |  | Date: |  |  | 2020-02-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | C3-202347 rises the issue on how to provide the initial (i.e., when there is no policy parameters at UE side) V2X policy/parameters to UE when UDR does not have related data in the policy set entry. One way to solve the issue is to define a new parameter set (namely the 'V2X policy parameter related data'). We propose to define such set within the UE context policy control subscription information so that it will be pre-stored in UDR. This V2X policy parameter data is outside of the Policy Set Entry for the PCF authorization in the UDR. We further clarify that “During registration or UE trigger parameters update procedure PSI will be used”, since in the current version of the TS the PSI-related description only appears in general description part. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add a new policy data within UE context policy control subscription information. - Clarify that during registration or UE trigger parameters update procedure PSI will be used |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It would be not clear for the other working groups how to process the V2X policy parameters. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1, 5.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 5.1.1	General

In 5GS, the parameters for V2X communications over PC5 and Uu reference points may be made available to the UE in following ways:

-	pre-configured in the ME; or

-	configured in the UICC; or

-	preconfigured in the ME and configured in the UICC; or

-	provided/updated by the V2X Application Server via PCF and/or V1 reference point; or

-	provided/updated by the PCF to the UE.

If the same parameters described in clauses 5.1.2.1 and 5.1.3.1 are provided by different sources, the UE shall consider them in the following priority order:

-	provided/updated by the PCF;

-	provided/updated by the V2X Application Server via V1 reference point;

-	configured in the UICC;

-	pre-configured in the ME.

The parameters provided/updated by the V2X Application Server via V1 reference point may need to be complemented with configuration data from other sources listed above.

The basic principles of service authorization and provisioning for V2X communication over PC5 reference point and provisioning for V2X communication over Uu reference point are:

-	The UE may be authorized to use V2X communication over PC5 reference point on a per PLMN basis by the PCF in the HPLMN.

-	The PCF in the HPLMN merges authorization information from home and other PLMNs and provides the UE with the final authorization information.

-	The PCF in the VPLMN or HPLMN may revoke the authorization (via H-PCF when roaming) at any time by using the UE Configuration Update procedure for transparent UE Policy delivery procedure defined in clause 4.2.4.3 of TS 23.502 [7].

-	The provisioning to UE for V2X communication over PC5 and Uu reference points is controlled by the PCF at Initial Registration or the Registration to 5GS when the UE moves from EPS to 5GS or may be triggered by UE. The PCF includes the V2X Policy/parameters for V2X communications over PC5 reference point as specified in clause 5.1.2.1 and/or the V2X Policy/parameters for V2X communications over Uu reference point as specified in clause 5.1.3.1 into a Policy Section identified by a Policy Section Identifier (PSI) as specified in clause 6.1.2.2.2 of TS 23.503 [16].

# * * * * End of changes * * * *