3GPP TSG-WG SA2 Meeting #139E e-meeting 	S2-2003908Rev01

Elbonia, June 1 – 12, 2020	(revision of S2-200xxxx)

# Title:	[DRAFT] Reply LS on subscription to V2X services

# Response to:	LS on subscription to V2X services from CT3 (S2-2003541/C3-202347)

# Release:	Rel-16

# Work Item:	eV2XARC

Source:	[Huawei to be] SA2

To:	CT3

Cc:	CT4

Contact Person:

Name:	Li Meng

E-mail Address:	Raymond DOT limeng AT huawei DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	TS 23.287 CR#0140;

1. Overall Description:

SA2 would like to thank CT3 for the LS on subscription to V2X services (C3-202347), and would like to provide the following answers:

Q1:	If there is no PSI stored in the Policy Set Entry, how does the PCF create the related V2X policies for the V2X UE? (e.g., first V2X UE registration, where the V2X UE may not report any PSI)? Is there any subscription information outside of the Policy Set Entry for the PCF authorization in the UDR?

SA2's answer: The V2X Policy is handled in the same way with URSP in PCF, thus there should be PSI associated to a set of V2X Policy. The PCF creates the related V2X policies irrespective of whether the PSI is stored or not. A new parameter set within UE context policy control subscription information, namely V2X policy parameter related data, has been defined to support the PCF creating the V2X policies..

Q2: As there is no procedure to define that the V2X UE reports the PSI during the initial registration, why does the H-PCF need to store the V2X Policy for the V2X UE in the UDR?

SA2's answer: This is incorrect understanding. In TS 23.287 it is mentioned that the PSI is used to identify the V2X policy/parameter. We further clarify that “During registration or UE trigger parameters update procedure PSI will be used”. Therefore, if the PSI available in the UE, the UE should provide it during initial registration.

Q3:	If the PCF updates the V2X Policy for the V2X UE based on the changes of the subscription information which UDR data set stores the above referred subscription information?

SA2's answer: The V2X Policy is updated using Nudr service for Data Set "Policy Data" and Data Subset "Policy Set Entry".

2. Actions:

To CT3:

ACTION: 	SA2 kindly asks CT3 group to take the above answer into account.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #140E		19 Aug – 02 Sep 2020	e-meeting

TSG-SA2 Meeting #141E		12 – 16 Oct 2020		e-meeting